| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0279-v1 |
| --- | --- |

| Title: | CE4-related: Simplification of motion vector storing process for triangle prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two methods to simplify the motion vector (MV) storing process at partition boundary for triangle prediction mode. Compared with VTM5.0, in method 1, the sequential reference picture search procedure is removed, and motion vector selection by partition direction is applied when the two MVs of the two partitions point to the same reference picture list. In method 2, the motion vector selection by partition direction is always applied. Results are summarized as follows.

Proposed method 1
VTM5.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM5.0-LB: {Y-BD-rate=0.06%, EncT=100%, DecT=100%}

Proposed method 2
VTM5.0-RA: {Y-BD-rate=0.04%, EncT=100%, DecT=102%}
VTM5.0-LB: {Y-BD-rate=0.17%, EncT=100%, DecT=101%}

# Introduction

In Versatile Video Coding Draft 5 [1], the motion vector storing process at partition boundary for the triangle prediction mode is unnecessarily complicated. It includes several sequential if-else conditions, especially when two MVs from two partitions both point to the same reference picture list. For that case, it requires additional search to check whether the reference picture of either one of the partitions exists in the other list. If the number of reference pictures in the list is large (e.g. 16 reference pictures in one list), the additional search process will increase the hardware design cost and difficulty for small coding units (CUs).

It is proposed to simplify the MV storing process for triangle prediction mode by using partition direction to replace the above mentioned search procedure. Details are described in the following sections.

# Proposed methods

## Method 1

It is proposed to choose one of the two MVs from two partitions according to the partition direction depending on merge_triangle_split_dir syntax element. As shown in Figure 1, the diagonal MVs are set to MVA when partition direction is equal to 0, and the diagonal MVs are set to MVB when partition direction is equal to 1. In this method, the simplified MV selection is only applied when two MVs from two partitions both point to the same reference picture list.

Figure 1. Derivation of diagonal MVs by partition direction

## Method 2

Method 2 is based on method 1. However, the simplified MV selection is always applied no matter what the reference picture lists these two partitions point to.

# Experiment results

The proposed methods has been implemented on top of VTM5.0. The common test conditions [2] are used for evaluation. Table 1 and 2 summarizes the results of method 1 and 2, respectively. Decoding time percentages are not reliable. It is asserted that the proposed methods should not result in noticeable runtime changes.

Table 1. Simplified MV selection when MVs of two partitions point to the same reference picture list

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.08% | 100% | 100% |
| Class A2 | -0.02% | 0.04% | -0.02% | 100% | 100% |
| Class B | 0.01% | 0.13% | -0.10% | 100% | 101% |
| Class C | -0.02% | -0.14% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.05% | 100% | 100% |
| Class D | 0.01% | 0.18% | 0.15% | 100% | 108% |
| Class F | -0.01% | 0.03% | 0.00% | 100% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.21% | -0.03% | 101% | 100% |
| Class C | 0.06% | 0.10% | 0.07% | 100% | 101% |
| Class E | 0.02% | 0.14% | -0.43% | 100% | 98% |
| Overall | 0.06% | 0.16% | -0.10% | 100% | 100% |
| Class D | 0.01% | 0.00% | 0.43% | 98% | 98% |
| Class F | 0.05% | 0.31% | -0.05% | 98% | 97% |

Table 2. Simplified MV selection, always applied

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.05% | 100% | 102% |
| Class A2 | 0.05% | 0.03% | 0.09% | 101% | 98% |
| Class B | 0.07% | 0.04% | -0.04% | 101% | 97% |
| Class C | -0.01% | -0.13% | -0.15% | 100% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.05% | -0.02% | 100% | 102% |
| Class D | 0.05% | -0.13% | 0.35% | 100% | 118% |
| Class F | -0.01% | 0.05% | 0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.26% | 0.05% | 100% | 98% |
| Class C | 0.16% | 0.12% | 0.21% | 101% | 104% |
| Class E | 0.23% | 0.30% | -0.29% | 100% | 102% |
| Overall | 0.17% | 0.22% | 0.02% | 100% | 101% |
| Class D | 0.16% | -0.21% | 1.03% | 99% | 101% |
| Class F | 0.01% | 0.04% | -0.22% | 98% | 99% |

# Conclusion

In this contribution, it is proposed to simplify the MV storing process at partition boundary for triangle prediction mode by using partition direction. It can remove the sequential search process and reduce the hardware design cost. Compared with VTM5.0, the BD rate change is minor.

# References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 5),” Document of Joint Video Experts Team, JVET-N1001.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).