| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0291-v2 |
| --- | --- |

| Title: | CE6-related: Constraint and simplification for LFNST signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The signalling of low frequency non-separable transform (LFNST) needs to count nonzero quantized transfrom coefficients of the entire luma coding unit (CU) and the LFNST index is signalled after all transform blocks (TBs) are signalled. This causes a serious processing issue for 64x64-luma/32x32-chroma (64x64-L/32x32-C) decoder pipelining when the CU is 128x128-luma/64x64-chroma (128x128-L/64x64-C) with four forced split transform units (TUs) of 64x64-L/32x32-C in inter slices. In this case, the first three 64x64-L/32x32-C TUs cannot be processed before the parsing of the fourth 64x64-L/32x32-C TU is done, which breaks the 64x64-L/32x32-C decoder pipelining. Three methods are proposed in this contribution to solve the issue as follows.

- Method 1: Disabling intra coding for CUs larger than 64x64-L/32x32-C samples by not signalling pred_mode_flag and inferring it to be 0 (MODE_INTER)

- Method 2: Disabling LFNST for CUs larger than 64x64-L/32x32-C samples

- Method 3: Signalling LFNST at TU level (three variants of method 3 are proposed)

It is reported that, compared to VTM5.0, the proposed method 1 results in very tiny (0.00% after rounding) BD-rate changes. For method 2 and method 3, BD-rates are not changed under the current common test conditions (CTC).

# Introduction

In VTM5.0 [1], LFNST was adopted, where LFNST can be applied to intra CUs for both luma and chroma. A LFNST index, denoted as lfnst_idx, specifies whether the LFNST is enabled and which one of the two LFNST kernels in the selected transform set is used. If lfnst_idx is equal to 0, LFNST is not used. If lfnst_idx is equal to 1 or 2, the first or the second LFNST kernerl is chosen from the selected transform set accordingly. The signalling of lfnst_idx is described as follows. If chroma separate tree (CST) is used, lfnst_idx for luma and chroma are signalled separately; otherwise, a single lfnst_idx is signalled and applied to both luma and chroma. lfnst_idx is signalled at the end of CU when all of the following conditions are satisfied; otherwise, lfnst_idx is inferred to be 0.

- TB width and height are greater than or equal to 4

- Transform skip mode is not used

- Intra subpartitions is not used

- Matrix-based intra prediction is not used

- Intra mode is used

- numSigCoeff is larger than a threshold (T)

- numZeroOutSigCoeff is equal to 0,

where numSigCoeff means the number of nonzero quantized transform coefficients within the current CU and the threshold is 1 for CST enabled and 2 for shared tree. numZeroOutSigCoeff means the number of nonzero quantized transform coefficients within the LFNST zero-out region for the current CU.

To signal lfnst_idx, it needs to count numSigCoeff and numZeroOutSigCoeff, so lfnst_idx is signalled at the end of CU. This causes a serious processing issue for 64x64-luma/32x32-chroma (64x64-L/32x32-C) decoder pipelining when the CU is 128x128-luma/64x64-chroma (128x128-L/64x64-C) with four forced split transform units (TUs) of 64x64-L/32x32-C in inter slices. In this case, the first three 64x64-L/32x32-C TUs cannot be processed before the parsing of the fourth 64x64-L/32x32-C TU is done, which breaks the 64x64-L/32x32-C decoder pipelining. In this proposal, three methods are proposed to solve the decoder pipeline issue by preventing deriving lfnst_idx across the 64x64-L/32x32 virtual pipeline data unit (VPDU) boudaries [2][3].

# Proposed methods

In this proposal, three methods are proposed as follows,

Method 1 is to disable intra coding for CUs larger than 64x64-L/32x32-C by not signalling pred_mode_flag and inferring it to be 0 (MODE_INTER).

Method 2 is to disable LFNST for CUs larger than 64x64-L/32x32-C samples.

Method 3 is to signal LFNST at TU level.

- Method 3.1: When multiple TUs are split in one CU, a single lfnst_idx is signalled at the end of the first TU and is used for all TUs in the current CU. numSigCoeff is defined as the number of nonzero coefficients within the first TU and numZeroOutSigCoeff is defined as the number of nonzero coefficients within the zero-out region of the first TU.

- Method 3.2: When multiple TUs are split in one CU, a single lfnst_idx is signalled at the end of the first TU and can only be used for the first TU in the current CU. numSigCoeff is defined as the number of nonzero coefficients within the first TU and numZeroOutSigCoeff is defined as the number of nonzero coefficients within the zero-out region of the first TU.

- Method 3.3: When multiple TUs are split in one CU, a lfnst_idx is signalled at the end of each TU in the current CU. numSigCoeff is defined as the number of nonzero coefficients within current TU and numZeroOutSigCoeff is defined as the number of nonzero coefficients within the zero-out region of the current TU.

# Experimental results

The proposed methods are implemented on VTM5.0. The simulations are performed following CTC [4]. Table 1 shows the simulation results of the proposed method 1. For method 2 and method 3, BD-rates are not changed under the current CTC. The runtime data are not accurate. It is claimed that the runtime change will not be noticeable if measured accurately using homogeneous computing clusters.

Table 1.  Results of method 1 (LB results to be updated)

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.03% | 104% | 100% |
| Class A2 | -0.02% | 0.02% | 0.03% | 106% | 107% |
| Class B | 0.02% | 0.04% | 0.00% | 103% | 99% |
| Class C | 0.00% | -0.01% | -0.02% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.01% | 103% | 100% |
| Class D | 0.00% | 0.00% | 0.01% | 97% | 98% |
| Class F | 0.00% | 0.03% | 0.02% | 98% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# Conclusions

This contribution proposes to solve the decoder pipeline issue for LFNST. Three aspects are proposed. Method 1 and method 2 are to apply a size constraint. In method 3, LFNST syntax is signalled at TU level. Compared to VTM5.0, method 1 results in very tiny BD-rate changes. Method 2 and method 3 have no BD-rate impact under the current CTC.

# References

- VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

- C.-M. Tsai, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1.2.1: Constraint for binary and ternary partitions,” Document of Joint Video Experts Team, JVET-L0081.

- C.-M. Tsai, S.-T. Hsiang, C.-W. Hsu, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE1-related: Picture boundary CU split satisfying the VPDU constraint,” Document of Joint Video Experts Team, JVET-M0888.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).