| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0297-v3 |
| --- | --- |

| Title: | CE9-related: Simplification for DMVR padding process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yucheng.lin, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In VTM 5.0, decoder motion vector refinement (DMVR) divides a coding block (CB) into subblocks and performs subblock-based motion search, but the reference sample padding for DMVR is performed at CB reference area boundaries, which introduces irregularity between subblocks. Additional memory bandwidth or internal storage is required to handle the irregularity. This contribution proposes to perform reference sample padding at subblock reference area boundaries to totally remove the irregularity. The BD-rate impact is reportedly close to zero (0.00% after rounding).

Introduction

In VTM5.0 [1], DMVR performs sample padding at CB reference area boundaries, where the CB reference area is defined as all reference samples needed for the current CB to perform motion compensation (MC) with fractional sample interpolation by 8-tap luma or 4-tap chroma filters based on the initial list 0 (L0) and list 1 (L1) motion vectors (MVs) of the DMVR CB. As shown in Figure 1, the current luma CB size is 128x128, the luma CB reference area is 135x135, the luma subblock size is 16x16.

Figure 1: Example of padding at CB reference area boundaries

After the MV refinement of each subblock is done, the final MC prediction samples of the subblock are generated using the CB reference area. To support MC with +-2 integer-sample MV refinement, each subblock needs a 27x27 subblock extended reference area. In case that the subblock extended reference area is partially outside CB reference area, padding at CB reference area boundaries is performed, but this introduces two problems. First, irregularity between subblocks occurs. For a multi-standard hardware decoder, which commonly supports the AVC standard, the basic processing unit for MC is usually 16x16. In this case, the padding design for right-most and bottom-most 16x16 subblocks is different as shown in Figure 1, where the subblock B needs to pad samples at the right side and the subblock C needs to pad samples at the bottom side. Even for a stand-alone VVC hardware decoder, it processes MC virtual pipeline data unit (VPDU) by VPDU (i.e., 64x64 luma samples and 32x32 Cb/Cr samples, denoted as 64x64-L/32x32-C). The irregularity still occurs between different VPDUs. For example, for a 128x64-L/64x32-C coding unit (CU), which has two VPDUs, the padding processes of the two VPDUs are different because of padding at CB reference area boundaries. The second problem is that the external memory bandwidth or the internal storage size needed for DMVR is increased. As shown in Figure 2, the subblocks D and E are horizontally adjacent. If performing fractional MC, when processing subblock E, the overlapped region between the two subblocks needs to be either re-fetched from external memory or stored in internal storage after processing subblock D. To support DMVR with padding at CB reference area boundaries, the overlapped region is enlarged from the overlapped region of the two subblock reference areas to the overlapped region of the two subblock extended reference areas.

Figure 2: Illustration of increased external bandwidth or internal storage

The padding for chroma in DMVR can be derived by analogy.

Proposed method

To reduce the padding irregularity between subblocks and to reduce the memory bandwidth or internal storage for DMVR, it is proposed to perform padding at subblock reference area boundaries. As shown in Figure 3, during MV refinement and MC after MV refinement for each subblock, only 23x23 samples within the subblock reference area are used. With the proposed method, more regular and simpler hardware design for DMVR can be achieved. Moreover, the memory bandwidth or internal storage is reduced.

Figure 3: Illustration of proposed method

The bandwidth savings of proposed method for a 128x128 luma CB and a 64x64 luma CB in comparison with padding at CB reference boundaries are shown in Table 1. It is assumed that a multi-standard decoder performs 16x16 MC without cache. In case of padding at CB reference boundaries, for each of the subblocks at the four corners of the CB, 25x25 reference samples are needed; for each of the subblocks at non-corner CU boundaries, 27x25 or 25x27 reference samples are needed; for each of the remaining subblocks inside the CU, 27x27 reference samples are needed. In case of padding at subblock reference area boundaries, for each subblock, 23x23 reference samples are needed.

Table 1: Bandwidth saving compared with padding at CB reference area boundaries

| Luma CB size | Proposed method |
| --- | --- |
| 128x128 | 24.67%* |
| 64x64 | 21.75% |

* = 100% - (23*23*64 / (25*25*4+25*27*24 +27*27*36))

Experimental results

The proposed method is implemented on top of VTM5.0. All the tests are conducted under common test conditions (CTC) [2] for evaluation. Test results of proposed method are summarized in Table 2.

Table 2: Results of proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | -0.01% | -0.13% |  | 0.02% |  | 100% |  | 103% |  |
| Class A2 | 0.00% | 0.06% |  | 0.06% |  | 100% |  | 104% |  |
| Class B | 0.02% | 0.02% |  | 0.04% |  | 100% |  | 103% |  |
| Class C | -0.01% | 0.03% |  | -0.06% |  | 99% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% |  | 0.01% |  | 100% |  | 103% |  |
| Class D | 0.00% | -0.02% |  | -0.03% |  | 99% |  | 108% |  |
| Class F (optional) | 0.01% | 0.01% |  | -0.03% |  | 98% |  | 110% |  |

Conclusions

In this contribution, it is proposed to perform subblock-based padding in DMVR. It is shown that the proposed method can achieve more regular hardware design and reduce the external memory bandwidth. The proposed method has almost no impact on coding efficiency.

References

[1] VTM5.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).