| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 Jul. 2019 | Document: JVET-O0299_r1 |
| --- | --- |

| Title: | AHG17: APS support for default and user defined scaling matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Jie Zhao Jaehyun Lim Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Tel: Email | 1-858-635-5300 seethal.paluri@lge.com jie.zhao@lge.com jaehyun.lim@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

This contribution proposes to add the support for signaling scaling list matrices in the Adaptation Parameter Set (APS). The existing design, that was adopted in the Geneva meeting, has three modes, i.e., OFF, DEFAULT and USER_DEFINED. The OFF mode specifies that no scaling list data is applied to the transform block. The DEFAULT mode specifies that a fixed value is used for generating the scaling matrices. The USER_DEFINED mode specifies the scaling matrices based on the block size, prediction mode and color component. The total number of scaling matrices to be supported in VVC is 44 (30 signalled + 14 DC values) using approximately 3000bits, which is a significant increase from the 28 (20 signalled + 8 DC values) from HEVC. Currently, the scaling list data is signaled in the Sequence Parameter Set and may also be conditionally present in the Picture Parameter Set (PPS). It is asserted that by signaling the scaling list data in the APS, redundant signaling of the same data in the SPS and PPS would be unnecessary and should therefore be removed.

# Introduction

The scaling matrices defined in the current WD is enabled in the sequence parameter set (SPS) using the flag scaling_list_enabled_flag. When this flag is enabled, scaling list is used for the scaling process for transform coefficient and when it is disabled, scaling list is not used for the scaling process for transform coefficient (i.e., OFF mode). A secondary flag i.e., sps_scaling_list_data_present_flag is parsed which indicates if the scaling list data is present in the SPS. In addition to the SPS signaling, the scaling list data may also be present in the picture parameter set (PPS). When the flag pps_scaling_list_data_present_flag is enabled in the PPS, the scaling list data is present in the PPS. When the scaling list data is present both in the SPS and in the PPS, then the data from the PPS is used for frames that refer to the active PPS. When the scaling_list_enable_flag is enabled but the scaling list data is only present in the SPS but not in the PPS, then the scaling list data from the SPS is referred to by the frames. When the scaling_list_enable_flag is enabled and the scaling list data is not present either in the SPS or in the PPS, then the DEFAULT mode is used. Additionally, the use of the DEFAULT mode can also be signaled within the scaling list data itself. The USER DEFINED mode is used when the scaling list data is explicitly signaled. The Scaling Factor for a given transform block is determined using the information signaled in the scaling list. This contribution proposes to signal the scaling list data in the adaptation parameter set (APS).

# Background

The scaling list data has been adopted into Versatile Video Coding (VVC) at the 14th meeting in Geneva. For the USER_DEFINED case, the number of matrices to be signaled are as follows:

- MatrixType: 30 = 2 (2 for intra & IBC/inter) × 3 (Y/Cb/Cr components) × 5 (square TB size: from 4×4 to 64×64 for luma, from 2×2 to 32×32 for chroma)

- MatrixType_DC: 14 = 2 (2 for intra & IBC/inter × 1 for Y component) × 3 (TB size: 16×16, 32×32, 64×64) + 4 (2 for intra & IBC/inter × 2 for Cb/Cr components) × 2 (TB size: 16×16, 32×32)

The DC values are separately coded for the following scaling matrices: 16x16, 32x32, and 64x64. For TB size smaller than 8x8, all elements in one scaling matrix are signaled. If the TB has a size greater than or equal to 8x8, only 64 elements in one 8x8 scaling matrix are signaled as a base scaling matrix. In order to obtain larger square matrices greater than 8x8, the base 8x8 matrix is up-sampled to correspond to the required size. When the DEFAULT mode is used the scaling matrices are set equal to 16. Therefore, VVC supports 44 different matrices while HEVC only supported 28 matrices. As the number of scaling matrices to be supported is more extensive than HEVC, and to avoid redundant signaling in the SPS and/or in the PPS, the Adaptation Parameter Set has been identified a more practical choice for the signaling of the scaling data. This would avoid unnecessary and redundant signaling of the scaling data.

# Proposal

Tables 1 -2 below show the proposed method to signal the scaling matrices in the APS. Scaling list data is signaled in the APS only instead of SPS and conditional PPS signaling. The APS ID is signaled in the slice header as shown in Table 4 below.

If the scaling_list_enable_flag is enabled in the SPS and an APS ID is not signaled in the Slice Header, then the DEFAULT scaling matrices are to be used. The Tables 1-5 shown below show the details of proposed syntax.

Table 1 shows the proposed APS structure used to carry the Scaling list data.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data(adaptation_parameter_set_id ) |  |
| else if ( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if ( aps_params_type = = SCALING_ APS ) // 2 |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_params_type specifies the type of APS parameters carried in the APS as specified in the Table 2 shown below.

Table 2 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | SCALING list data parameters |
| 3..7 | Reserved | Reserved |

The Table 1 show the structure of APS which incorporates the scaling list data. A data type may be assigned to signify the presence of the Scaling List data.

Table 3 shows an excerpt from the SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| … |  |
| } |  |

Where the semantics may be described as:

scaling_list_enabled_flag equal to 1 specifies that a scaling list is used for the scaling process for transform coefficients. scaling_list_enabled_flag equal to 0 specifies that scaling list is not used for the scaling process for transform coefficients.

sps_scaling_list_data_present_flag equal to 1 specifies that the scaling_list_data( ) syntax structure is present in the SPS. sps_scaling_list_data_present_flag equal to 0 specifies that the scaling_list_data( ) syntax structure is not present in the SPS. When not present, the value of sps_scaling_list_data_present_flag is inferred to be equal to 0.

Aditionally, the signalling of the scaling list data in the PPS can be removed as shown in the Table 4 shown below.

Table 4 shows the the modified structure of the PPS.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … | ue(v) |
| pps_scaling_list_data_present_flag | u(1) |
| if(pps_scaling_list_data_present_flag) { |  |
| scaling_list_data( ) |  |
| } |  |
| … |  |
| } |  |

Where the semantics may be described as:

pps_scaling_list_data_present_flag equal to 1 specifies that the scaling list data used for the pictures referring to the PPS are derived based on the scaling lists specified by the active SPS and the scaling lists specified by the PPS. pps_scaling_list_data_present_flag equal to 0 specifies that the scaling list data used for the pictures referring to the PPS are inferred to be equal to those specified by the active SPS. When scaling_list_enabled_flag is equal to 0, the value of pps_scaling_list_data_present_flag shall be equal to 0. When scaling_list_enabled_flag is equal to 1, sps_scaling_list_data_present_flag is equal to 0 and pps_scaling_list_data_present_flag is equal to 0, the default scaling list data are used to derive the array ScalingFactor as as specified in clause 7.4.3.14 the scaling list data semantics.

Table 5 shows the the structure of the slice header.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if( scaling_list_enabled_flag ) { |  |
| slice_scaling_list_present_flag | u(1) |
| if(slice_scaling_list_present_flag) { |  |
| slice_scaling_list_aps_id | u(5) |
| } |  |
| } |  |
| … |  |
| byte_alignment( ) |  |
| } |  |

Where the semantics may be described as:

When present, the value of each of the slice header syntax elements slice_pic_parameter_set_id, slice_pic_order_cnt_lsb, and slice_temporal_mvp_enabled_flag shall be the same in all slice headers of a coded picture.

slice_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of slice_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

slice_scaling_list_present_flag equal to 1 specifies that scaling list matrices is present for the current slice. slice_scaling_list_present_flag equal to 0 specifies that the default scaling list data are used to derive the array ScalingFactor as described in scaling list data semantics as specified in 7.4.3.14. When slice_scaling_list_enabled_flag is not present, it is inferred to be equal to 0.

slice_scaling_list_aps_id specifies the adaptation_parameter_set_id of the SCALING DATA  APS that the slice refers to. The TemporalId of the SCALING DATA APS NAL unit having adaptation_parameter_set_id equal to slice_scaling_list_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. If slice_scaling_list_aps_id is not present, it is inferred to be 0.

When multiple SCALING DATA APSs with the same value of adaptation_parameter_set_id are referred to by two or more slices of the same picture, the multiple SCALING DATA APSs with the same value of adaptation_parameter_set_id shall have the same content.

#### References

[1] O. Chubach, C.-Y. Lai, C-Y Checn, T-Z Chaung, C-W Hsu, Y-W Huang, S-M Lei, T. Toma, K. Abe, S-C Lim, J. Kang, “CE7-related: Support for quantization matrices for VVC,” document JVET-N0847, 14th JVET meeting: Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).