| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0316 |
| --- | --- |

| Title: | Non-CE3: Unification of ISP and Non-ISP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dong Jiang, Jucai Lin, Feiyang Zeng and Cheng Fang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution gives four improvement points of ISP in other to combine the intra prediction process of ISP and non-ISP. All proposed methods are based on VTM5.0. Firstly, the proposal analyzes the similarities and differences between sub-block splitting (ISP) and non-sub-block splitting in VTM 5.0. Then, four improvement points of ISP are proposed for the corresponding four comparison points. The result is that compression ratio of the encoding is improved by unifying the intra prediction process of ISP and non-ISP. The results of the proposal and VTM5.0 under three experiments are compared. The average BD-rate gains of Test1 are -0.01%, 0.03% and -0.02% for luminance and both chrominance components. The average BD-rate gains of Test2 are 0.00%, 0.03% and -0.02% for luminance and both chrominance components. The average BD-rate gains of Test3 are 0.01%, 0.08% and 0.05% for luminance and both chrominance components.

Algorithm description

Current Technology

The method proposed is based on VTM5.0 [1]. Firstly, the sub-block splitting technique is described below.

ISP splitting technique has two division modes: vertical split mode and horizontal split mode, only for CU blocks lager than 4x4. The 4x8 and 8x4 CU blocks are divided into 2 identical sub-blocks, and other sized CU blocks are divided into 4 identical sub-blocks. Each CU block can only perform one ISP split mode in one division. Figure 1 is an example of dividing a CU into 4 sub-blocks.

Figure 1: ISP splitting Examples in VTM5.0

Then, analyze the similarities and differences between sub-block splitting (ISP) and non-sub-block splitting in VTM5.0, which are listed separately below.

- Wide angle mode acquisition

- The CU of the non-sub-block partitioning determines the wide angle mode according to the relationship between the width and height of the current CU block and the prediction mode.

- Each sub-block of a sub-block partitioned CU determines the wide angle mode according to the relationship between the width and height of the current sub-block and the prediction mode.

- Number of reference samples

- The number of reference samples for a non-sub-block partitioned CU is 2*（Width of current CU + Height of current CU）+ 1.

- The number of reference samples per sub-block of a sub-block partitioned CU is also 2*（Width of current CU + Height of current CU）+ 1.

- Reference sample filtering in diagonal mode

- The diagonal mode of the CU of the non-sub-block partitioning is smoothed by the filter [1, 2, 1] when the condition that Width of current CU * Height of current CU > 3 is satisfied.

- The diagonal mode of the CU of the sub-block partitioning is not filtered.

- PDPC

- The non-sub-block partitioned CU block does PDPC for the Planer, DC, angle mode greater than 75 degrees, and angle mode less than 11 degrees, and does not limit the size of CU block.

- Each sub-block of the sub-block partitioned CU does not do PDPC for all modes.

Proposed Algorithm

This proposal can improve the compression ratio of the encoding by unifying the intra prediction process of ISP and non-ISP. Four improvement points of ISP are proposed for the above four different parts.

- The wide angle mode of the ISP block is obtained according to the sub-block size and prediction mode.

Considering that the wide angle mode is strongly correlated with the current coding block size, and if the sub-block partitioning is performed, the current coding block is the sub-block, therefore, it is theoretically reasonable to use the sub-block size and sub-block prediction mode to obtain the sub-block wide angle mode.

- The number of reference samples of the ISP block is obtained according to the size of sub-block.

When a sub-block is being predicted, too many reference samples are selected. Theoretically, 2*(Width of current CU + Height of current CU) + 1 samples is sufficient, and the extra reference samples will consume the memory space of the decoder.

- The reference samples in the ISP block diagonal mode use the same filtering method as non-ISP.

The purpose of reference samples filtering is to avoid the phenomenon that the prediction block obtained by using the reference samples may generate a direction boundary, so that the visual effect is improved by filtering. In general, the smaller the prediction block, the less filtering is needed, and the closer to the prediction mode in the diagonal direction, the more filtering is required. The sub-block diagonal mode filtering mode and the non-sub-block diagonal are consistent in filtering conditions and filters in this proposal.

- Add PDPC method to ISP

In general, after the current block performs mode prediction, PDPC compensation is performed on the prediction block of the current block, which can further improve the accuracy of the prediction block. But for blocks that are too small, the effect is not obvious. Therefore, in this proposal, the PDPC prediction compensation is only for prediction blocks whose the sub-blocks are wider and higher than or equal to 4, and the processing mode is the same as that of the non-sub-block PDPC.

Experimental results

Experimental Scheme

This proposal sets up four sets of experiments. The experimental settings are as follows:

Test1. Improvement 1 + Improvement 2

Test2. Improvement 1 + Improvement 2 + Improvement 3

Test3. Improvement 1 + Improvement 2 + Improvement 3 + Improvement 4

Experimental Condition

The proposed method is implemented on top of VTM5.0 software. The simulations are performed on Linux cluster using VTM AI configurations under the common test condition described in JVET-N1010 [2].

Experimental Data

Table 1, 2, and 3 show experimental results of Test1, Test2, and Test3 compared with VTM5.0, respectively. The experimental settings of the three tests are the same. All sequences from Class A to Class E have been tested.

Table 1 Coding performance of Test1 over VTM-5.0

Table 2 Coding performance of Test2 over VTM-5.0

Table 3 Coding performance of Test3 over VTM-5.0

Conclusion

In this contribution, four improvement points of ISP are proposed. Combining the results between VTM5.0 and the proposed method, the average BD-rate can gains 0.01%, 0.08% and 0.05% for luminance and both chrominance components, while brings negligible increase of encoding and decoding complexity.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Suehring “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

Dahua may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).