| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0322 |
| --- | --- |

| Title: | Non-CE3: Simplification of PDPC for chroma component |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Shuai Wan, Yanzhuo Ma, Jinkun Guo, Fuzheng Yang Yuanfang Yu Yang Liu Ming Li | Email: | jyhuo@mail.xidian.edu.cn swan@nwpu.edu.cn yzma@mail.xidian.edu.cn guojinkun@mail.nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com myron.li@oppo.com |
| Source: | Northwestern Polytechnical University, Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This document proposes two PDPC modifications in chroma component prediction. In test 1, PDPC is not applied for all chroma prediction modes. In test 2, PDPC is applied for Planar/DC/horizontal/vertical mode in chroma prediction.

Simulation results reportedly show BD rate on Y, Cb and Cr components and encoding/decoding time comparison over VTM5.0:

- Test 1, PDPC is not invoked in intra chroma prediction:

  - For AI configuration: 0.14%, -0.34%, and -0.43%, with 100% EncT, 99% DecT;

  - For RA configuration: 0.09%, -0.69%, and -0.70%, with 100% EncT, 100% DecT.

- Test 2, in intra chroma prediction, PDPC is invoked for Planar, DC, horizontal/vertical only:

  - For AI configuration: 0.03%, -0.16%, and -0.14%, with 100% EncT, 99% DecT;

  - For RA configuration: 0.02%, -0.26%, and -0.32%, with 100% EncT, 100% DecT;

Introduction

In VTM5.0, the position dependent intra prediction combination (PDPC) is applied to both intra luma and chroma predicted samples for all block sizes when intra prediction mode is one of DC, Planar, horizontal, vertical, an angular mode which is less than or equal to mode 10, and an angular mode which is greater than or equal to mode 58.

Proposed method

In current VTM, luma and chroma components are predicted individually. In this contribution, two kinds of PDPC modifications are proposed. In Test 1, PDPC is only not applied for chroma prediction modes. In Test 2, for chroma predicted samples, PDPC is only applied for DC mode, Planar mode, horizontal mode or vertical mode.

Experimental results

Experiments are performed based on VTM 5.0 software [1] under JVET common test condition [2].

Table 1 and Table 2 show experimental results of the each proposed method compared to VTM 5.0.

Table 1. Results of Test 1 over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.10% | -0.37% | 100% | 100% |
| Class A2 | 0.17% | -0.35% | -0.17% | 100% | 99% |
| Class B | 0.12% | -0.38% | -0.52% | 100% | 99% |
| Class C | 0.13% | -0.37% | -0.44% | 100% | 100% |
| Class E | 0.19% | -0.45% | -0.58% | 98% | 98% |
| Overall | 0.14% | -0.34% | -0.43% | 100% | 99% |
| Class D | 0.14% | -0.30% | -0.56% | 100% | 100% |
| Class F (optional) | 0.14% | -0.19% | -0.44% | 101% | 100% |
|  | Random Access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.57% | -0.38% | 100% | 100% |
| Class A2 | 0.10% | -0.54% | -0.31% | 100% | 100% |
| Class B | 0.10% | -0.82% | -1.15% | 100% | 100% |
| Class C | 0.08% | -0.72% | -0.66% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | -0.69% | -0.70% | 100% | 100% |
| Class D | 0.08% | -0.19% | -0.57% | 100% | 100% |
| Class F (optional) | 0.10% | -0.19% | -0.28% | 100% | 100% |

Table 2. Results of Test 2 over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.12% | -0.03% | 100% | 99% |
| Class A2 | 0.05% | -0.15% | -0.08% | 100% | 101% |
| Class B | 0.03% | -0.09% | -0.13% | 99% | 99% |
| Class C | 0.03% | -0.12% | -0.16% | 101% | 100% |
| Class E | 0.04% | -0.37% | -0.27% | 100% | 98% |
| Overall | 0.03% | -0.16% | -0.14% | 100% | 99% |
| Class D | 0.02% | -0.02% | -0.24% | 101% | 100% |
| Class F (optional) | 0.04% | -0.12% | -0.06% | 101% | 100% |
|  | Random Access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.30% | -0.23% | 100% | 100% |
| Class A2 | 0.04% | -0.20% | -0.12% | 100% | 100% |
| Class B | 0.02% | -0.29% | -0.43% | 100% | 101% |
| Class C | 0.00% | -0.26% | -0.40% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.26% | -0.32% | 100% | 100% |
| Class D | 0.05% | -0.31% | -0.05% | 100% | 100% |
| Class F (optional) | 0.03% | -0.20% | -0.01% | 100% | 100% |

Conclusions

This contribution reports two restrictions of PDPC to reduce complexity. We propose to study this method in CE.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-5.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, June 2019.

[3] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, June 2019.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).