| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0334 |
| --- | --- |

| Title: | AHG8/AHG12: On sub-picture partitioning support with layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, signaling a layout information of sub-picture layers and a mapping information from a sub-picture region to an independent sub-picture layer in VPS is proposed. Using the information, one or more sub-picture layers can collectively form a picture, or a sub-set of sub-picture layers, which cover a sub-region of a picture, can be extracted.

Introduction

In Geneva meeting, a layer concept (more specifically independent sub-picture layers) without inter-layer referencing was adopted. Based on the captures from the meeting note, decoded pictures of the same timestamp and of different layers can be composed into the same output picture, or outputted separately for use by the system for different purposes (e.g. view scalability, texture versus depth. etc.).

In this contribution, signaling a layout information of sub-picture layers and a mapping information from a sub-picture region to an independent sub-picture layer in VPS is proposed.

Proposal

In this contribution, the term sub-picture indicates a rectangular region of samples, which is independently coded. One or more sub-pictures can collectively form a picture and can be extracted from a picture. The sub-picture partitioning is isolated from slice/tile/brick partitioning. Thus, each sub-picture may or may not be partitioned into slice/tile/brick(s), with its own tile partitioning layout. Each sub-picture is independent coded as a picture and mapped to a layer. Thus, a picture consists of one or more independent sub-picture layers, which are independently coded as pictures and cover sub-regions of the picture. Any in-loop filtering across sub-picture boundaries is disabled.

The partitioning and layout information of sub-pictures are signaled in VPS, which conventionally contains layer-related information. Mapping information to connect each sub-picture region to each layer is also needed. Fig. 1 shows a simple example of sub-picture partitioning with a layer mapping information, which includes the offset value of the left-top position and the region size of each sub-picture (layer). The sub-picture size signalled in VPS does not indicate the actual decoded picture size, but indicates the region size covered by the sub-picture (layer) in the (input) picture. The decoded picture size of each sub-picture (layer) is identical to the picture size signaled in SPS. Using the layout information, one or more sub-picture layers can collectively form a picture, or a sub-set of sub-picture layers, which cover a sub-region of a picture, can be extracted.

Fig.2 shows an example of the sequence structure with multiple (independent) sub-picture layers. Any prediction structure within an independent sub-picture layer can be supported by the RPL signaling, based on the POC delta indication. Any prediction across sub-picture layers needs further studies.

In this contribution, each sub-picture region is mapped to each layer. However, JVET-O0336 introduces the case that each one or more layers with different resolutions are mapped to each sub-picture region, to support a region-wise scalability.

Figure 1. An example of sub-picture (layer) partitioning

Figure 2. An example of a sequence structure with (independent) sub-picture layers

Proposed specification text

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| vps_num_sub_pictures_in_pic | u(8) |
| for( i = 0; i < vps_num_sub_pictures_in_pic; i++) { |  |
| vps_sub_pic_id[ i ] | u(8) |
| vps_sub_pic_width_in_luma_samples[ i ] | u(16) |
| vps_sub_pic_height_in_luma_samples[ i ] | u(16) |
| if( i > 0 ) { |  |
| vps_sub_pic_pos_x_in_luma_samples[ i ] | u(16) |
| vps_sub_pic_pos_y_in_luma_samples[ i ] | u(16) |
| } |  |
| } |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_sub_pic_id[ i ] | u(8) |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

vps_num_sub_pictures_in_pic specifies the number of sub-pictures in each picture referring to the VPS.

vps_sub_pic_id[ i ] specifies the sub-picture identifier of the i-th sub-picture.

vps_sub_pic_width_in_luma_samples[ i ] specifies the width of the i-th sub-picture in units of luma samples. vps_sub_pic_width_in_luma_samples shall not be equal to 0.

vps_sub_pic_height_in_luma_samples[ i ] specifies the height of the i-th sub-picture in units of luma samples. vps_sub_pic_height_in_luma_samples shall not be equal to 0.

vps_sub_pic_pos_x_in_luma_samples[ i ] specifies the column position of the first pixel of the i-th sub-picture.

vps_sub_pic_pos_y_in_luma_samples[ i ] specifies the row position of the first pixel of the i-th sub-picture.

vps_layer_sub_pic_id[ i ] specifies the sub-picture identifier associated with the i-th layer.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).