| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0351 |
| --- | --- |

| Title: | Non-CE3 : Clean-up intra type signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Junghak Nam Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, junghak.nam, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes two clean-up methods on intra type signaling. The first method changes the intra type signaling order. In this method, after MPM flag, MRL index and ISP flag are signaled. The proposed intra type signaling order change provides the coding performance 0.01% and 0.00% BD-rate in AI and RA configuration, respectively. The second method signals a unified intra type by aligning the MPM flag, reference index for MRL, and split flag for ISP. The proposed unified intra type signaling method provides the coding performance 0.00% and XX% BD-rate in AI and RA configuration, respectively without increasing the complexity of encoding and decoding.

Introduction

This contribution proposes a clean-up intra syntax signaling structure. Figure 1 shows the current intra syntax signaling design. As shown in Figure 1, the MRL [1] index and the ISP [2] flag are first signaled before MPM flag. However, since MRL or ISP mode can only be selected among MPM candidates, it makes sense to signal the MRL index and ISP flag after signaling the MPM flag. Therefore, the first contribution proposes to change intra type signaling order.

The second contribution proposes a simple intra syntax signaling structure. In order to signal three intra syntax elements, the corresponding signaling condition and results should be checked each time, as shown in Figure 1. In the current VVC draft, three syntax elements (intra_luma_ref_idx, intra_subpartitions_mode_flag, and intra_luma_mpm_flag) are used to indicate the intra type except for pcm, bdpcm, and mip.

- intra_luma_ref_idx specifies the intra prediction reference line index.

- intra_subpartitions_mode_flag specifies whether the current intra coding unit is partitioned into the number of transform block sub-partitions.

- intra_luma_mpm_flag specifies whether the intra prediction mode is inferred from a neighbouring intra predicted coding unit.

In addition, since signaling process is performed sequentially, the complexity of the decoder is high. Therefore, this contribution proposes to integrate intra type signaling with the unified intra syntax.

Figure 1. Current intra syntax parsing design

Proposed method

Change on intra type signaling order

Table 1 represents the modified syntax of the proposed intra type signaling order change.

Table 1. Change on intra type signaling order

| VVC Draft | Proposed |
| --- | --- |
| coding_unit() | coding_unit() |
| ... | ... |
| if ( intra_mip_flag ) | if ( intra_mip_flag ) |
| else { | else { |
| if( sps_mrl_enabled_flag && (( y0 % CtbSizeY ) > 0 )) | intra_luma_mpm_flag |
| intra_luma_ref_idx | if( intra_luma_mpm_flag ) { |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) | if( sps_mrl_enabled_flag && (( y0 % CtbSizeY ) > 0 )) |
| intra_subpartitions_mode_flag | intra_luma_ref_idx |
| if ( intra_subpartitions_mode_flag = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) | if ( sps_isp_enabled_flag && intra_luma_ref_idx = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |
| intra_subpartitions_split_flag | intra_subpartitions_mode_flag |
| if( intra_luma_ref_idx = = 0 && intra_subpartitions_mode_flag = = 0 ) | if ( intra_subpartitions_mode_flag = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |
| intra_luma_mpm_flag | intra_subpartitions_split_flag |
| if( intra_luma_mpm_flag ) { | if( intra_luma_ref_idx = = 0 ) |
| if( intra_luma_ref_idx = = 0 ) | intra_luma_not_planar_flag |
| intra_luma_not_planar_flag | if( intra_luma_not_planar_flag ) |
| if( intra_luma_not_planar_flag ) | intra_luma_mpm_idx |
| intra_luma_mpm_idx | } else |
| } else | intra_luma_mpm_remainder |
| intra_luma_mpm_remainder | } |
| } |  |

Unification on intra type signaling

Table 2 represents the modified syntax of the proposed joint syntax element intra_luma_type_idx compared to VVC Draft.

Table 2. Unification on intra type signaling

| VVC Draft | Proposed |
| --- | --- |
| coding_unit() | coding_unit() |
| ... | ... |
| if ( intra_mip_flag ) | if ( intra_mip_flag ) |
| else { | else { |
| if( sps_mrl_enabled_flag && (( y0 % CtbSizeY ) > 0 )) | intra_luma_type_idx |
| intra_luma_ref_idx | if ( intra_luma_type_idx > 1 ) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) | intra_luma_ref_split_idx |
| intra_subpartitions_mode_flag |  |
| if ( intra_subpartitions_mode_flag = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) |  |
| intra_subpartitions_split_flag |  |
| if( intra_luma_ref_idx = = 0 && intra_subpartitions_mode_flag = = 0 ) |  |
| intra_luma_mpm_flag |  |
| if( intra_luma_mpm_flag ) { | if( intra_luma_type_idx ! = non-MPM ) { |
| if( intra_luma_ref_idx = = 0 ) | if( intra_luma_ref_idx = = 0 ) |
| intra_luma_not_planar_flag | intra_luma_not_planar_flag |
| if( intra_luma_not_planar_flag ) | if( intra_luma_not_planar_flag ) |
| intra_luma_mpm_idx | intra_luma_mpm_idx |
| } else | } else |
| intra_luma_mpm_remainder | intra_luma_mpm_remainder |
| } | } |

Instead of parsing the intra_luma_ref_idx, intra_subpartitions_mode_flag, intra_subpartitions_split_flag, and intra_luma_mpm_flag, a joint syntax element intra_luma_type_idx uses truncated unary binarization. The complete binarization is shown in Table 3.

Table 3. Proposed binarization

| intra_luma_type_idx | Intra type | Binarization |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MRL & ISP enabled | MRL enabled | ISP enabled | MRL & ISP disabled |
| 0 | MPM | 0 | 0 | 0 | 0 |
| 1 | Non-MPM | 10 | 10 | 10 | 1 |
| 2 | MRL flag | 110 | 11 | - | - |
| 3 | ISP flag | 111 | - | 11 | - |

- intra_luma_type_idx specifies whether mpm, non-mpm, MRL, or ISP is used for the current intra prediction unit.

- intra_luma_ref_split_idx specifies the intra prediction reference line index for MRL or the intra sub-partitions split type for ISP.

Figure 2 shows the proposed intra syntax signaling design.

Figure 2. Proposed intra syntax parsing design

Experimental result

The proposed method has been implemented using VTM-5.0 and under the common test conditions [3]. The experimental results from two sub-tests are evaluated.

- Test 1: Change on intra type signaling order

Table 4. Coding performance of the proposed method (AI)

Table 5. Coding performance of the proposed method (RA)

- Test 2: Unification on intra type signaling

Table 6. Coding performance of the proposed method (AI)

Table 7. Coding performance of the proposed method (RA)

Conclusion

In conclusion, it is proposed to clean-up methods on intra type signaling. The first method changes the intra type signaling order. The second method combines three intra syntaxes; mpm flag, reference index for MRL, and split flag for ISP into one unified syntax. The proposed intra type signaling order change provides the coding performance 0.01% and 0.00% BD-rate in AI and RA configuration, respectively. The proposed joint signaling provides the coding performance 0.00% and XX% BD-rate in AI and RA configuration, respectively without increasing the complexity of encoding and decoding. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- B. Bross, et al., “CE3: Multiple reference line intra prediction,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0283, 12th Meeting, Macao, CN, 3–12 Oct. 2018.

- S. De-Luxán-Hernández, “CE3: Intra Sub-Partitions coding Mode”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11JVET-M0102, 13th Meeting, Marrakech, Ma, 9–18 Jan. 2019.

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).