| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0352-v2 |
| --- | --- |

| Title: | Non-CE6: TU/TB-level LFNST index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes two LFNSTLFNST index coding to resolve the parsing dependency (latency problem) at 128x128 CU: a) TU-level LFNST index coding, and b) TB-level LFNST index coding rather than CU-level LFNST index coding. For TU-level LFNST index coding, lfnst_idx is coded at the end of transform unit. For TB-level LFNST index coding, lfnst_idx is coded per component at the end of residual coding.

It is reported that TU-level LFNST index coding provides average BD-rate differences (Y, U, V) of (-0.01 %,0.01% and 0.02%) for AI, (-0.02%, 0.02%, -0.07%) for RA, and (0.00%, 0.00%, 0.00%) for LB with negligible runtime impacts; TB-level LFNST index coding provides average BD-rate differences (Y, U, V) of (0.15%,0.53% and 0.60%) for AI, (0.02%, 0.29%, 0.30%) for RA, and (0.XX%, 0.XX%, 0.XX%) for LB with negligible runtime impacts.

It is recommended to adopt TU-level LFNST index signaling into the next VTM/VVC WD and study TB-level LFNST index coding in CE for LFNST control flexibility per component.

Introduction

In VVC WD [1], LFNST index (lfnst_idx) is signaled at the CU footer to derive two non-zero transformed coefficients parameters (numSigCoeff, numZeroOutSigCoeff) in all TBs in a CU as illustrated in Table 1-1.

When CU size is 128x128, CU is implicitly split into four 64x64 TUs; First TU cannot start inverse transform process without waiting for LFNST index parsing. It also increases the data buffering by 4 times for a 64x64 VPDU based decoder pipeline. Therefore, VPDU-friendly LFNST index coding is desirable.

Table 1-1 CU-level LFNST index signaling

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| …skip… |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Proposal

To resolve the parsing dependency (latency problem) at 128x128 CU on LFNST index, this contribution proposes a) TU-level LFNST index coding, and b) TB-level LFNST index coding. For TU-level LFNST index coding, lfnst_idx is coded at the end of transform unit. For TB-level LFNST index coding, lfnst_idx is coded per component at the end of residual_coding.

Table 2-1 summarizes TU/TB-level LFNST index coding and CU-level LFNST index coding (VTM-5.0).

Table 2-1: Summary of TU/TB-level LFNST index signaling and CU-level LFNST index signaling in VTM

|  | VTM-5.0 | TU-level signaling | TB-level signaling |
| --- | --- | --- | --- |
| LFNST index signaling | - CU footer - For dual tree, LFNST index for luma and chorma is signaled separately. - For single tree, LFNST index for luma and chorma is signalied | - TU footer (at the end of transform_unit()) - For dual tree, LFNST index for luma and chorma is signaled separately. - For single tree, LFNST index for luma is signaled. LFNST index for chroma is inferred from LFNST index for luma. | - TB footer (at the end of resudal_coding()) - LFNST index is signaled per component. |
| Derivation on numSigCoeff, numZeroOutSigCoeff | - Check all NonTS-TBs in CU | - For luma dual tree and single tree, check one luma NonTS TB. - For chroma dual tree, check two chroma NonTS TBs (Cb/Cr). | - Check one NonTS TB per component. |
| LFNST control flexibility | - CU level | - TU level | - Component level |
| Parsing dependency at 128x128 CU (latency problem) | - Yes (not solved) | - No (solved) | - No (solved) |

Spec Changes for TU-level LFNST index coding

TU-level LFNST index coding applies following modifications into VTM-5.0:

- Signal lfnst_idx at the TU footer with no changes of both context modeling and binarization for LFNST index signaling

  - If a dual tree is enabled, lfnst_idx for luma and chrom are signaled separately.

  - If a dual tree is disabled, lfnst_idx for luma is signaled, and lfnst_idx for chorma is inferred to be equal lfnst_idx for luma.

Syntax changes are as below:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| …skip… |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| …skip… |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] ) { |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| if (((treeType == DUAL_TREE_LUMA ¦¦ treeType == SINGLE_TREE) && tu_cbf_luma[x0][y0]) ¦¦ (treeType == DUAL_TREE_CHROMA && (tu_cbf_cb[x0][y0] ¦¦ tu_cbf_cr[x0][y0] ))){ |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? wc : tbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? wh : tbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > 1 && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … skip … |  |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| if( (!transform_skip_flag[ x0 ][ y0 ] && cIdx==0) ¦¦ cIdx!=0 ) { |  |  |
| if ( ((treeType == DUAL_TREE_LUMA ¦¦ treeType == SINGLE_TREE) && cIdx==0)) ¦¦ ((treeType == DUAL_TREE_CHROMA) && cIdx!=0) |  |  |
| { |  |  |
| numSigCoeff++ |  |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |  |
| numZeroOutSigCoeff++ |  |  |
| } |  |  |
| } |  |
| } |  |
| … skip … |  |
| } |  |  |

lfnst_idx[ x0 ][ y0 ] specifies whether and which one of the two low frequency non-separable transform kernels in a selected transform set is used. lfnst_idx[ x0 ][ y0 ] equal to 0 specifies that the low frequency non-separable transform is not used. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left sample of the considered transform block relative to the top-left sample of the picture.

When lfnst_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

Spec changes for TB-level LFNST index coding

TB-level LFNST index coding applies following modifications into VTM-5.0:

- Signal lfnst_idx per component at the TB footer with changes of context modeling for LFNST index signaling:

  - Context variable for first bin of LFNST index is assigned depending on cIdx

  - No binarization changes

Syntax changes are as below:

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| …skip… |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| numSigCoeff = 0 |  |  |
| numZeroOutSigCoeff = 0 |  |  |
| … skip … |  |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| if( (!transform_skip_flag[ x0 ][ y0 ] && cIdx==0) ¦¦ cIdx!=0 ) { |  |  |
| numSigCoeff++ |  |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |  |
| numZeroOutSigCoeff++ |  |  |
| } |  |  |
| } |  |
| … skip … |  |
| } |  |
| … skip … |  |
| if( Min( tbWidth, tbHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > 1 && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ][cIdx] | ae(v) |
| } |  |
| } |  |  |

lfnst_idx[ x0 ][ y0 ][ cIdx ] specifies whether and which one of the two low frequency non-separable transform kernels in a selected transform set is used for the associated transform block with cIdx. lfnst_idx[ x0 ][ y0 ][ cIdx ] equal to 0 specifies that the low frequency non-separable transform is not used for the associated transform block with cIdx. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left sample of the considered transform block relative to the top-left sample of the picture. The array index cIdx specifies an indicator for the colour component; it is equal to 0 for Y, 1 for Cb, and 2 for Cr.

When lfnst_idx[ x0 ][ y0 ][ cIdx ] is not present, it is inferred to be equal to 0.

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| lfnst_idx[ ][ ][ ] | cIdx==0 ? 0 : 1 ( tu_mts_idx[ x0 ][ y0 ] = = 0 && treeType != SINGLE_TREE ) ? 1 : 0 | bypass | na | na | na | na |

Simulation Results

The proposed methods have been implemented on top of VTM-5.0 and following simulations were conducted under common test condition [2]:

- Test1:	VTM-5.0 vs TU-level LFNST index coding

- Test2: VTM-5.0 vs TB-level LFNST index coding

Table 3-1 to 3-2 summarizes results of each Test1 and Test2, respectively.

Table 3-1: Results of TU-level LFNST index coding (Test1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.05% | 0.04% | 100% | 97% |
| Class A2 | -0.01% | 0.01% | 0.02% | 100% | 96% |
| Class B | -0.01% | -0.02% | 0.03% | 100% | 98% |
| Class C | -0.01% | 0.02% | 0.09% | 101% | 99% |
| Class E | -0.02% | 0.09% | -0.06% | 101% | 99% |
| Overall | -0.01% | 0.01% | 0.02% | 100% | 98% |
| Class D | -0.02% | 0.07% | 0.05% | 100% | 100% |
| Class F | -0.02% | -0.01% | 0.07% | 101% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | -0.11% | 98% | 97% |
| Class A2 | -0.01% | 0.03% | 0.07% | 100% | 97% |
| Class B | 0.00% | 0.04% | -0.05% | 99% | 99% |
| Class C | -0.04% | 0.06% | -0.17% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.02% | -0.07% | 99% | 98% |
| Class D | -0.04% | 0.14% | 0.18% | 100% | 99% |
| Class F | -0.01% | 0.07% | 0.22% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

Table 3-2: Results of TB-level LFNST index coding (Test2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.65% | 0.06% | 98% | 98% |
| Class A2 | 0.19% | 0.91% | 0.20% | 97% | 99% |
| Class B | 0.13% | 0.27% | 0.82% | 99% | 99% |
| Class C | 0.07% | 0.48% | 0.84% | 98% | 99% |
| Class E | 0.16% | 0.56% | 0.85% | 99% | 99% |
| Overall | 0.15% | 0.53% | 0.60% | 98% | 99% |
| Class D | 0.06% | 0.77% | 0.74% | 98% | 98% |
| Class F | 0.05% | 0.42% | 0.46% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.10% | 101% | 98% |
| Class A2 | 0.06% | 0.73% | 0.20% | 102% | 99% |
| Class B | 0.04% | 0.26% | 0.46% | 101% | 99% |
| Class C | 0.00% | 0.20% | 0.31% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.29% | 0.30% | 101% | 99% |
| Class D | 0.03% | 0.33% | 0.42% | 102% | 98% |
| Class F | 0.02% | 0.23% | 0.44% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |

Conclusion

It is recommended to adopt TU-level LFNST index coding into the next VTM/VVC WD and study TB-level LFNST index coding in CE for LFNST control flexibility per component.

Reference

[1] B. Bross, J. Chen, S. Liu, “	Versatile Video Coding (Draft 5), ” JVET-N1001, Apr. 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).