| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0353 |
| --- | --- |

| Title: | Non-CE3 : Simplification on transform selection for multiple reference line (MRL) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Moonmo Koo Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, moonmo.koo, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes two simplification methods on transform selection for multiple reference line (MRL) intra prediction. The first simplification unifies the multiple transform selection (MTS) method for both MRL and ISP. The first method provides 0.02% and 0.02% coding performance for AI and RA configuration, respectively. The second simplification restricts MTS method in MRL intra prediction. The second provides 0.00% and 0.02% coding performance for AI and RA configuration, respectively.

Introduction

In the current VTM, if the current block is ISP [1] mode, DCT2 and DST7 transform cores are implicitly used with the size restriction. However, if the current block is MRL [2], the MTS [3] transform cores (DCT8, DCT7, and DCT2) could be explicitly selected. In this contribution, two simplification methods are proposed. In the first simplification method, both ISP and MRL are only using DST7 and DCT2 transform cores with the same block size restriction which means that their implicit transform core selections are unified. In the second simplification method, MTS is disable for MRL intra prediction.

Proposed method

Implicit transform for MRL

In this contribution, the implicit transform selection method is proposed for MRL intra prediction. This means that if the current block is MRL, both DST7 and DCT2 transform cores are implicitly selected in the same way as used in ISP mode. The horizontal and vertical transform types (trTypeHor and trTypeVer) are derived as follows.

trTypeHor = ( nTbW >= 4 && nTbW < 16 ) ? 1 : 0                     (1)

trTypeVer = (nTbH >= 4 && nTbH < 16 ) ? 1 : 0                      (2)

Disable MTS for MRL

In this method, if the current block is MRL, MTS is disable, which means DCT2 transform core is only used.

Experimental result

The proposed method has been implemented using VTM-5.0 and under the common test conditions [4]. The experimental results from two sub-tests are evaluated.

- Test1: Implicit transform for MRL

Table 1. Coding Performance of the proposed method (AI)

Table 2. Coding Performance of the proposed method (RA)

- Test2: Disable MTS for MRL

Table 3. Coding Performance of the proposed method (AI)

Table 4. Coding Performance of the proposed method (RA)

Conclusion

In conclusion, this contribution proposes two simplification methods on transform selection for MRL intra prediction. The First simplification unifies the MTS method for both MRL and ISP. The second simplification restricts the MTS for MRL intra prediction. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- S. De-Luxán-Hernández, “CE3: Intra Sub-Partitions coding Mode”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11JVET-M0102, 13th Meeting, Marrakech, Ma, 9–18 Jan. 2019.

- B. Bross, et al., “CE3: Multiple reference line intra prediction,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0283, 12th Meeting, Macao, CN, 3–12 Oct. 2018.

- H. Gao, “Non-CE6: Combined Test of JVET-N0172/JVET-N0375/JVET-N0419/JVET-N0420 on Unification of Implicit Transform Selection,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 JVET-N0866, 14th Meeting, Geneva, CH, 19–27 March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).