| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0354-v2 |
| --- | --- |

| Title: | Non-CE6: Context modeling for MTS index coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes two simplified context modeling for MTS index coding to reduce the number of context variables and context-coded bins in worst case: For proposal#1, MTS index (tu_mts_idx) is binarized with Truncated Unary coding (TU), bins with binIdx=0..1 of MTS index are context-coded with single context variable per binIdx and the reminder of bins (binIdx=2..3) are bypassed. For proposal#2, 1st bin (binIdx=0) is context-coded with two context variables based on transform block size, and the 2nd bin (binIdx=1) is context-coded with single context variable, and the reminder of bins is bypassed.

It is reported that proposal#1 reduces 7 context variables (9 to 2) and 2 context-coded bins (4 to 2) for MTS index coding with average BD-rate differences of 0.01%, -0.01% and 0.02% for AI, RA and LB; Proposal#2 reduces 6 context variables (9 to 3) and 2 context-coded bin (4 to 2) with average BD-rate differences of 0.00%, -0.01% and 0.01% for AI, RA and LB.

Introduction

In VVC WD [1], MTS index (tu_mts_idx) is binarized with truncated unary coding, 1st bin of MTS index is context-coded with five context variables based on cqtDepth, and the reminder of bins (binIdx=1..4) are context-coded with single context variable per binIdx as shown in Table 1-1. As can be seen, current MTS index coding uses in total 9 context variables and the number of context-coded bins is 4 in worst case.

Table 1-1: Context assignments for MTS index

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| tu_mts_idx[ ][ ] | cqtDepth | 6 | 7 | 8 | na | na |

Since MTS is available for 4x4 to 32x32 transform block sizes, MTS index never appears for 128x128/64x64 CU (cqtDepth=0..1) when quad tree split is only used up to 5 depth, and associated context variables are redundant.

It is also desirable to reduce the number of context-coded bins with coding performance kept.

Proposal

This contribution proposes two simplified context modeling for MTS index coding as shown in Table 2-1.

For proposal#1, MTS index (tu_mts_idx) is binarized with Truncated Unary coding (TU), bins with binIdx=0..1 of MTS index are context-coded with single context variable per binIdx and the reminder of bins (binIdx=2..3) are bypassed. Thus, proposal#1 reduces 7 context variables (9 to 2) and 2 context-bin (4 to 2).

For proposal#2, 1st bin (binIdx=0) is context-coded with two context variables based on transform block size (max(tbWidth, tbHeight)>4), and the 2nd bin (binIdx=1) is context-coded with single context variable, and the reminder of bins is bypassed. Thus, proposal#2 reduces 6 context variables (9 to 3) and 2 context-coded bin (4 to 2).

Table 2-1: Summary of proposed context assignments for MTS index (tu_mts_index)

|  | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >=5 |
| VTM-5.0 | cqtDepth | 6 | 7 | 8 | na | na |
| Proposal#1 | 0 | 1 | bypass | bypass | na | na |
| Proposal#2 | Max(tbWidth, tbHeight) > 4 ? 1 : 0 | 2 | bypass | bypass | na | na |

Simulation Results

The proposed chroma TS has been implemented on top of VTM-5.0 and following simulations were conducted under common test condition [2]:

- Test1:	VTM-5.0 vs Proposal#1 (2 context variables)

- Test2: VTM-5.0 vs Proposal#2 (3 context variables)

Table 3-1 to 3-2 shows the results of each Test1 and Test2, respectively.

Table 3-1: Results of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.01% | -0.04% | -0.04% | 100% | 100% |
| Class B | 0.02% | -0.07% | 0.03% | 100% | 100% |
| Class C | 0.01% | 0.05% | 0.01% | 100% | 100% |
| Class E | 0.01% | -0.02% | -0.02% | 100% | 100% |
| Overall | 0.01% | -0.02% | 0.00% | 100% | 100% |
| Class D | 0.02% | 0.11% | -0.07% | 100% | 100% |
| Class F | 0.01% | -0.05% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.06% | 0.12% | 100% | 100% |
| Class A2 | 0.03% | -0.07% | 0.06% | 100% | 100% |
| Class B | 0.02% | -0.08% | 0.01% | 100% | 100% |
| Class C | -0.06% | -0.18% | -0.20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.10% | -0.02% | 100% | 100% |
| Class D | 0.02% | -0.03% | 0.31% | 100% | 100% |
| Class F | -0.02% | -0.04% | 0.17% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.01% | -0.01% | 100% | 100% |
| Class C | -0.01% | 0.25% | -0.12% | 100% | 100% |
| Class E | 0.07% | 0.15% | 1.34% | 100% | 100% |
| Overall | 0.02% | 0.12% | 0.29% | 100% | 100% |
| Class D | 0.03% | -0.25% | 0.71% | 100% | 100% |
| Class F | 0.00% | -0.18% | -0.52% | 100% | 100% |

Table 3-1: Results of Test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.02% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | -0.05% | 100% | 101% |
| Class B | 0.00% | -0.05% | 0.03% | 99% | 99% |
| Class C | 0.00% | -0.07% | 0.05% | 99% | 98% |
| Class E | 0.00% | -0.07% | -0.02% | 100% | 101% |
| Overall | 0.00% | -0.04% | 0.01% | 99% | 99% |
| Class D | 0.00% | 0.14% | -0.02% | 100% | 100% |
| Class F | -0.02% | -0.08% | -0.02% | 100% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.37% | 0.07% | 100% | 100% |
| Class A2 | 0.01% | -0.05% | -0.03% | 100% | 100% |
| Class B | -0.01% | -0.02% | -0.16% | 100% | 100% |
| Class C | -0.01% | -0.18% | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.14% | -0.08% | 100% | 100% |
| Class D | 0.01% | -0.15% | 0.15% | 100% | 100% |
| Class F | -0.03% | -0.17% | 0.04% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.45% | -0.28% | 101% | 101% |
| Class C | 0.02% | 0.06% | -0.01% | 100% | 100% |
| Class E | 0.02% | -0.43% | -0.04% | 100% | 100% |
| Overall | 0.01% | 0.10% | -0.13% | 100% | 100% |
| Class D | 0.00% | -0.17% | 0.63% | 100% | 100% |
| Class F | 0.06% | -0.04% | -0.27% | 101% | 101% |

Conclusion

It is recommended to adopt one of proposed context modeling for MTS index coding into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “	Versatile Video Coding (Draft 5), ” JVET-N1001, Apr. 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).