| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0355 |
| --- | --- |

| Title: | Non-CE3 : MRL restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, jangwon84.choi, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to restrict MRL intra prediction at the boundaries of picture, slice, tile, or brick. The proposed restriction provides 0.00% and 0.00% BD-rate changes in AI and RA configurations, respectively.

Proposed method

In VTM-5.0, multiple reference line (MRL) [1] intra prediction is used to utilize more reference lines for intra prediction. MRL is disabled for the upper boundary of each CTU to prevent using extended reference samples outside the current CTU. If neighbouring reference samples are not available for intra prediction, all sample values of reference sample are set equal to 1 << (bitDepth – 1). This means that if the current block is at the boundaries of the picture, slice, tile, or brick, all reference samples have the same values. In this case, multiple reference line is not needed anymore, because always the same predictor is generated from the reference samples which have the same values. Therefore, this contribution propose to restrict MRL intra prediction at the boundaries (i.e., left and above boundaries) of the picture, slice, tile, or brick.

Experimental Result

The proposed method has been implemented using VTM-5.0 and under the common test conditions [2].

Table 1. Experimental results for AI

Table 2. Experimental results for RA

Conclusion

In this contribution, it is proposed to restrict MRL intra prediction at the boundaries of the picture, slice, tile, or brick. The experimental results reportedly show 0.00% (AI) and 0.00% (RA) BD-rate changes. It is recommended to adopt the proposed method in the next VTM.

Reference

- B. Bross, et al., “CE3: Multiple reference line intra prediction,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0283, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).