| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0356-v2 |
| --- | --- |

| Title: | Non-CE3 : Restriction on WAIP for MRL and ISP intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Ling Li Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, jangwon84.choi, l.li, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to restrict wide-angle intra prediction (WAIP) for multiple reference line (MRL) and intra sub-partitions (ISP) intra prediction. The experimental results on three sub-tests are reported as follows:

- Test 1: disable WAIP for MRL : 0.00% (AI), 0.01% (RA)

- Test 2: disable WAIP for ISP: 0.01% (AI), 0.00% (RA)

- Test 3: disable WAIP for both MRL and ISP: 0.01% (AI), 0.02% (RA)

Introduction

WAIP [1] was proposed to allow extended intra prediction angles beyond 45 degree top-right and bottom-left directions to improve the accuracy of prediction blocks using closer reference samples for non-square blocks. WAIP replaces several conventional angular intra prediction modes with wide angular modes according to aspect ratio of non-square block. Therefore, if the current block is a non-square block, WAIP is always performed. However, WAIP is not working as it is supposed to when it works on MRL [2] and ISP [3] mode. In Figure 1, the solid and dotted lines represent the normal intra prediction mode and WAIP mode in MRL intra prediction, respectively. In order to predict the prediction value of current sample C in MRL intra prediction with reference line index 3, the reference sample A3 is used considering WAIP mode. However, the distance between A3 and C is longer than the distance between L3 and C. In Figure 2, the solid and dotted lines show the normal intra prediction mode and WAIP mode in ISP intra prediction, respectively. In order to predict sub-block 2, above-right reference samples (A1 to A8) have to be used. But, in this case, these reference samples are not available so A1 to A8 are padded and set equal to the value of reference sample A0. Applying WAIP mode in ISP intra prediction to predict sub-blocks can decrease the prediction accuracy because more padded values are used for prediction.

Figure 1. Problem with WAIP in MRL intra prediction

Figure 2. Problem with WAIP in ISP intra prediction

Proposed

As shown in Figure 1 and 2, WAIP requires using farther located samples for MRL and ISP, which is opposing to the original intention of WAIP. There is no coding performance benefit for allowing WAIP for MRL and ISP mode. Therefore, WAIP is to be disabled for MRL or ISP intra prediction modes.

Experimental result

The proposed method has been implemented using VTM-5.0 and under the common test conditions [4]. Three sub-tests are evaluated as follows.

- Test 1: disable WAIP for MRL

Table 1. Coding performance of the proposed method (AI)

Table 2. Coding performance of the proposed method (RA)

- Test 2: disable WAIP for ISP

Table 3. Coding performance of the proposed method (AI)

Table 4. Coding performance of the proposed method (RA)

- Test 3: disable WAIP for both MRL and ISP

Table 5. Coding performance of the proposed method (AI)

Table 6. Coding performance of the proposed method (RA)

Conclusion

It is proposed to restrict WAIP for MRL and ISP intra prediction. It is reported that Test 1, Test 2, and Test 3 provide 0.00%, 0.01%, and 0.01% BD-rate changes, respectively for AI configuration. It is recommended to adopt the proposed method in the next VTM.

Reference

- Fabien Racapé, et al., “CE3-related: Wide-angle intra prediction for non-square blocks”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-K0500, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- B. Bross, et al., “CE3: Multiple reference line intra prediction,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-L0283, 12th Meeting, Macao, CN, 3–12 Oct. 2018.

- S. De-Luxán-Hernández, “CE3: Intra Sub-Partitions coding Mode”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11JVET-M0102, 13th Meeting, Marrakech, Ma, 9–18 Jan. 2019.

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).