| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0364-v1 |
| --- | --- |

| Title: | On general intra sample prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd |  |  |

_____________________________

# Abstract

Modifications to the general intra sample prediction process are proposed: reference sample generation when ISP is used, reference sample padding and projection rules, reference sample filtering for angular modes, PDPC process for DC mode, PDPC process for angular modes, and interpolation filter selection rules when ISP is used. It is asserted that these modifications are beneficial for implementation purposes and remove quirks in the specification. The cumulative impact of the proposed modifications on coding efficiency is a small net gain: -0.05% for AI conditions and -0.02% for RA conditions.

# Proposal

Several modifications are proposed:

- Align ISP reference sample fetching with other modes

- Padding and projection rules for angular prediction

- Never filter reference samples before angular prediction

- Make PDPC process the same for DC and planar modes

- Single process for PDPC for all angular modes (except HOR/VER), where a sample-based range check is further avoided

- Simpler rules for interpolation filter determination

Each modification is further detailed in the sections below.

## Reference samples fetching in ISP mode

For consistency purposes, it is desirable for reference samples fetched from previously coded CUs for the purpose of intra prediction to be the same regardless of whether ISP is used for a given CU. This is currently not the case in the VVC draft, where a CU coded in ISP mode may retrieve samples outside the “normal” range. It is proposed to reduce the amount of reference samples retrieved for a given partition within a CU in ISP mode by an amount equal the offset of the partition within the CU. The figure below shows a current CU (blue), reference samples from above (orange), padded reference samples from above (yellow) for four different cases:

- a 4Nx4N CU,

- a Nx4N CU,

- a 4Nx4N CU in ISP mode, as in the current VVC draft, and

- a 4Nx4N CU is ISP mode, as proposed.

Another benefit is that for the “max angle” depicted in the figure above, the predictions obtained with and without ISP are identical (save for PDPC and possibly smoothing).

Specification text modifications are provided below.

In section 8.4.5.1

- For xPartIdx = 0..numPartsX − 1 and yPartIdx = 0..numPartsY − 1, the following applies:

- The intra sample prediction process as specified in clause 8.4.5.2 is invoked with the location ( xTbCmp, yTbCmp ) set equal to ( xTb0 + nW * xPartIdx, yTb0 + nH * yPartIdx ), the location ( xTb0, yTb0 ), the intra prediction mode predModeIntra, the transform block width nTbW and height nTbH set equal to nW and nH, the coding block width nCbW and height nCbH set equal to nTbW and nTbH, and the variable cIdx as inputs, and the output is an (nTbW)x(nTbH) array predSamples.

In section 8.4.5.2

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a sample location ( xTb0, yTb0 ) specifying the top-left sample of the first transform block in the current coding unit relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The predicted samples predSamples[ x ][ y ] are derived as follows:

- If intra_mip_flag[ xTbComp ][ yTbComp ] is equal to 1 and cIdx is equal to 0, the matrix-based intra sample prediction process as specified in clause 8.4.5.2.1 is invoked with the location ( xTbCmp, yTbCmp ), the location ( xTb0, yTb0 ), the intra prediction mode predModeIntra, the transform block width nTbW and height nTbH as inputs, and the output is predSamples.

- Otherwise, the general intra sample prediction process as specified in clause 8.4.5.2.5 is invoked with the location ( xTbCmp, yTbCmp ), the intra prediction mode predModeIntra, the transform block width nTbW and height nTbH, the coding block width nCbW and height nCbH, and the variable cIdx as inputs, and the output is predSamples.

In section 8.4.5.2.5

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a sample location ( xTb0, yTb0 ) specifying the top-left sample of the first transform block in the current coding unit relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables refW and refH are derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0, the following applies:

refW = nTbW * 2		(8-92)

refH = nTbH * 2		(8-93)

- Otherwise ( IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 ), the following applies:

refW = nCbW * 2 + xTb0 − xTbCmp 				(8-94)

refH = nCbH * 2 + yTb0 − yTbCmp				(8-95)

## Padding and projection rules for angular prediction

Padding of the reference sample array is defined as follows in the current VVC draft (assuming a “vertical” prediction using the “above” samples):

- If the angle is positive, the first sample is replicated to the left and the last sample is replicated to the right

- If the angle is negative, the sample aligned with the rightmost column of the current PU is replicated to the right, and samples from the left reference array are projected to extend the reference array to the left

Padding to the left when the angle is positive appears unnecessary as the padded sample is never referenced.

Padding to the right when the angle is negative is undesirable as a valid sample is generally available. In an encoder, if using the same reference array for testing various angles, this sample is needlessly changed from mode to mode. It is therefore suggested to remove this padding.

The projection process also has some shortcomings, as further described below, where limits are analyzed. The figure below illustrates the various proposed modifications.

The table below analyzes bounds in the sample projection process. The value x is set to ( nTbH * intraPredAngle ) >> 5 and represents the smallest value it can take according to equation (8-120) in the draft VVC specification. The value y represents the corresponding index used to address the p array in that same equation.

Values marked in red are values that exceed nTbH. Such values are undesirable. For example, given nTbH = 8, it is not possible to load the 8 contiguous samples from p[][0..7] and shuffle them according to the value of intraPredAngle to obtain to projected values of ref[]. Even introducing an offset and loading p[][o+0..o+7] would not enable such shuffling.

| intraPredAngle | invAngle | nTbH = 4 |  | nTbH = 8 |  | nTbH = 16 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | x | y | x | y | x | y |
| -1 | -8192 | -1 |  | -1 |  | -1 |  |
| -2 | -4096 | -1 |  | -1 |  | -1 |  |
| -3 | -2731 | -1 |  | -1 |  | -2 | 20 |
| -4 | -2048 | -1 |  | -1 |  | -2 | 15 |
| -6 | -1365 | -1 |  | -2 | 10 | -3 | 15 |
| -8 | -1024 | -1 |  | -2 | 7 | -4 | 15 |
| -10 | -819 | -2 | 5 | -3 | 9 | -5 | 15 |
| -12 | -683 | -2 | 4 | -3 | 7 | -6 | 15 |
| -14 | -585 | -2 | 4 | -4 | 8 | -7 | 15 |
| -16 | -512 | -2 | 3 | -4 | 7 | -8 | 15 |
| -18 | -455 | -3 | 4 | -5 | 8 | -9 | 15 |
| -20 | -410 | -3 | 4 | -5 | 7 | -10 | 15 |
| -23 | -356 | -3 | 3 | -6 | 7 | -12 | 16 |
| -26 | -315 | -4 | 4 | -7 | 8 | -13 | 15 |
| -29 | -282 | -4 | 3 | -8 | 8 | -15 | 16 |
| -32 | -256 | -4 | 3 | -8 | 7 | -16 | 15 |

It is proposed to modify the projection process such as to not reference samples that are far out. The index into the reference array is thus clipped to not exceed nTbH − 1.

Additionally it is desirable to have the following property: 32 + ((x * invAngle + 128) >> 8) equal to (((x + intraPredAngle) * invAngle + 128) >> 8) for x in 0..intraPredAngle − 1 such that when projecting an edge of size 64, the process can be separated into 2 identical projections of size 32. In the current VVC draft this property is not satisfied when the slope is equal to 29. Adding 1 bit of precision to the inverse angle derivation fixes this.

Below is an example C code using SIMD that implements the proposed projection process, where the number of operations per group of 8 samples are 2 vector loads, 1 vector shuffle, and 1 vector store:

const int sizeDiv8 = (1 << log2Size) >> 3;

int64_t offset = offsets[angleIdx];

for (int i = 0; i < sizeDiv8; i++)

{

const __m128i x = _mm_loadu_si128((const __m128i *)&side[1 + 8 * i]);

const __m128i y = _mm_loadu_si128((const __m128i *)shuffle[angleIdx][i & 3]);

_mm_storeu_si128((__m128i *)&main[offset >> 56], _mm_shuffle_epi8(x, y));

offset <<= 8;

}

Specification text modifications are provided below.

In section 8.4.5.2.11

The inverse angle parameter invAngle is derived based on intraPredAngle as follows:

invAngle = Round256512*32intraPredAngle		(8-118)

In section 8.4.5.2.11

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = 0..nTbW + refIdx + 1	(8-119)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbH * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbH ) ],
	with x = −nTbH..−1	(8-120)

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + ( ( x * invAngle + 128 )  >>  8 ) ],
	with x = −1..( nTbH * intraPredAngle )  >>  5	(8-120)

ref[ ( ( nTbH * intraPredAngle )  >>  5 ) − 1 ] = ref[ ( nTbH * intraPredAngle )  >>  5 ]	(8-121)

ref[ nTbW + 1 + refIdx ] = ref[ nTbW + refIdx ]	(8-122)

- Otherwise,

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = nTbW + 12 + refIdx..refW + refIdx	(8-123)

ref[ −1 ] = ref[ 0 ]		(8-124)

- The additional samples ref[ refW + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 12) are derived as follows:

ref[ refW + refIdx +x ] = p[ −1 + refW ][ −1 − refIdx ]	(8-125)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( y + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	 (8-126)

iFact = ( ( y + 1 + refIdx ) * intraPredAngle ) & 31	(8-127)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(8-128)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(8-129)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(8-130)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(8-131)

- Otherwise (predModeIntra is less than 34), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = 0..nTbH + refIdx + 1	(8-132)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

- When ( nTbW * intraPredAngle )  >>  5 is less than −1,

ref[ x ] = p[ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbW ) ][ −1 − refIdx ],
	with x = −nTbW..−1	(8-133)

ref[ x ] = p[ −1 − refIdx + ( ( x * invAngle + 128 )  >>  8 ) ][ −1 − refIdx ],
	with x = −1..( nTbW * intraPredAngle )  >>  5	(8-133)

ref[ ( ( nTbW * intraPredAngle )  >>  5 ) − 1 ] = ref[ ( nTbW * intraPredAngle )  >>  5 ]	(8-134)

ref[ nTbG + 1 + refIdx ] = ref[ nTbH + refIdx ]	(8-135)

- Otherwise,

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = nTbH + 12 + refIdx..refH + refIdx	(8-136)

ref[ −1 ] = ref[ 0 ]		(8-137)

- The additional samples ref[ refH + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 12) are derived as follows:

ref[ refH + refIdx +x ] = p[ −1 + refH ][ −1 − refIdx ]	(8-138)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( x + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	(8-139)

iFact = ( ( x + 1 + refIdx ) * intraPredAngle ) & 31	(8-140)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(8-141)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1Y( ( ( i=03fT[ i ]*ref[ y+iIdx+i ] ) + 32 )  >>  6 )	(8-142)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(8-143)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(8-144)

## No reference sample filter before angular prediction

This was previously proposed in JVET-M0095 (second item). Having special rules for angles that have integer slopes has led to a lot of confusion and needlessly complicates the VVC specification. It is thus proposed to do away with such rules.

A side benefit of the proposed modification is that reference sample smoothing does not need to be applied to as many samples.

Specification text modifications are provided below.

In section 8.4.5.2.5

The variable refFilterFlag is derived as follows:

- If predModeIntra is equal to one of the following values: 0, −14, −12, −10, −6, 2, 34, 66, 72, 76, 78, 80, then refFilterFlag is set equal to 1.

- Otherwise, refFilterFlag is set equal to 0.

For the generation of the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, the following ordered steps apply:

- The reference sample availability marking process as specified in clause 8.4.5.2.6 is invoked with the sample location ( xTbCmp, yTbCmp ), the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the colour component index cIdx as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = − refIdx..refW − 1, y = −1 − refIdx as output.

- When at least one sample refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.4.5.2.7 is invoked with the intra prediction reference line index refIdx, the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

- The reference sample filtering process as specified in clause 8.4.5.2.8 is invoked with the intra prediction reference line index refIdx, the transform block width nTbW and height nTbH, the reference sample width refW, the reference sample height refH, the reference filter flag refFilterFlag, the unfiltered samples refUnfilt[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx, and the colour component index cIdx as inputs, and the reference samples p[ x ][ y ] with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx as output.

The intra sample prediction process according to predModeIntra applies as follows:

- If predModeIntra is equal to INTRA_PLANAR, the corresponding intra prediction mode process specified in clause 8.4.5.2.9 is invoked with the transform block width nTbW, and the transform block height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_DC, the corresponding intra prediction mode process specified in clause 8.4.5.2.10 is invoked with the transform block width nTbW, the transform block height nTbH, the intra prediction reference line index refIdx, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, if predModeIntra is equal to INTRA_LT_CCLM, INTRA_L_CCLM or INTRA_T_CCLM, the corresponding intra prediction mode process specified in clause 8.4.5.2.12 is invoked with the intra prediction mode predModeIntra, the sample location ( xTbC, yTbC ) set equal to ( xTbCmp, yTbCmp ), the transform block width nTbW and height nTbH, and the reference sample array p as inputs, and the output is the predicted sample array predSamples.

- Otherwise, the corresponding intra prediction mode process specified in clause 8.4.5.2.11 is invoked with the intra prediction mode predModeIntra, the intra prediction reference line index refIdx, the transform block width nTbW, the transform block height nTbH, the reference sample width refW, the reference sample height refH, the coding block width nCbW and height nCbH, the reference filter flag refFilterFlag, the colour component index cIdx, and the reference sample array p as inputs, and the predicted sample array predSamples as outputs.

In section 8.4.5.2.8

Inputs to this process are:

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable refFilterFlag specifying the value of reference filter flag,

- the (unfiltered) neighbouring samples refUnfilt[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the reference samples p[ x ][ y ], with x = −1 − refIdx, y = −1 − refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1 − refIdx.

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- predModeIntra is equal to INTRA_PLANAR

- refFilterFlag is equal to 1

- Otherwise, filterFlag is set equal to 0.

For the derivation of the reference samples p[ x ][ y ] the following applies:

- If filterFlag is equal to 1, the filtered sample values p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 are derived as follows:

p[ −1 ][ −1 ] = ( refUnfilt[ −1 ][ 0 ] + 2 * refUnfilt[ −1 ][ −1 ] + refUnfilt[ 0 ][ −1 ] + 2 )  >>  2	(8-104)

p[ −1 ][ y ] = ( refUnfilt[ −1 ][ y + 1 ] + 2 * refUnfilt[ −1 ][ y ] + refUnfilt[ −1 ][ y − 1 ] + 2 )  >>  2
	for y = 0..nTbHrefH − 2		(8-105)

p[ −1 ][ refH − 1y ] = refUnfilt[ −1 ][ refH − 1y ] for y = nTbH + 1..refH − 1	(8-106)

p[ x ][ −1 ] = ( refUnfilt[ x − 1 ][ −1 ] + 2 * refUnfilt[ x ][ −1 ] + refUnfilt[ x + 1 ][ −1 ] + 2 )  >>  2
	for x = 0..nTbWrefW − 2 		(8-107)

p[ refW − 1x ][ −1 ] = refUnfilt[ refW − 1x ][ −1 ] for y = nTbW + 1..refH − 1	(8-108)

- Otherwise, the reference samples values p[ x ][ y ] are set equal to the unfiltered sample values refUnfilt[ x ][ y ] with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

In section 8.4.5.2.11

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable refFilterFlag specifying the value of reference filter flag,

- a variable cIdx specifying the colour component of the current block,

- the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0.

- predModeIntra is equal to INTRA_ANGULAR2, INTRA_ANGULAR34 or INTRA_ANGULAR66

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nTbW is less than or equal to 8

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is less than INTRA_ANGULAR34 and nTbH is less than or equal to 8.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-9.

- The variable filterFlag is derived as follows:

- If one mor more of the following conditions is true, filterFlag is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- minDistVerHor is greater than intraHorVerDistThres[ nTbS ] and refFilterFlag is equal to 0

- Otherwise, filterFlag is set equal to 0.

## Same PDPC process for DC and planar modes

The PDPC process is identical for DC and planar modes, except that for the DC mode, the top-left sample is used in addition to top and left samples. It is proposed to remove use of top-left sample for the DC mode such as to align the PDPC process for DC and planar modes. This also avoids having to do any clipping in the PDPC process for DC.

Specification text modifications are provided below.

In section 8.4.5.2.13

The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

- If predModeIntra is equal to INTRA_PLANAR or INTRA_DC, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ] 		(8-212)

refT[ x ][ y ] = p[ x ][ −1 ] 		(8-213)

wT[ y ] = 32  >>  ( ( y  <<  1 )  >>  nScale )		(8-214)

wL[ x ] = 32  >>  ( ( x  <<  1 )  >>  nScale )		(8-215)

wTL[ x ][ y ] = ( predModeIntra  = =  INTRA_DC )  ?  ( ( wL[ x ]  >>  4 ) + ( wT[ y ]  >>  4 ) )  :  0	(8-216)

## Single PDPC process for angular modes

Currently there are two modes of PDPC operation for angular modes (other than HOR/VER). Indeed, special rules apply to modes 2 and 66, which is undesirable. It is proposed to have a single mode of operation.

Also, there is a need to do a per-sample check to determine whether the referenced sample is within range. If it is not within range, the PDPC weight is set to 0 for the current sample. This aspect is illustrated in the figure below.

It is proposed to modify the derivation of the PDPC scale such that referenced samples are always within range.

Specification text modifications are provided below. Note that rounding for determining the sample position to fetch is also cleaned up (currently the rounding offset is the sum of two rounding offsets).

In section 8.4.5.2.5

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.13 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ xTbCmp ] is equal to 0

- predModeIntra is less than or equal to INTRA_ANGULAR18 or greater than or equal to INTRA_ANGULAR50

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is equal to INTRA_ANGULAR18

- predModeIntra is equal to INTRA_ANGULAR50

- predModeIntra is less than or equal to INTRA_ANGULAR10

- predModeIntra is greater than or equal to INTRA_ANGULAR58

In section 8.4.5.2.13

The variable nScale is set to ( ( Log2( nTbW ) + Log2( nTbH ) − 2 )  >>  2 ).

If predModeIntra is greater than INTRA_ANGULAR50, the variable nScale is further set to Min( 2, Log2( nTbH ) − Floor( Log2( 3 * invAngle – 2 ) ) + 7 ), using invAngle as specified in clause 8.4.5.2.11.

If predModeIntra is less than INTRA_ANGULAR18 and not equal to either INTRA_PLANAR or INTRA_DC, the variable nScale is further set to Min( 2, Log2( nTbW ) − Floor( Log2( 3 * invAngle – 2 ) ) + 7 ), using invAngle as specified in clause 8.4.5.2.11.

The reference sample arrays mainRef[ x ] and sideRef[ y ], with x = 0..refW − 1 and y = 0..refH − 1 are derived as follows:

mainRef[ x ] = p[ x ][ −1 ]		(8-211)
sideRef[ y ] = p[ −1 ][ y ]

The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] with x = 0..nTbW − 1, y =0..nTbH − 1 are derived as follows:

- If predModeIntra is equal to INTRA_PLANAR or INTRA_DC, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ] 		(8-212)

refT[ x ][ y ] = p[ x ][ −1 ] 		(8-213)

wT[ y ] = 32  >>  ( ( y  <<  1 )  >>  nScale )		(8-214)

wL[ x ] = 32  >>  ( ( x  <<  1 )  >>  nScale )		(8-215)

wTL[ x ][ y ] = ( predModeIntra  = =  INTRA_DC )  ?  ( ( wL[ x ]  >>  4 ) + ( wT[ y ]  >>  4 ) )  :  0	(8-216)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR18 or INTRA_ANGULAR50, the following applies:

refL[ x ][ y ] = p[ −1 ][ y ]		(8-217)

refT[ x ][ y ] = p[ x ][ −1 ]		(8-218)

wT[ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ? 32  >>  ( ( y  <<  1 )  >>  nScale ) : 0	(8-219)

wL[ x ] = ( predModeIntra  = = INTRA_ANGULAR50 ) ? 32  >>  ( ( x  <<  1 )  >>  nScale ) : 0	(8-220)

wTL[ x ][ y ] = ( predModeIntra  = = INTRA_ANGULAR18 ) ?  wT[ y ] : wL[ x ] 	(8-221)

- Otherwise, if predModeIntra is equal to INTRA_ANGULAR2 or INTRA_ANGULAR66, the following applies:

refL[ x ][ y ] = p[ −1 ][ x + y + 1 ] 		(8-222)

refT[ x ][ y ] = p[ x + y + 1 ][ −1 ] 		(8-223)

wT[ y ] = ( 32  >>  1 )  >>  ( ( y  <<  1 )  >>  nScale )	(8-224)

wL[ x ] = ( 32  >>  1 )  >>  ( ( x  <<  1 )  >>  nScale )	(8-225)

wTL[ x ][ y ] = 0		(8-226)

- Otherwise, if predModeIntra is less than or equal to INTRA_ANGULAR108 and nScale is equal to or greater than 0, the following ordered steps apply:

- The variables dXPos[ y ], dXFrac[ y ], dXInt[ y ] and dX[ x ][ y ] are derived as follows using invAngle as specified in clause 8.4.5.2.11 depending on intraPredMode:

dXPosdXInt[ y ] = ( ( y + 1 ) * invAngle + 2128 )  >>  28
dXFrac[ y ] =	dXPos[ y ] & 63		(8-227)
dXInt[ y ] =	dXPos [ y ]  >>  6
dX[ x ][ y ] =	x + dXInt[ y ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = 0		(8-228)

refT[ x ][ y ] = ( dX[ x ][ y ] < refW − 1 )  ?  mainRef[ dX[ x ][ y ] + ( dXFrac[ y ]  >>  5 ) ]  :  0	(8-229)

refT[ x ][ y ] = ( y < ( 3 << nScale ) )  ?  mainRef[ dX[ x ][ y ] ]  :  0	(8-229)

wT[ y ] = ( dX[ x ][ y ] < refW − 1 )  ?  32  >>  ( ( y  <<  1 )  >>  nScale )  :  0	(8-230)

wL[ x ] = 0		(8-231)

wTL[ x ][ y ] = 0		(8-232)

- Otherwise, if predModeIntra is greater than or equal to INTRA_ANGULAR580 and nScale is equal to or greater than 0, the following ordered steps apply:

- The variables dYPos[ x ], dYFrac[ x ], dYInt[ x ] and dY[ x ][ y ] are derived as follows using invAngle as specified in clause 8.4.5.2.11 depending on intraPredMode:

dYPosdyInt[ x ] = ( ( x + 1 ) * invAngle + 2128 )  >>  28
dYFrac[ x ] =	dYPos[ x ] & 63		(8-233)
dYInt[ x ] =	dYPos[ x ]  >>  6
dY[ x ][ y ] =	y + dYInt[ x ]

- The variables refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] are derived as follows:

refL[ x ][ y ] = ( dY[ x ][ y ] < refH − 1 )  ?  sideRef[ dY[ x ][ y ] + ( dYFrac[ x ]  >>  5 ) ]  :  0	(8-234)

refL[ x ][ y ] = ( x < ( 3 << nScale ) )  ?  sideRef[ dY[ x ][ y ] ]  :  0	(8-234)

refT[ x ][ y ] = 0		(8-235)

wT[ y ] = 0		(8-236)

wL[ x ] = ( dY[ x ][ y ] < refH − 1 )  ?  32  >>  ( ( x  <<  1 )  >>  nScale )  :  0	(8-237)

wTL[ x ][ y ] = 0		(8-238)

- Otherwise, refL[ x ][ y ], refT[ x ][ y ], wT[ y ], wL[ x ] and wTL[ x ][ y ] are all set equal to 0.

## Smoothing rules

Different rules apply to determine the variable filterFlag when ISP is used, which is undesirable. The variable filterFlag determines whether the prediction is smoothed. A unified derivation that is independent of ISP use is proposed. In essence the “regular” derivation rules are also applied when ISP is used.

Note: the current specification may be incorrect in that it suggests that filterFlag can be true for HOR/VER modes when ISP is used. The VTM software never filters in such case.

Specification text modifications are provided below.

In section 8.4.5.2.11

The variable filterFlag is derived as follows:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-9.

- If refIdx is equal to 0 and minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

- If one or more of the following conditions is true, filterFlag is set equal to 0.

- predModeIntra is equal to INTRA_ANGULAR2, INTRA_ANGULAR34 or INTRA_ANGULAR66

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nTbW is less than or equal to 8

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is less than INTRA_ANGULAR34 and nTbH is less than or equal to 8.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-9.

- The variable filterFlag is derived as follows:

- If one mor more of the following conditions is true, filterFlag is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- minDistVerHor is greater than intraHorVerDistThres[ nTbS ] and refFilterFlag is equal to 0

- Otherwise, filterFlag is set equal to 0.

# Results

Results are presented for CTC on top of VTM version 5.0. Given that this proposal relates to intra coding, only AI and RA conditions are reported. Cumulative results are provided in the sections below.

## Reference samples fetching in ISP mode

The number of cases affected by this is very small, so small that no change in coding efficiency is observed.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 101% |

## Padding and projection rules for angular prediction

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.00% | 0.00% | 102% | 101% |
| Class A2 | 0.00% | -0.05% | -0.07% | -0.01% | 96% | 100% |
| Class B | -0.01% | -0.06% | 0.05% | -0.02% | 100% | 102% |
| Class C | -0.04% | 0.00% | 0.06% | -0.02% | 102% | 100% |
| Class E | -0.03% | -0.05% | -0.01% | -0.03% | 96% | 99% |
| Overall | -0.02% | -0.03% | 0.01% | -0.02% | 100% | 101% |
| Class D | -0.01% | 0.03% | -0.14% | 0.00% | 96% | 99% |
| Class F | -0.01% | 0.00% | 0.11% | 0.01% | 96% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | -0.18% | 0.06% | -0.03% | 100% | 102% |
| Class A2 | 0.03% | -0.13% | -0.07% | 0.01% | 100% | 101% |
| Class B | 0.01% | 0.09% | -0.07% | 0.00% | 101% | 101% |
| Class C | -0.03% | -0.05% | 0.06% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | -0.05% | -0.01% | -0.01% | 100% | 101% |
| Class D | 0.03% | -0.14% | 0.05% | 0.03% | 100% | 100% |
| Class F | -0.05% | -0.07% | 0.02% | -0.01% | 100% | 101% |

## No reference sample filter before angular prediction

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.02% | 0.00% | 102% | 101% |
| Class A2 | 0.00% | -0.06% | -0.04% | -0.01% | 98% | 100% |
| Class B | -0.01% | -0.09% | 0.00% | -0.01% | 101% | 101% |
| Class C | -0.04% | -0.02% | 0.03% | -0.03% | 101% | 101% |
| Class E | -0.05% | 0.03% | 0.02% | -0.04% | 98% | 98% |
| Overall | -0.02% | -0.03% | 0.00% | -0.02% | 100% | 100% |
| Class D | -0.02% | 0.01% | -0.23% | -0.02% | 97% | 99% |
| Class F | -0.01% | 0.03% | 0.05% | -0.01% | 100% | 101% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.08% | 0.00% | 101% | 102% |
| Class A2 | 0.01% | 0.07% | -0.08% | 0.01% | 100% | 102% |
| Class B | 0.02% | -0.02% | -0.21% | 0.00% | 100% | 101% |
| Class C | -0.05% | -0.08% | -0.07% | -0.05% | 101% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.09% | -0.01% | 100% | 101% |
| Class D | 0.01% | 0.04% | 0.21% | 0.01% | 102% | 101% |
| Class F | -0.04% | -0.06% | 0.07% | -0.02% | 101% | 101% |

## Same PDPC process for DC and planar modes

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.06% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | -0.04% | -0.06% | 0.00% | 97% | 100% |
| Class B | 0.00% | -0.05% | -0.01% | -0.01% | 98% | 102% |
| Class C | -0.01% | -0.03% | 0.07% | -0.01% | 101% | 98% |
| Class E | -0.01% | -0.03% | -0.09% | -0.01% | 98% | 98% |
| Overall | 0.00% | -0.04% | -0.02% | -0.01% | 99% | 100% |
| Class D | -0.02% | 0.10% | -0.12% | -0.01% | 100% | 99% |
| Class F | -0.01% | 0.04% | 0.09% | 0.00% | 98% | 98% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.01% | -0.28% | 0.11% | -0.02% | 100% | 101% |
| Class A2 | 0.02% | 0.08% | 0.13% | 0.03% | 99% | 101% |
| Class B | 0.01% | 0.00% | -0.19% | 0.00% | 100% | 100% |
| Class C | -0.04% | -0.10% | -0.14% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.01% | -0.07% | -0.05% | -0.01% | 100% | 100% |
| Class D | -0.02% | 0.21% | 0.43% | 0.01% | 101% | 101% |
| Class F | 0.02% | -0.09% | 0.11% | 0.03% | 100% | 100% |

## Single PDPC process for angular modes

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.08% | 0.02% | -0.10% | -0.08% | 99% | 101% |
| Class A2 | -0.05% | -0.03% | -0.08% | -0.05% | 98% | 99% |
| Class B | -0.04% | -0.04% | 0.05% | -0.04% | 100% | 101% |
| Class C | -0.03% | -0.05% | 0.04% | -0.03% | 100% | 100% |
| Class E | -0.09% | -0.15% | -0.18% | -0.10% | 97% | 99% |
| Overall | -0.05% | -0.05% | -0.04% | -0.05% | 99% | 100% |
| Class D | -0.02% | 0.14% | -0.03% | -0.01% | 95% | 98% |
| Class F | -0.02% | 0.10% | 0.02% | 0.01% | 99% | 100% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.03% | -0.03% | -0.07% | -0.04% | 100% | 102% |
| Class A2 | 0.00% | 0.09% | -0.06% | 0.00% | 100% | 101% |
| Class B | 0.00% | -0.08% | -0.12% | -0.02% | 100% | 101% |
| Class C | -0.04% | -0.18% | -0.17% | -0.05% | 99% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | -0.06% | -0.11% | -0.03% | 100% | 101% |
| Class D | 0.02% | 0.07% | 0.00% | 0.03% | 100% | 100% |
| Class F | -0.03% | -0.09% | 0.02% | -0.02% | 100% | 101% |

## Smoothing rules

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.07% | -0.07% | -0.12% | -0.07% | 99% | 97% |
| Class A2 | -0.05% | -0.04% | -0.08% | -0.05% | 97% | 97% |
| Class B | -0.04% | -0.09% | -0.01% | -0.04% | 97% | 100% |
| Class C | -0.02% | -0.13% | 0.07% | -0.02% | 99% | 97% |
| Class E | -0.07% | -0.20% | -0.18% | -0.08% | 96% | 95% |
| Overall | -0.05% | -0.10% | -0.05% | -0.05% | 98% | 97% |
| Class D | 0.00% | 0.14% | 0.02% | 0.01% | 98% | 97% |
| Class F | -0.01% | 0.05% | 0.02% | 0.01% | 97% | 97% |
|  |  |  | Random Access |  |  |  |
|  |  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.05% | -0.29% | 0.15% | -0.05% | 97% | 100% |
| Class A2 | -0.02% | -0.04% | -0.11% | -0.03% | 99% | 101% |
| Class B | 0.00% | -0.02% | -0.20% | -0.02% | 99% | 99% |
| Class C | -0.01% | -0.10% | -0.16% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |  |
| Overall | -0.02% | -0.10% | -0.10% | -0.03% | 99% | 100% |
| Class D | 0.02% | -0.27% | 0.09% | 0.02% | 102% | 101% |
| Class F | -0.05% | -0.10% | -0.03% | -0.02% | 99% | 99% |

# Patent rights declaration(s)

Sharp Laboratories of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).