| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0378-v3 |
| --- | --- |

| Title: | Non-CE4: Simplification of MV Context Storage for Triangle Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

Currently, the MV context storage of triangle merge mode seems to be unnecessarily complicated. In this contribution, three methods are proposed to simplify the MV context storage. The first method selects one of the uni-prediction motion vector based on the split mode for the weighted areas. The second one just selects the bottom MV. The third one selects one of the uni-prediction motion vector based on the split mode or the average MVs for the weighted areas. The forth one keeps current method unchanged if the two reference lists are totally the same. It is reported that the first method brings 0.00% and 0.06% loss for RA and LB configurations respectively compared to VTM-5.0. The second one only brings -0.01% and 0.07% loss for RA and LB respectively compared to VTM-5.0. The third one only brings 0.02% and 0.04% loss for RA and LB respectively compared to VTM-5.0. And the forth one only brings -0.01% and 0.00% loss for RA and LB respectively compared to VTM-5.0.

Introduction

The motion vectors of a CU coded in triangle partition mode are stored in 4x4 units. Depending on the position of each 4x4 unit, either uni-prediction or bi-prediction motion vectors are stored. Denote Mv1 and Mv2 as uni-prediction motion vectors for partition 1 and partition 2 (Figure 1), respectively. If a 4x4 unit is located in the non-weighted area, either Mv1 or Mv2 is stored for that 4x4 unit. Otherwise, if the 4x4 unit is located in the weighted area, a bi-prediction motion vector is stored. The bi-prediction motion vector is derived from Mv1 and Mv2 according to the following process:

1)	If Mv1 and Mv2 are from different reference picture lists (one from L0 and the other from L1), then Mv1 and Mv2 are simply combined to form the bi-prediction motion vector.

2)	Otherwise, if Mv1 and Mv2 are from the same list, and without loss of generality, assume they are both from L0. In this case,

2.a)	 If the reference picture of either Mv2 (or Mv1) appears in L1, then that Mv2 (or Mv1) is converted to a L1 motion vector using that reference picture in L1. Then the two motion vectors are combined to form the bi-prediction motion vector;

2.b)	 Otherwise, instead of bi-prediction motion, only uni-prediction motion Mv1 is stored.

Figure 1 – Triangle partition based inter prediction

Proposed Modification

The following four methods just modify the MV context storage for the case that Mv1 and Mv2 are from the same list. If Mv1 and Mv2 are from different reference picture lists (one from L0 and the other from L1), then Mv1 and Mv2 are simply combined to form the bi-prediction motion vector, which is the same as VTM-5.0.

Method 1

When Mv1 and Mv2 are from the same list, the following strategy is used:

If triangle direction (merge_triangle_split_dir) is equal to 0, uni-prediction motion Mv1 is stored. Otherwise, uni-prediction motion Mv2 is stored.

Method 2

When Mv1 and Mv2 are from the same list, uni-prediction motion Mv2 is stored.

Method 3

When Mv1 and Mv2 are from the same list, the following strategy is used:

If Mv1 and Mv2 are from the same reference picture, the average of Mv1 and Mv2 (uni-prediction) is stored. Otherwise, the strategy in Method 1 is adopted.

Method 4

When Mv1 and Mv2 are from the same list, the following strategy is used:

In the slice level, it is tested that whether the two reference lists are totally the same. If they are totally the same, current method can be reused without mapping process. If they have different reference pictures in the slice level, the strategy in Method 1 is adopted.

Experimental Results

To evaluate the coding performance of the proposed method, the VTM-5.0 is served as the experimental platform. The RA and LB configurations specified in [1], are adopted, and all the sequences are tested across the configurations. BD-rate is used to evaluate the final coding performance.

The results are given as Table 1-4. It is reported that the first method brings 0.00% and 0.06% loss for RA and LB configurations respectively compared to VTM-5.0. The second one only brings -0.01% and 0.07% loss for RA and LB respectively compared to VTM-5.0. The third one only brings 0.02% and 0.04% loss for RA and LB respectively compared to VTM-5.0. And Method 4 only brings -0.01% and 0.00% loss for RA and LB respectively compared to VTM-5.0.

Table 1 Results of Method 1

Table 2 Results of Method 2

Table 3 Results of Method 3

Table 4 Results of Method 4

Conclusion

In this contribution, three methods are proposed to simplify the MV context storage. These modifications bring no coding loss and help to save complexity. Based on the above results, it is suggested to adopt one of them into the VTM.

Reference

- F. Bossen,J. Boyce, X. Li, V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).