| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0386-v2 |
| --- | --- |

| Title: | Non-CE5: A simplification of clipping thresholds for Non-Linear ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-70-4560-2752 ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes new definition of clipping thresholds for Non-Linear ALF. The proposed method changes the definition of the thresholds to keep magnitude relationship of the luma thresholds and the chroma thresholds regardless of internal bit-depth. The proposed method doesn’t have any impact for CTC case, because the thresholds for 10 bits case are not changed from VTM-5.0. It is reported that experimental results when 8 bits case and 12 bits case for both anchor (VTM-5.0) and the proposed method. In the 8 bits case, BD-rates for luma are 0.00%/-0.05%/-0.06%/-0.11% (AI/RA/LDB/LDP) in comparison with VTM-5.0. In the 12 bits case, BD-rates for luma are -0.02%/-0.03%/-0.03%/-0.05% (AI/RA/LDB/LDP) in comparison with VTM-5.0.

Introduction

Non-Linear ALF was adopted to VVC Draft 5 [1] and VTM at the14th meeting at Geneva on March, 2019. Non-Linear ALF is a technology that clips a difference value between center position pixel value and another reference pixel value. The threshold values for the clipping are defined as follows in VVC Draft 5.

Th_Luma (BitDepthY, n) = Round( 2( BitDepthY * ( 4 − n ) / 4 ) )

Th_Chroma (BitDepthC, n) = Round( 2( BitDepthC − 8 ) * 2( 8 * ( 3 − n ) / 3 ) )

Table 1 Actual threshold values of VTM-5.0.

| BitDepth | n = | 0 | n = | 1 | n = | 2 | n = | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| 8 | 256 | 256 | 64 | 40 | 16 | 6 | 4 | 1 |
| 9 | 512 | 512 | 108 | 81 | 23 | 13 | 5 | 2 |
| 10 | 1024 | 1024 | 181 | 161 | 32 | 25 | 6 | 4 |
| 11 | 2048 | 2048 | 304 | 323 | 45 | 51 | 7 | 8 |
| 12 | 4096 | 4096 | 512 | 645 | 64 | 102 | 8 | 16 |

The thresholds for luma sample are designed as to be larger than the thresholds for chroma sample at same internal bit-depth, because the variance of the local differences is often higher for luma than for chroma [2]. However, when internal bit-depth is larger than 10 bits, the thresholds for chroma are larger than the thresholds for luma as shown in Table 1.

In this contribution, it is proposed that a simplification of definition of the thresholds to keep magnitude relationship of the thresholds regardless of internal bit-depth. The remainder of this contribution is divided into two parts;

- Proposed method, and

- Experimental results.

Proposed Method

In this contribution, the new definitions of the thresholds are proposed as follows.

Th_Luma_mod (BitDepthY, n) = BitDepthY >= 10 ?

Th_Luma (10, n) << (BitDepthY - 10) :

(Th_Luma (10, n) + (1 << (10 – BitDepthY – 1))) >> (10 – BitDepthY)

Th_Chroma_mod (BitDepthC, n) = BitDepthC >= 10 ?

Th_Chroma (10, n) << (BitDepthC - 10) :

(Th_Chroma (10, n) + (1 << (10 – BitDepthC – 1))) >> (10 – BitDepthC)

Table 2 Actual threshold values of the proposed method.

| BitDepth | n = | 0 | n = | 1 | n = | 2 | n = | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma | Luma | Chroma | Luma | Chroma |
| 8 | 256 | 256 | 45 | 40 | 8 | 6 | 2 | 1 |
| 9 | 512 | 512 | 91 | 81 | 16 | 13 | 3 | 2 |
| 10 | 1024 | 1024 | 181 | 161 | 32 | 25 | 6 | 4 |
| 11 | 2048 | 2048 | 362 | 322 | 64 | 50 | 12 | 8 |
| 12 | 4096 | 4096 | 724 | 644 | 128 | 100 | 24 | 16 |

As shown in Table 1 and Table 2, it can be found that the thresholds for 10 bits internal bit-depth in Table 1 are simply shifted according to internal bit-depth for luma and chroma. Therefore, the proposed change doesn’t have any impact to simulation results under the common test conditions (CTC) [3], because internal bit-depth of the CTC is 10 bits.

Additionally, as shown in Table 2, magnitude relationship between the luma thresholds and the chroma thresholds are never swapped regardless of bit-depth.

Experimental Results

The proposed method is implemented on top of VTM-5.0. In order to verify the performance of the proposed method, we have run simulations according to the CTC [3] with internal bit-depth setting 8 bits and 12 bits for both the anchor (VTM-5.0) and the proposed method. We haven’t run pure CTC case (internal bit-depth is equal to 10 bits), because actual thresholds of proposed method are exact same as VTM-5.0.

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

Test results

From Table 3 to Table 6 show the experimental results when internal bit-depth is equal to 8 bits. BD-rates for luma are 0.00%/-0.05%/-0.06%/-0.11% (AI/RA/LDB/LDP) in comparison with VTM-5.0.

Table 3 Experimental results for AI condition when internal bit-depth is equal to 8 bits.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 8bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.01% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E | -0.03% | -0.01% | -0.01% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | -0.01% | 0.00% | 0.00% | 100% | 100% |

Table 4 Experimental results for RA condition when internal bit-depth is equal to 8 bits.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 8bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.05% | 0.17% | 100% | 100% |
| Class A2 | -0.09% | 0.07% | 0.00% | 100% | 100% |
| Class B | -0.03% | -0.10% | 0.22% | 100% | 100% |
| Class C | -0.04% | -0.22% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.07% | 0.12% | 100% | 100% |
| Class D | 0.00% | 0.21% | 0.22% | 101% | 98% |
| Class F | -0.07% | 0.04% | -0.03% | 100% | 101% |

Table 5 Experimental results for LDB condition when internal bit-depth is equal to 8 bits.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 8bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.25% | -0.20% | 100% | 100% |
| Class C | -0.08% | -0.27% | 0.03% | 100% | 101% |
| Class E | -0.02% | 0.46% | -0.78% | 100% | 101% |
| Overall | -0.06% | 0.13% | -0.27% | 100% | 101% |
| Class D | 0.02% | -0.34% | 0.61% | 101% | 100% |
| Class F | -0.18% | 0.33% | -0.49% | 100% | 102% |

Table 6 Experimental results for LDP condition when internal bit-depth is equal to 8 bits.

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 8bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | -0.04% | -0.30% | 100% | 100% |
| Class C | -0.08% | 0.05% | 0.17% | 100% | 100% |
| Class E | -0.21% | -0.29% | 0.75% | 100% | 101% |
| Overall | -0.11% | -0.08% | 0.12% | 100% | 100% |
| Class D | 0.08% | -0.18% | -0.28% | 101% | 99% |
| Class F | -0.24% | -0.06% | 0.35% | 101% | 101% |

From Table 7 to Table 10 show the experimental results when internal bit-depth is equal to 12 bits. BD-rates for luma are -0.02%/-0.03%/-0.03%/-0.05% (AI/RA/LDB/LDP) in comparison with VTM-5.0.

Table 7 Experimental results for AI condition when internal bit-depth is equal to 12 bits.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 12bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.01% | 0.00% | 0.00% | 101% | 99% |
| Class B | -0.02% | -0.01% | 0.00% | 100% | 100% |
| Class C | -0.04% | 0.00% | 0.00% | 100% | 100% |
| Class E | -0.04% | -0.01% | -0.01% | 101% | 100% |
| Overall | -0.02% | 0.00% | 0.00% | 101% | 100% |
| Class D | -0.03% | 0.03% | 0.02% | 100% | 99% |
| Class F | -0.07% | -0.01% | 0.00% | 100% | 100% |

Table 8 Experimental results for RA condition when internal bit-depth is equal to 12 bits.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 12bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.09% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | 0.06% | 100% | 100% |
| Class B | -0.03% | -0.06% | -0.11% | 100% | 100% |
| Class C | -0.04% | 0.02% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.03% | -0.02% | 100% | 100% |
| Class D | -0.05% | 0.10% | 0.01% | 100% | 99% |
| Class F | -0.08% | 0.01% | -0.05% | 100% | 100% |

Table 9 Experimental results for LDB condition when internal bit-depth is equal to 12 bits.

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 12bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.31% | 0.23% | 100% | 100% |
| Class C | -0.08% | -0.02% | 0.09% | 100% | 100% |
| Class E | 0.01% | 0.22% | -0.56% | 100% | 98% |
| Overall | -0.03% | 0.18% | -0.01% | 100% | 100% |
| Class D | -0.07% | -0.28% | -0.68% | 99% | 101% |
| Class F | -0.06% | -0.37% | -0.69% | 100% | 99% |

Table 10 Experimental results for LDP condition when internal bit-depth is equal to 12 bits.

|  |  |  | Low delay P Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 12bit |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.02% | -0.14% | 100% | 101% |
| Class C | -0.02% | -0.03% | -0.12% | 100% | 100% |
| Class E | -0.11% | -0.07% | -0.33% | 100% | 100% |
| Overall | -0.05% | -0.03% | -0.18% | 100% | 100% |
| Class D | -0.11% | 0.34% | -0.20% | 99% | 100% |
| Class F | -0.20% | 0.06% | -0.28% | 100% | 101% |

Conclusion

In this contribution, it was proposed that a simplification of clipping thresholds for Non-Linear ALF. The proposed method changed the definition of the thresholds to keep magnitude relationship of the luma thresholds and the chroma thresholds regardless of internal bit-depth. The proposed method didn’t have any impact for CTC case, because the threshold for 10 bits case were not changed from VTM-5.0. It was reported that experimental results when 8 bits case and 12 bits case for both anchor (VTM-5.0) and the proposed method. In the 8 bits case, BD-rates for luma were 0.00%/-0.05%/-0.06%/-0.11%  (AI/RA/LDB/LDP) in comparison with VTM-5.0. In the 12 bits case, BD-rates for luma were -0.02%/-0.03%/-0.03%/-0.05% (AI/RA/LDB/LDP) in comparison with VTM-5.0.

- Acknowledgment

We would like to thank Canon for doing crosschecking of our proposal.

References

- B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, Mar. 2019.

- J. Taquet, P. Onno, C. Gisquet and G. Laroche, “CE5: Results of tests CE5-3.1 to CE5-3.4 on Non-Linear Adaptive Loop Filter.”, JVET-N0242, Geneva, CH, Mar. 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, Mar. 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).