| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0391 |
| --- | --- |

| Title: | AHG17: Avoiding dependencies on externally controlled variables in the decoding process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kashyap Kammachi-Sreedhar Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The proposal is summarized as follows:

- A decoding control NAL unit is proposed with the syntax that includes the following syntax elements, gated by presence flags:

- target_layer_id: the identifier of the layer that is to be decoded

- highest_temporal_id: the identifier of the highest sub-layer that is to be decoded

- handle_as_cvs_start_flag, indicating if the associated CRA or GRA picture is treated as a picture that starts a CLVS

- no_output_of_prior_pics_flag, when equal to 1, specifying that the pictures whose output time precedes the decoding time of the associated IDR picture are not output

- The variables TargetLayerId, HighestTid, and HandleAsCvsStartFlag are derived from the respective syntax elements of the decoding control NAL unit(s) rather than set by external means.

- no_output_of_prior_pics_flag is removed from the slice header syntax. NoOutputOfPriorPicsFlag is derived from no_output_of_prior_pics_flag in the decoding control NAL unit.

# Problem statement

## External means for setting variables not available in all decoders

The decoding process of VVC Draft 5 provides the options to set the values of HandleCraAsCvsStartFlag, TargetLayerId, and HighestTid by external means. The following phrasing is used:

If some external means, not specified in this Specification, is available to set {HandleCraAsCvsStartFlag | TargetLayerId | HighestTid}, {HandleCraAsCvsStartFlag | TargetLayerId | HighestTid} is set by the external means.

However, such phrasing does not mandate all decoder APIs to provides such external means and hence the functionality might not be available in all decoders.

## On seek operation

Setting HandleCraAsCvsStartFlag equal to 1 is useful after a user-originated seek operation after which the decoding continues from a CRA picture. The proponent believes it is desirable to design the VVC standard so that external entities can set HandleCraAsCvsStartFlag in a manner that all decoders support, thus enabling seek functionality.

It is remarked that it might not be a feasible option for a player to insert an end of sequence NAL unit to the bitstream to be decoded to achieve functionality similar to what HandleCraAsCvsStartFlag equal to 1 would provide, because the latest coded picture preceding the the seek operation might already have been passed to decoding before the seek operation was initiated. Consequently, the end of sequence NAL unit would be orphaned, i.e. not included in any access unit, which is not allowed in VVC.

It is also remarked that the option of establishing a new decoder instance or re-initializing the earlier decoder instance might not be preferred as it may involve delay.

Similarly to HandleCraAsCvsStartFlag, the proponent proposes that the VVC specification should provide an external entity the capability of indicating a GRA picture starting a CVS after a seek operation for all decoders.

## On indicating the target layer and highest TemporalId

If no external means are available or used to set the values of TargetLayerId and HighestTid, the decoding process sets TargetLayerId equal to vps_included_layer_id[ 0 ] and HighestTid equal to sps_max_sub_layers_minus1. If this "default" decoding process is not suitable for the entity using the decoder, a sub-bitstream extraction process is needed prior to decoding. The proponent is of the opinion that it is desirable to design the VVC standard so that external entities can set TargetLayerId and HighestTid in a manner that all decoders support.

## On splicing and indicating that no prior pictures are to be output

no_output_of_prior_pics_flag equal to 1 can be used when a CVS starting with an IDR picture is spliced next to another CVS. The splicer needs to rewrite the slice headers of the IDR picture to change the value of no_output_of_prior_pics_flag. The flag appears after ue(v), se(v), and u(v) coded syntax elements in the slice header. Thus, some parsing complexity is involved in changing the value of the flag. The proponent therefore prefers a design where no slice header rewriting is required.

# Summary of the proposal

## Overview

The proposal is summarized as follows:

- A decoding control NAL unit is proposed with the syntax that includes the following syntax elements, gated by presence flags:

- target_layer_id: the identifier of the layer that is to be decoded

- highest_temporal_id: the identifier of the highest sub-layer that is to be decoded

- handle_as_cvs_start_flag, indicating if the associated CRA or GRA picture is treated as a picture that starts a CLVS

- no_output_of_prior_pics_flag, when equal to 1, specifying that the pictures whose output time precedes the decoding time of the associated IDR picture are not output

- The variables TargetLayerId, HighestTid, and HandleAsCvsStartFlag are derived from the respective syntax elements of the decoding control NAL unit(s) rather than set by external means.

- no_output_of_prior_pics_flag is removed from the slice header syntax. NoOutputOfPriorPicsFlag is derived from no_output_of_prior_pics_flag in the decoding control NAL unit.

## Example

The presented example describes how a user-originated seek operation is handled. It is assumed that there is a player that accesses the bitstream and passes it to decoding one access unit at a time. The decoding and playback start originally from the beginning of the bitstream. At some point, the end-user decides to seek to another position that has a CRA picture, and the decoding and playback is continued from the CRA picture. The bitstream that the player gets as input is illustrated in Figure 1.

Figure 1 – An example bitstream. A solid rectangle is a NAL unit.

The performed sequence of operations is illustrated in Figure 2 and described as follows:

- The bitstream is decoded and played starting from the beginning of the bitstream.

- The user seeks to the position of the CRA picture.

- The player skips the NAL units between the current decoding position and the CRA picture (exclusive). In other words, the player does not pass the NAL units between the current decoding position and the CRA picture (exclusive) to the decoder.

- The player inserts a decoding control NAL unit including handle_as_cvs_start_flag equal to 1 before the VCL NAL units of the CRA picture into the access unit that is passed to decoding.

- The bitstream is decoded and played starting from the access unit containing the CRA picture (and now containing also the decoding control NAL unit).

Figure 2 – The bitstream that is passed to decoding when a seek operation is handled by inserting a decoding control NAL unit.

# Proposed specification text changes

## Syntax

| decoding_control_rbsp( ) { | Descriptor |
| --- | --- |
| target_lid_present_flag | u(1) |
| highest_tid_present_flag | u(1) |
| handle_as_cvs_start_present_flag | u(1) |
| no_output_of_prior_pics_present_flag | u(1) |
| control_extension_flag | u(1) |
| if( target_lid_present_flag ) |  |
| target_layer_id | u(7) |
| if( highest_tid_present_flag ) |  |
| highest_temporal_id | u(3) |
| if( handle_as_cvs_start_present_flag ) |  |
| handle_as_cvs_start_flag | u(1) |
| if( no_output_of_prior_pics_present_flag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( control_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| control_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Semantics

target_lid_present_flag equal to 0 specifies that target_layer_id is not present. target_lid_present_flag equal to 1 specifies that target_layer_id is present.

highest_tid_present_flag equal to 0 specifies that highest_temporal_id is not present. highest_tid_present_flag equal to 1 specifies that highest_temporal_id is present.

handle_as_cvs_start_present_flag equal to 0 specifies that handle_as_cvs_start_flag is not present. handle_as_cvs_start_present_flag equal to 1 specifies that handle_as_cvs_start_flag is present.

no_output_of_prior_pics_present_flag equal to 0 specifies that no_output_of_prior_pics_flag is not present. no_output_of_prior_pics_present_flag equal to 1 specifies that no_output_of_prior_pics_flag is present.

control_extension_flag equal to 0 specifies that no control_extension_data_flag syntax elements are present in the decoding control RBSP syntax structure. control_extension_flag equal to 1 specifies that there are control_extension_data_flag syntax elements present in the decoding control RBSP syntax structure.

target_layer_id, when present, specifies the value of TargetLayerId that applies from the access unit containing this decoding control RBSP up to but excluding the next access unit, in decoding order, containing a decoding control NAL unit with target_layer_id. When no decoding control NAL unit with target_layer_id is present in the first access unit of the bitstream, TargetLayerId is set equal to vps_included_layer_id[ 0 ] from the beginning of the bitstream up to but excluding the first access unit, in decoding order, containing a decoding control NAL unit with target_layer_id. When present in multiple decoding control NAL units in an access unit, all target_layer_id values in the access unit shall be the same. A decoding control NAL unit with target_layer_id shall not be present in an access unit that is not a CVSS access unit.

highest_temporal_id, when present, specifies the value of HighestTid that applies from the access unit containing this decoding control RBSP up to but excluding the next access unit, in decoding order, containing a decoding control NAL unit with highest_temporal_id. When no decoding control NAL unit with highest_temporal_id is present in the first access unit of the bitstream, HighestTid is set equal to sps_max_sub_layers_minus1 from the beginning of the bitstream up to but excluding the first access unit, in decoding order, containing a decoding control NAL unit with highest_temporal_id. When present in multiple decoding control NAL units in an access unit, all highest_temporal_id values in the access unit shall be the same. A decoding control NAL unit with highest_temporal_id shall not be present in an access unit that is not a CVSS access unit.

The coded picture associated with this decoding control NAL unit is the coded picture that is contained in the access unit that contains this decoding control NAL unit and has the same NuhLayerId value as this decoding control NAL unit.

handle_as_cvs_start_flag, when present, specifies the value of HandleAsCvsStartFlag for the coded picture associated with this decoding control NAL unit. When the coded picture associated with this decoding control NAL unit is not an IRAP picture and is not a GRA picture, handle_as_cvs_start_flag shall not be present. When multiple decoding control NAL units containing handle_as_cvs_start_flag and having the same value of NuhLayerId are present in an access unit, all handle_as_cvs_start_flag values in these decoding control NAL units shall be the same. When no decoding control NAL unit with handle_as_cvs_start_flag equal to 1 is associated with a coded picture, HandleAsCvsStartFlag is set equal to 0 for the coded picture.

no_output_of_prior_pics_flag, when present, specifies the value of NoOutputOfPriorPicsFlag for coded picture associated with this decoding control NAL unit. When the coded picture associated with this decoding control NAL unit is not an IDR picture, no_output_of_prior_pics_flag shall not be present. When multiple decoding control NAL units containing no_output_of_prior_pics_flag and having the same value of NuhLayerId are present in an access unit, all no_output_of_prior_pics_flag values in these decoding control NAL units shall be the same.

control_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to this version of this Specification shall ignore all control_extension_data_flag syntax elements.

## Specification of the NAL unit type and its constraints

#### 7.4.2.2	NAL unit header semantics

…

| NalUnitType | Name of NalUnitType | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 5 | DECO_NUT | Decoding control decoding_control_rbsp( ) | non-VCL |

…

#### 7.4.2.4.4	Order of NAL units and coded pictures and their association to access units

…

Let firstBlPicNalUnit be the first VCL NAL unit of a coded picture. The first of any of the following NAL units preceding firstBlPicNalUnit and succeeding the last VCL NAL unit preceding firstBlPicNalUnit, if any, specifies the start of a new access unit:

–	access unit delimiter NAL unit (when present),

–	DPS NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit (when present),

–	PPS NAL unit (when present),

–	APS NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	Decoding control NAL unit (when present),

–	NAL units with NalUnitType equal to RSV_NVCL_5, RSV_NVCL_6, RSV_NVCL_21, or RSV_NVCL_22 (when present),

NOTE – The first NAL unit preceding firstBlPicNalUnit and succeeding the last VCL NAL unit preceding firstBlPicNalUnit, if any, can only be one of the above-listed NAL units.

When there is none of the above NAL units preceding firstBlPicNalUnit and succeeding the last VCL NAL preceding firstBlPicNalUnit, if any, firstBlPicNalUnit starts a new access unit.

The order of the coded pictures and non-VCL NAL units within an access unit shall obey the following constraints:

–	When an access unit delimiter NAL unit is present, it shall be the first NAL unit. There shall be at most one access unit delimiter NAL unit in any access unit.

–	Decoding control NAL unit(s) with a particular NuhLayerId value, when present, shall precede the first VCL NAL unit with the same NuhLayerId value, if any, within the access unit.

–	When any DPS NAL units, VPS NAL units, SPS NAL units, PPS NAL units, APS NAL units, prefix SEI NAL units, NAL units with  NalUnitType equal to RSV_NVCL_5, RSV_NVCL_6, RSV_NVCL_21, or RSV_NVCL_22, or NAL units with NalUnitType in the range of UNSPEC28..UNSPEC29 are present, they shall not follow the last VCL NAL unit of the access unit.

## General decoding process changes (clause 8.1.1)

Input to this process is a bitstream. Output of this process is a list of decoded pictures.

The decoding process is specified such that all decoders that conform to a specified profile and level will produce numerically identical cropped decoded output pictures when invoking the decoding process associated with that profile for a bitstream conforming to that profile and level. Any decoding process that produces identical cropped decoded output pictures to those produced by the process described herein (with the correct output order or output timing, as specified) conforms to the decoding process requirements of this Specification.

For each CVS in the bitstream, the variable TargetLayerId, which identifies the target layer to be decoded, and the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, are specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetLayerId, TargetLayerId is set by the external means.

–	Otherwise, TargetLayerId is set equal to vps_included_layer_id[ 0 ].

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

For each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10 is applied with the CVS, TargetLayerId, and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode. After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

[Ed. (MH): The following paragraphs were moved downwards within this clause so that the bitstream BitstreamToDecode is derived before them.]

For each IRAP picture in the bitstream BitstreamToDecode, the following applies:

–	If the picture is an IDR picture, the first picture in the bitstream in decoding order, or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1.

–	Otherwise, if some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag to a value for the picture, HandleCraAsCvsStartFlag is set equal to the value provided by the external means and NoIncorrectPicOutputFlag is set equal to HandleCraAsCvsStartFlag.

–	Otherwise, HandleCraAsCvsStartFlag and NoIncorrectPicOutputFlag are both set equal to 0.

For each GRA picture in the bitstream BitstreamToDecode, the following applies:

–	If the current picture is the first picture in the bitstream in decoding order or the first picture that follows an end of sequence NAL unit in decoding order, the variable NoIncorrectPicOutputFlag is set equal to 1.

–	Otherwise, NoIncorrectPicOutputFlag is set equal to 0HandleAsCvsStartFlag.

NOTE – The above operations, for both IRAP pictures and GRA pictuures, are needed for identification of the CVSs in the bitstream.

Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

## Removal of no_output_of_prior_pics_flag from the slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| if( rect_slice_flag ¦ ¦ NumBricksInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && !single_brick_per_slice_flag ) |  |
| num_bricks_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( NalUnitType = = GRA_NUT ) |  |
| recovery_poc_cnt | se(v) |
| slice_pic_order_cnt_lsb | u(v) |
| if( NalUnitType = = IDR_W_RADL ¦ ¦ NalUnitType = = IDR_N_LP ¦ ¦ NalUnitType = = CRA_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| … |  |

## HRD changes related to no_output_of_prior_pics_flag

### C.3.2	Removal of pictures from the DPB before decoding of the current picture

The removal of pictures from the DPB before decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously at the CPB removal time of the first decoding unit of access unit n (containing the current picture) and proceeds as follows:

–	The decoding process for reference picture list construction as specified in clause 8.3.2 is invoked and decoding process for reference picture marking as specified in clause 8.3.3 is invoked.

–	When the current picture is an IRAP picture with NoIncorrectPicOutputFlag equal to 1 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_flag, bit_depth_luma_minus8, bit_depth_chroma_minus8 or sps_max_dec_pic_‌buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_‌flag, bit_depth_luma_minus8, bit_depth_chroma_minus8 or sps_max_dec_pic_buffering_‌minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is associated with a decoding control NAL unit including no_output_of_prior_pics_flag, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to 0.

…

### C.3.3	Picture output

The processes specified in this clause happen instantaneously at the CPB removal time of access unit n, CpbRemovalTime[ n ].

When picture n has PictureOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows:

DpbOutputTime[ n ] = CpbRemovalTime[ n ] + ClockTick * picDpbOutputDelay –	(C-12)
				picDpbOutputDelta[ OpTid ]

where picDpbOutputDelay is the value of pic_dpb_output_delay and picDpbOutputDelta is the value of picDpbOutputDelta[ OpTid ] derived according to cpb_removal_delay_minus1[ OpTid ], and cpb_removal_delay_delta_idx[ OpTid ] in the picture timing SEI message associated with access unit n, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ OpTid ] ] in the buffering period SEI message associated with access unit n.

The output of the current picture is specified as follows:

–	If PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is equal to CpbRemovalTime[ n ], the current picture is output.

–	Otherwise, if PictureOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in clause C.3.4.

–	Otherwise (PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than CpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause C.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flagNoOutputOfPriorPicsFlag is derived to be equal to 1 at a time that precedes DpbOutputTime[ n ].

…

#### C.5.2.2	Output and removal of pictures from the DPB

The output and removal of pictures from the DPB before the decoding of the current picture (but after parsing the slice header of the first slice of the current picture) happens instantaneously when the first decoding unit of the access unit containing the current picture is removed from the CPB and proceeds as follows:

–	The decoding process for reference picture list construction as specified in clause 8.3.2 and decoding process for reference picture marking as specified in clause 8.3.3 is invoked.

–	If the current picture is an IRAP picture with NoIncorrectPicOutputFlag equal to 1 that is not picture 0, the following ordered steps are applied:

1.	The variable NoOutputOfPriorPicsFlag is derived for the decoder under test as follows:

–	If the current picture is a CRA picture, NoOutputOfPriorPicsFlag is set equal to 1 (regardless of the value of no_output_of_prior_pics_flag).

–	Otherwise, if the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_flag, bit_depth_luma_minus8, bit_depth_chroma_minus8 or sps_max_dec_pic_‌buffering_minus1[ HighestTid ] derived from the active SPS is different from the value of pic_width_in_luma_samples, pic_height_in_luma_samples, chroma_format_idc, separate_colour_plane_‌flag, bit_depth_luma_minus8, bit_depth_chroma_minus8 or sps_max_dec_pic_buffering_‌minus1[ HighestTid ], respectively, derived from the SPS active for the preceding picture, NoOutputOfPriorPicsFlag may (but should not) be set to 1 by the decoder under test, regardless of the value of no_output_of_prior_pics_flag.

NOTE – Although setting NoOutputOfPriorPicsFlag equal to no_output_of_prior_pics_flag is preferred under these conditions, the decoder under test is allowed to set NoOutputOfPriorPicsFlag to 1 in this case.

–	Otherwise, if the current picture is associated with a decoding control NAL unit including no_output_of_prior_pics_flag, NoOutputOfPriorPicsFlag is set equal to no_output_of_prior_pics_flag.

–	Otherwise, NoOutputOfPriorPicsFlag is set equal to 0.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).