| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0396-v2 |
| --- | --- |

| Title: | Non-CE3: Simplified mode mapping in MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jangwon Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 { jangwon84.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to simplify mode mapping method in matrix intra prediction (MIP). The proposed method modifies the mode mapping method between MIP and intra mode, as follows:

- Method 1a: All MIP modes are mapped as PLANAR intra mode

- Method 1b: All MIP modes are mapped as DC intra mode

- Method 2: All intra modes are mapped as MIP mode 0

With this method, the mode mapping table of MIP can be removed with the negligible BD-rate loss.

Experimental results for Method 1a and 1b show 0.01%, -0.02%, 0.04% and 0.00%, -0.04%, 0.01% BD-rate changes on Y, Cb, and Cr components, respectively in All Intra configuration, and Method 2 shows 0.00%, -0.04%, and -0.01% BD-rates on Y, Cb, and Cr components, respectively in All Intra configuration.

Introduction

Matrix intra prediction (MIP) was adopted in the 14th JVET meeting. Since MIP mode considers various intra prediction modes and its intra prediction mode also needs to be considered in regular intra prediction mode (i.e., non MIP mode) , the following mapping tables shown in Table 1 and Table 2 are required.

Table 1 – Specification of mapping between intra prediction and MIP modes

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 17 | 0 | 5 |
| 1 | 17 | 0 | 1 |
| 2, 3 | 17 | 10 | 3 |
| 4, 5 | 9 | 10 | 3 |
| 6,7 | 9 | 10 | 3 |
| 8, 9 | 9 | 10 | 3 |
| 10, 11 | 9 | 10 | 0 |
| 12, 13 | 17 | 4 | 0 |
| 14, 15 | 17 | 6 | 0 |
| 16, 17 | 17 | 7 | 4 |
| 18, 19 | 17 | 7 | 4 |
| 20, 21 | 17 | 7 | 4 |
| 22, 23 | 17 | 5 | 5 |
| 24, 25 | 17 | 5 | 1 |
| 26, 27 | 5 | 0 | 1 |
| 28, 29 | 5 | 0 | 1 |
| 30, 31 | 5 | 3 | 1 |
| 32, 33 | 5 | 3 | 1 |
| 34, 35 | 34 | 12 | 6 |
| 36, 37 | 22 | 12 | 6 |
| 38, 39 | 22 | 12 | 6 |
| 40, 41 | 22 | 12 | 6 |
| 42, 43 | 22 | 14 | 6 |
| 44, 45 | 34 | 14 | 10 |
| 46, 47 | 34 | 14 | 10 |
| 48, 49 | 34 | 16 | 9 |
| 50, 51 | 34 | 16 | 9 |
| 52, 53 | 34 | 16 | 9 |
| 54, 55 | 34 | 15 | 9 |
| 56, 57 | 34 | 13 | 9 |
| 58, 59 | 26 | 1 | 8 |
| 60, 61 | 26 | 1 | 8 |
| 62, 63 | 26 | 1 | 8 |
| 64, 65 | 26 | 1 | 8 |
| 66 | 26 | 1 | 8 |

Table 2 – Specification of mapping between MIP and intra prediction modes

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 0 | 0 | 1 |
| 1 | 18 | 1 | 1 |
| 2 | 18 | 0 | 1 |
| 3 | 0 | 1 | 1 |
| 4 | 18 | 0 | 18 |
| 5 | 0 | 22 | 0 |
| 6 | 12 | 18 | 1 |
| 7 | 0 | 18 | 0 |
| 8 | 18 | 1 | 1 |
| 9 | 2 | 0 | 50 |
| 10 | 18 | 1 | 0 |
| 11 | 12 | 0 |  |
| 12 | 18 | 1 |  |
| 13 | 18 | 0 |  |
| 14 | 1 | 44 |  |
| 15 | 18 | 0 |  |
| 16 | 18 | 50 |  |
| 17 | 0 | 1 |  |
| 18 | 0 | 0 |  |
| 19 | 50 |  |  |
| 20 | 0 |  |  |
| 21 | 50 |  |  |
| 22 | 0 |  |  |
| 23 | 56 |  |  |
| 24 | 0 |  |  |
| 25 | 50 |  |  |
| 26 | 66 |  |  |
| 27 | 50 |  |  |
| 28 | 56 |  |  |
| 29 | 50 |  |  |
| 30 | 50 |  |  |
| 31 | 1 |  |  |
| 32 | 50 |  |  |
| 33 | 50 |  |  |
| 34 | 50 |  |  |

Proposed method

To remove the mapping tables between MIP and intra mode, the mode mapping between MIP and intra mode is  defined as follows:

- Method 1a: All MIP modes are mapped as PLANAR intra mode

- Method 1b: All MIP modes are mapped as DC intra mode

- Method 2: All intra modes are mapped as MIP mode 0

With the method 1a (or 1b), all MIP modes are mapped as PLANAR (or DC) intra mode and thus the table 2 can be removed. As similar approach, with the method 2, all intra modes are mapped as MIP mode 0 and thus the table 1 can be removed.

Experimental results

The proposed method has been implemented using VTM5.0 as software base according to the common test conditions defined in JVET-N1010 [1].

Test1 (Method 1a)

In the test 1, all MIP modes are mapped as PLANAR intra mode.

Table 1 and Table 2 show the test results in AI, RA configuration.

Table 1 Experimental results of test1 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.04% | 99% | 100% |
| Class A2 | 0.03% | 0.02% | -0.02% | 99% | 100% |
| Class B | 0.01% | -0.04% | 0.09% | 99% | 98% |
| Class C | 0.00% | -0.12% | 0.05% | 100% | 99% |
| Class E | 0.03% | 0.01% | 0.03% | 98% | 96% |
| Overall | 0.01% | -0.02% | 0.04% | 99% | 98% |
| Class D | -0.03% | 0.19% | 0.00% | 99% | 87% |
| Class F | -0.02% | -0.02% | 0.08% | 99% | 98% |

Table 2 Experimental results of test1 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.22% | 98% | 98% |
| Class A2 | 0.07% | 0.22% | 0.06% | 100% | 100% |
| Class B | 0.01% | 0.04% | 0.01% | 100% | 99% |
| Class C | -0.02% | 0.15% | 0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% | 100% | 99% |
| Class D | 0.00% | 0.07% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.13% | 0.17% | 100% | 100% |

Test2 (Method 1b)

In the test 2, all MIP modes are mapped as DC intra mode. Table 3 and Table 4 show the test results in AI, RA configuration

Table 3 Experimental results of test2 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | -0.05% | 100% | 101% |
| Class A2 | 0.01% | -0.05% | 0.00% | 101% | 102% |
| Class B | 0.01% | -0.04% | 0.02% | 100% | 100% |
| Class C | 0.00% | -0.02% | 0.04% | 100% | 100% |
| Class E | 0.00% | 0.03% | 0.03% | 100% | 101% |
| Overall | 0.00% | -0.04% | 0.01% | 100% | 101% |
| Class D | 0.00% | 0.18% | -0.01% | 100% | 101% |
| Class F | -0.01% | 0.01% | -0.08% | 101% | 101% |

Table 4 Experimental results of test2 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.13% | -0.01% | 98% | 98% |
| Class A2 | 0.03% | -0.01% | 0.03% | 99% | 98% |
| Class B | 0.05% | -0.14% | -0.31% | 100% | 100% |
| Class C | -0.02% | -0.25% | -0.12% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.14% | -0.13% | 100% | 100% |
| Class D | 0.02% | 0.22% | 0.20% | 100% | 100% |
| Class F | 0.01% | 0.21% | 0.08% | 100% | 100% |

Test3 (Method 2)

In the test 3, all intra modes are mapped as MIP mode 0.

Table 5 and Table 6 show the test results in AI, RA configuration.

Table 1 Experimental results of test1 for all-intra (AI) test condition; anchor is VTM5

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.03% | 98% | 98% |
| Class A2 | 0.00% | -0.01% | -0.02% | 98% | 96% |
| Class B | 0.01% | -0.04% | -0.01% | 99% | 98% |
| Class C | 0.01% | -0.09% | -0.11% | 100% | 99% |
| Class E | 0.00% | -0.04% | 0.10% | 99% | 99% |
| Overall | 0.00% | -0.04% | -0.01% | 99% | 98% |
| Class D | 0.00% | 0.14% | -0.03% | 100% | 97% |
| Class F | -0.01% | 0.00% | 0.05% | 99% | 98% |

Table 2 Experimental results of test1 for random-access (RA) test condition; anchor is VTM5

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 CTC |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.13% | 99% | 99% |
| Class A2 | 0.02% | -0.07% | -0.01% | 100% | 101% |
| Class B | 0.02% | -0.02% | -0.14% | 100% | 100% |
| Class C | -0.03% | -0.19% | 0.00% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.20% | 0.15% | 99% | 97% |
| Class F | 0.03% | -0.06% | 0.02% | 99% | 99% |

Conclusion

To remove the mapping tables between MIP and intra mode, the simplified mode mapping methods are proposed. With the proposed methods, the mode mapping table of MIP can be removed without meaningful coding performance changes. It is recommended to adopt the proposed method to be included in the next VTM.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th meeting, Geneva, CH, 19-27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).