| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0402-v3 |
| --- | --- |

| Title: | Non-CE6: Simplification of context modeling on MTS index |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, moonmo.koo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

This contribution proposes simplification of context modeling on MTS index. The number of context coded bins and context models is reduced from 4 to 2 and from 9 to 2, respectively. The proposed scheme is reported to provide BD-rate changes of 0.0%, 0.0%, and 0.04% in AI, RA, and LDB.

Introduction

In VVC-5.0, tu_mts_idx syntax which specifies transform kernels of multiple transform selection (MTS) uses 9 context models as shown in Table 1. The first bin for the syntax is selected by depending on cqtDepth which means qualtree depth for current luma coding block. The value shall be in the range of 0 to 5. Each bin of second, the third, and fourth positions is coded by a context model, respectively.

Table 1: Assignment of ctxInc to syntax elements for tu_mts_idx

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | cqtDepth | 6 | 7 | 8 | na | Na |

Proposed method

This contribution proposes simplification of context modeling on MTS index. Both the first and the second bins are coded by context coded bin. The others are coded by bypass coding as shown in Table 2. The number of context model can be reduced from 9 to 2. In addition, the worst case number of context coded bin can be reduced from 4 to 2.

Table 2: Assignment of ctxInc to syntax elements for tu_mts_idx of proposed method

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| tu_mts_idx[ ][ ] | 0 | 1 | bypass | bypass | na | na |

Experimental results

The proposed methods have been implemented on top of VTM-5.0 and tested under common test conditions [1].

CTC results

Table 3. Performance of proposed method in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | -0.08% | -0.05% | 100% | 101% |
| Class B | 0.00% | -0.01% | 0.06% | 100% | 100% |
| Class C | -0.01% | -0.07% | 0.09% | 100% | 99% |
| Class E | -0.01% | 0.05% | -0.01% | 100% | 100% |
| Overall | 0.00% | -0.02% | 0.03% | 100% | 100% |
| Class D | 0.01% | 0.04% | -0.10% | 100% | 101% |
| Class F | -0.03% | 0.00% | -0.10% | 101% | 100% |

Table 4. Performance of proposed method in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.13% | 0.08% | 101% | 101% |
| Class A2 | 0.02% | -0.10% | 0.03% | 101% | 101% |
| Class B | 0.01% | -0.10% | -0.13% | 100% | 100% |
| Class C | -0.03% | -0.07% | -0.23% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.08% | 100% | 101% |
| Class D | 0.01% | 0.12% | -0.18% | 100% | 101% |
| Class F | -0.01% | -0.15% | -0.01% | 100% | 100% |

Table 5. Performance of proposed method in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.03% | -0.16% | 102% | 103% |
| Class C | 0.06% | 0.20% | -0.27% | 101% | 101% |
| Class E | 0.05% | -0.07% | 0.49% | 100% | 101% |
| Overall | 0.04% | 0.06% | -0.04% | 101% | 102% |
| Class D | -0.03% | -0.89% | 0.75% | 101% | 101% |
| Class F | -0.02% | -0.31% | -0.52% | 101% | 102% |

Non-CTC results with Inter MTS on

Table 6. Performance of proposed method in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | -0.03% | -0.02% | 100% | 101% |
| Class B | -0.01% | -0.03% | -0.10% | 100% | 101% |
| Class C | -0.02% | 0.08% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.00% | -0.04% | 100% | 100% |
| Class D | -0.03% | 0.18% | -0.37% | 100% | 100% |
| Class F | -0.03% | -0.16% | 0.07% | 100% | 100% |

Table 7. Performance of proposed method in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.06% | 0.14% | -0.30% | 100% | 101% |
| Class E | -0.01% | -0.06% | 1.58% | 100% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | 0.09% | -0.91% | 101% | 102% |
| Class F | -0.05% | -0.04% | -0.29% | 101% | 102% |

Conclusion

In this contribution, simplification of context modeling on MTS index is proposed. It is recommended to adopt the proposed method.

Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1028, 14th Meeting: Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).