| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0408-v2 |
| --- | --- |

| Title: | Non-CE3: On rounding shift of MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@sony.com, Masaru.Ikeda@sony.com, teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, experimental results are reported which an adaptive rounding shift in the matrix based intra prediction (MIP) is simplified. It is introduced that MIP process, needs an adaptively shift operation. To simplify it, the adaptive shift operation is fixed, it is tested using VTM-5.0. The simulation results are reported that 0.00%, 0.00% and 0.03% BD-rate for AI, RA and LB case, the encoding times are 100%, 100% and 99% for AI, RA and LB, the decoding times are 100%, 100% and 100% for AI, RA and LB.

Introduction

A technique to improve intra prediction called MIP was adopted in the 14th JVET meeting in Geneva [1]. In this MIP process, there are three steps that are averaging, matrix computation and up-sampling. The matrix computation process is described in [2], section 8.4.5.2.1	“Matrix-based intra sample prediction”.

In the matrix computation process, a matrix multiplication and a biasing are computed as shown as below equation.

oW = 1 << ( sW − 1 )

predMip[ x ][ y ] = ( (i = 02 * boundarySize - 1mWeight i  y*incH*predC+x*incW *p i ) + 
	( vBias[ y * incH * predC + x * incW ]  << sB ) + oW ) >> sW	(8-69)

Where, the variable sW is derived using MipSizeId and modeId as specified in following Table.

Table 8-8 – Specification of weight shift sW depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| 1 | 8 | 8 | 8 | 9 | 8 | 8 | 8 | 8 | 9 | 8 |  |  |  |  |  |  |  |  |
| 2 | 8 | 8 | 8 | 8 | 8 | 8 |  |  |  |  |  |  |  |  |  |  |  |  |

The variable mWeight and vBias are weight parameters and bias vectors for MIP.

In this contribution, to reduce complexity and simplify the text, it is considered to fix the value of sW. For this purpose, the weight and bias are changed. In the experiment, we integrated and tested to fix sW with 8.

For the simplification of adoptive rounding shift, the specification draft is changed as shown in sub clause 1.1.

In this contribution, the BD-rate impact is reported when VTM-5.0 was modified as the sub clause 1.1.

Text changes

- The variable sW is derived using MipSizeId[ xTbCmp ][ yTbCmp ] and modeId as specified in Table 8-8.

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipW − 1, y = 0..mipH − 1 are derived as follows:

oW = 1 << ( sW − 1 )		(8-65)

sB = BitDepthY − 1		(8-66)

incW = ( predC > mipW )  ?  2  :  1		(8-67)

incH = ( predC > mipH )  ?  2  :  1		(8-68)

predMip[ x ][ y ] = ( (i = 02 * boundarySize - 1mWeight i  y*incH*predC+x*incW *p i ) + 
	( vBias[ y * incH * predC + x * incW ]  << sB ) + oW 1<<7 ) >> sW 8	(8-69)

…

Table 8-8 – Specification of weight shift sW depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
| 1 | 8 | 8 | 8 | 9 | 8 | 8 | 8 | 8 | 9 | 8 |  |  |  |  |  |  |  |  |
| 2 | 8 | 8 | 8 | 8 | 8 | 8 |  |  |  |  |  |  |  |  |  |  |  |  |

Experiment

As introduced above, three tests carried out. JVET common test condition [3] is followed. For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: AVX2.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | -0.03% | 102% | 100% |
| Class B | 0.01% | -0.08% | 0.01% | 100% | 100% |
| Class C | 0.01% | -0.01% | -0.03% | 99% | 100% |
| Class E | -0.01% | -0.04% | -0.03% | 100% | 100% |
| Overall | 0.00% | -0.03% | -0.01% | 100% | 100% |
| Class D | -0.01% | 0.05% | -0.14% | 100% | 100% |
| Class F | 0.00% | 0.06% | 0.07% | 100% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.23% | 0.10% | 101% | 100% |
| Class A2 | 0.00% | 0.15% | 0.09% | 103% | 100% |
| Class B | 0.00% | -0.01% | -0.11% | 100% | 100% |
| Class C | 0.01% | -0.09% | 0.09% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | 0.03% | 101% | 100% |
| Class D | 0.07% | 0.11% | 0.01% | 100% | 100% |
| Class F | 0.01% | 0.05% | 0.12% | 100% | 100% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.35% | -0.21% | 99% | 100% |
| Class C | 0.03% | 0.20% | -0.17% | 98% | 100% |
| Class E | 0.10% | 0.16% | 0.41% | 99% | 100% |
| Overall | 0.03% | 0.25% | -0.04% | 99% | 100% |
| Class D | 0.04% | -0.03% | 0.84% | 99% | 100% |
| Class F | -0.03% | -0.01% | 0.17% | 98% | 100% |

Recommendation

In this contribution, an adaptive rounding shift in MIP was introduced. The test results were reported when the shift was fixed. It’s confirmed that the BD-rate impact is negligible. Based on the experimental results, it is recommended to adopt into the next version of specification of VVC.

References

[1]	J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019.

[2]	B. Bross et al., “Versatile Video Coding (Draft 5)”, JVET-N1001-v7, Geneva, Switzerland, March 2019

[3]	F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).