| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0410-v3 |
| --- | --- |

| Title: | CE6-related: Signalling of Low Frequency Non-Separable Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kyohei Unno Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ky-unno@kddi-research.jp |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes the signalling of Low Frequency Non-Separable Transform (LFNST) which was adopted into VVC WD5 and VTM-5.0 in the previous meeting. In the current design, the necessity of decoding lfnst_idx, which indicates a kernel type of LFNST, is determined according to the two type of the numbers of non-zero transform coefficients in a coding block. These counters are, however, utilized only for LFNST signalling. In this contribution, it is proposed to remove the whole counters from the condition check by using existing syntax elements, i.e., cbf, last_sig_coeff and coded_sub_block_flag. In addition, it is also proposed that the block size restriction is added to LFNST to meet VPDU constraints. Simulation results of the proposed method implemented on top of VTM-5.0 reportedly show that the overall BD-rates changing for luma in AI/RA/LDB are 0.02%/0.00%/0.00% comparing VTM-5.0. In addition, the results of the proposed method implemented on top of CE6-2.1a show the overall BD-rates changing for luma in AI/RA/LDB are 0.08%/0.01%/0.00% comparing VTM-5.0..

Introduction

Low Frequency Non-Separable Transform (LFNST) [1] was adopted to improve coding performance in the previous meeting and it is included in VVC WD5 [2] and VTM-5.0 [3]. In the LFNST signalling, two counters are used to determine the necessity of decoding lfnst_idx, which indicates a kernel type of LFSNT; one is the number of non-zero transform coefficients in the whole area and the other is the number of non-zero transform coefficients in the zeroing area of each coding block. The first counter is named by numSigCoeff and the second one is named by numZeroOutSigCoeff as described in Table 1 and Table 2. These counters, which are utilized only for determining LFNST applicability, requires an additional function block as coefficients counting. The worst case for coefficient scanning in each TU is 4,096 steps when TU size is 64 x 64. Therefore, these counters should be removed in terms of reducing hardware complexity. In addition, it is pointed out that the current decoding process of lfnst_idx violates the VPDU constraints in case that at least either CU width or height is larger than 64 pixels since the lfnst_idx is located not in the transform unit but in the coding unit.

Proposed Method

The proposed method is to remove the two counters by using the existing syntax elements, i.e., cbf, last_sig_coeff and coded_sub_block_flag. In addition, it is also proposed that the block size restriction is added to LFNST to meet VPDU constraints. If all of the following conditions are true, lfnst_idx is decoded.

- Both height and width of CU are equal or smaller than 64. It is noted that this restriction is added to meet VPDU constraints and doesn’t affect experiment results under common test conditionof JVET-N1010 [4].

- Both last_sig_coeff and coded_sub_block_flag indicate the no existence of non-zero coefficients in the zeroing area which is variable depended on coding block size.

- cbf and last_sig_coeff satisfy one or more of the following conditions:

  - The block tree type is dual tree, cbf in luma block is not equal to 0 and last_sig_coeff of luma block indicates the non-DC position.

  - The block tree type is dual tree, one of the cbf in chroma block is not equal to 0 and last_sig_coeff of chroma blocks indicates the non-DC position.

  - The block tree type is single tree, one of the cbf in luma/chroma blocks and one of the last_sig_coeff in luma/chroma blocks indicates the non-DC position.

Considering that the above condition check, the LFNST relevant part of syntax table can be modified as described from Table 1 to Table 3. In the proposed syntax table, additional parameter, i.e., cbf_check and lastScanPos_check, which are used for the above condition 3, are utilized to determine the necessity of decoding lfnst_idx in place of the two counters. The maximum numbers of cbf_check and lastScanPos_check steps are equal to the number of TU, so hardware complexity of the proposed condition check is smaller than those of the two counters.

Table 1 The LFNST relevant part of coding unit syntax table in VVC WD5

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| … |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| cbf_check = 0 |  |
| lastScanPos_check = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] && Max( lfnstWidth, lfnstHeight ) <= 64 && ( ( ( lfnstWidth = = 4 && lfnstHeight = = 4) ¦ ¦ ( lfnstWidth = = 8 && lfnstHeight = = 8) ) && !( lastScanPos >= 8 && lastScanPos < 48) && !( coded_sub_block_flag[ 0 ][ 1 ] = = 1 && coded_sub_block_flag[ 1 ][ 0 ] = = 1 ) ) && ( !cbf_check = = 0 && lastScanPos_check > 0 ) ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| … |  |

Table 2 The LFNST relevant part of transform unit syntax table in VVC WD5

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex ) { | Descriptor |
| --- | --- |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cbf_cb[ x0 ][ y0 ] | ae(v) |
| tu_cbf_cr[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_cb ¦ ¦ tu_cbf_cr) |  |
| cbf_check = 1 |  |
| } |  |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ¦ ¦ CbWidth[ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_cbf_luma[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma) |  |
| cbf_check = 1 |  |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_cbf_luma[ x0 ][ y0 ] |  |
| } |  |

Table 3 The LFNST relevant part of residual coding syntax table in VVC WD5

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |
| log2ZoTbWidth = 4 |  |
| else |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |
| log2ZoTbHeight = 4 |  |
| else |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |
| if( log2TbWidth > 0 ) |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| log2SbW = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| log2SbH = log2SbW |  |
| if( log2TbWidth + log2TbHeight > 3 ) { |  |
| if( log2TbWidth < 2 ) { |  |
| log2SbW = log2TbWidth |  |
| log2SbH = 4 − log2SbW |  |
| } else if( log2TbHeight < 2 ) { |  |
| log2SbH = log2TbHeight |  |
| log2SbW = 4 − log2SbH |  |
| } |  |
| } |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( !lastScanPos ) |  |
| lastScanPos_check = 1 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| remBinsPass1 = ( ( log2SbW + log2SbH ) < 4 ? 8 : 32 ) |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) { |  |
| numSigCoeff++ |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |
| numZeroOutSigCoeff++ |  |
| } |  |
| … |  |

Experimental Results

The proposed method is implemented on top of the VTM-5.0 and CE6-2.1a [5]. CE6-2.1a is the simplification proposal for LFNST by zeroing out all non-zero coefficients which are at the high-frequency area. In order to verify the performance of the proposed method, we have run simulations according to the common test conditions.

Results under the common test conditions

Computing platforms

The software was compiled by the same compiler which is GCC 4.8.5 on 64-bit CentOS Linux on the different cluster PCs. Since the same coding condition utilizes the same cluster PCs, running time can be comparable within the same condition. It is noted that we disabled the function of Hyperthreading and TurboBoost for getting accurate running times.

    - Test results

      - Results of the proposal implemented on top on  VTM-5.0

From Table 4 to Table 6 show the results of the proposal implemented on top on VTM-5.0 in AI/RA/LDB condition comparing VTM-5.0.

Table 4 The simulation results of the prposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.11% | -0.24% | 100% | 100% |
| Class A2 | 0.03% | -0.10% | -0.17% | 99% | 99% |
| Class B | 0.02% | -0.18% | -0.13% | 100% | 100% |
| Class C | 0.02% | -0.15% | -0.04% | 99% | 99% |
| Class E | 0.02% | -0.12% | -0.11% | 99% | 99% |
| Overall | 0.02% | -0.14% | -0.13% | 99% | 100% |
| Class D | 0.01% | 0.07% | -0.05% | 100% | 100% |
| Class F | 0.00% | -0.05% | 0.05% | 99% | 99% |

Table 5 The simulation results of the prposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.20% | 0.04% | 100% | 100% |
| Class A2 | 0.01% | 0.13% | -0.03% | 100% | 99% |
| Class B | 0.03% | -0.11% | -0.06% | 100% | 100% |
| Class C | -0.04% | -0.30% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.13% | -0.03% | 100% | 100% |
| Class D | 0.02% | -0.01% | 0.03% | 100% | 100% |
| Class F | -0.01% | -0.01% | -0.01% | 100% | 99% |

Table 6 The simulation results of the proposed method

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

      - Results of the proposal implemented on top on CE6-2.1a

From Table 7 to Table 9 show the results of the proposal implemented on top on CE6-2.1a  in AI/RA/LDB condition comparing VTM-5.0.

Table 7 The simulation results of the prposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.03% | -0.21% | 90% | 99% |
| Class A2 | 0.02% | 0.05% | -0.04% | 88% | 99% |
| Class B | 0.10% | -0.22% | -0.08% | 89% | 99% |
| Class C | 0.12% | -0.17% | -0.10% | 87% | 100% |
| Class E | 0.11% | -0.18% | -0.06% | 90% | 99% |
| Overall | 0.08% | -0.12% | -0.09% | 89% | 99% |
| Class D | 0.12% | 0.07% | -0.12% | 88% | 102% |
| Class F | 0.04% | -0.03% | 0.06% | 92% | 99% |

Table 8 The simulation results of the prposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.13% | 0.09% | 97% | 99% |
| Class A2 | 0.00% | 0.10% | 0.07% | 98% | 100% |
| Class B | 0.04% | 0.04% | -0.14% | 98% | 100% |
| Class C | 0.02% | -0.09% | -0.03% | 97% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.02% | 97% | 99% |
| Class D | 0.06% | 0.38% | -0.02% | 98% | 98% |
| Class F | 0.03% | -0.03% | 0.09% | 97% | 100% |

Table 9 The simulation results of the proposed method

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 99% |

    - Reference results of the proposal implemented on top on VTM-5.0

From Table 10 to Table 12 show the results of the proposal implemented on top on VTM-5.0 in AI/RA/LDB condition comparing VTM-5.0 but a part of the proposed signaling of LFNST is different from in 3.1.1.1 as follows:

  - The block tree type is single tree, the cbf in luma blocks and the last_sig_coeff in luma block indicates the non-DC position.

Table 10 The simulation results of the prposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.11% | -0.24% | 100% | 100% |
| Class A2 | 0.03% | -0.10% | -0.17% | 99% | 99% |
| Class B | 0.02% | -0.18% | -0.13% | 100% | 100% |
| Class C | 0.02% | -0.15% | -0.04% | 99% | 99% |
| Class E | 0.02% | -0.12% | -0.11% | 99% | 99% |
| Overall | 0.02% | -0.14% | -0.13% | 99% | 100% |
| Class D | 0.01% | 0.07% | -0.05% | 100% | 100% |
| Class F | 0.00% | -0.05% | 0.05% | 99% | 99% |

Table 11 The simulation results of the prposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.04% | 100% | 100% |
| Class A2 | 0.01% | 0.06% | -0.09% | 100% | 99% |
| Class B | 0.02% | -0.14% | 0.02% | 100% | 100% |
| Class C | -0.07% | -0.21% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.11% | 0.01% | 100% | 100% |
| Class D | 0.02% | 0.05% | 0.06% | 100% | 100% |
| Class F | -0.01% | 0.02% | 0.04% | 100% | 100% |

Table 12 The simulation results of the proposed method

|  | Low Delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |

Conclusion

This contribution proposed the signalling of Low Frequency Non-Separable Transform (LFNST). In the current design, LFNST was determined according to the two types of the number of non-zero transform coefficients in a coding block. The proposed method removed two counters by using block size restriction and existing syntax elements, i.e., cbf, last_sig_coeff and coded_sub_block_flag. Simulation results of the proposed method implemented on top of VTM-5.0 reportedly showed that no BD-rates loss for luma in AI/RA/LDB are 0.02%/0.00%/0.00% comparing VTM-5.0. In addition, the results of the proposed method implemented on top of CE6-2.1a show also no BD-rates loss for luma in AI/RA/LDB are 0.08%/0.01%/0.00% comparing VTM-5.0., Therefore, the adopt of proposed signaling into VVC is recommended in terms of reducing hardware complexity.

- Acknowledgment

We would like to thank LGE for doing crosschecking of our proposal.

References

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, March 2019.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 5 (VTM 5)”, JVET-N1002, Geneva, CH, March 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Suehring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

- M. Siekmann, H. Schwarz, D. Marpe, T. Wiegand, “CE6-2.1: Simplification of Low Frequency Non-Separable Transform”, JVET-O0094, Gothenburg, SE, July 2019.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).