| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July. 2019 | Document: JVET-O0416_v3 |
| --- | --- |

| Title: | Non-CE3: Removal of CCLM context derivation dependency |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiaqi Zhang1, Xiaozhen Zheng2, Chuanmin Jia1, Shanshe Wang1, Siwei Ma1 Peking University1, DJI2 | Email: | zhangjiaqi163mail@163.com xiaozhen.zheng@dji.com |
| Source: | SZ DJI Technology Co., Ltd2., Peking University1 |  |  |

_____________________________

- Abstract

At current VVC working draft, the CCLM mode (INTRA_LT_CCLM) and the CCLM modes using top or left templates (INTRA_L_CCLM, INTRA_T_CCLM) were adopted. When using CCLM prediction, the parsing process of chroma prediction mode exist dependency which causes latency for hardware design. In order to remove such dependency of parsing process of chroma CCLM prediction, this contribution removes the parsing dependency by using the following method:

- Use bypass coding mode when parsing the fourth bin of intra_chroma_pred_mode.

- Use bypass coding mode when parsing the third and fourth bin of intra_chroma_pred_mode.

Simulation results show negligible coding performance impact compared to VTM-5.0 anchor with minor encoding and decoding time change.

Introduction

In intra chroma prediction mode, there are 8 chroma prediction modes when CCLM is turned on. As summarized in Table 1, the context derivation of the fourth bin (binIdx equal to 3) is relied on the value of the third bin (binIdx equal 2). When the bin value at binIdx equal to 2 is ‘0’(chroma prediction mode is either 0,1,2,3), uses bypass mode. Otherwise, when the bin value at binIdx equal to 2 is ‘1’ (chroma prediction mode is either INTRA_L_CCLM, INTRA_T_CCLM), the fourth bin (binIdx equal to 3) uses context based binary coding.

Table. 1: Current context design for chroma prediction mode

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypassss | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |

Proposed Method

In order to remove the context derivation dependency for chroma prediction mode, this contribution suggests always using bypass coding mode for the bin at binIdx equal to 3 that is summarized at Table 2.

Table 2: Proposed modification (the change of context for the bin at binIdx equal to 3 is marked in red)

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | bypass | na | na |

To reduce context number for the syntax intra_chroma_pred_mode, the contribution also suggest using bypass coding mode for the bin at binIdx equal to 2 that is summarized at Table 3.

Table 2: Proposed modification (the change of context for the bin at binIdx equal to 2 and 3 is marked in red)

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | bypass | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | bypass | bypass | na | na |

Experimental Results

The experiments were performed under JVET common test conditions [1].

BD-Rate results of the test on top of the VTM-5.0 compared to the anchor with AI, RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets.

Results of bypass coding mode for the fourth bin of intra_chroma_pred_mode

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.15% | -0.22% | 100% | 99% |
| Class A2 | 0.00% | -0.04% | -0.04% | 101% | 101% |
| Class B | 0.00% | -0.09% | -0.13% | 100% | 101% |
| Class C | 0.01% | -0.04% | -0.02% | 101% | 96% |
| Class E | -0.01% | -0.07% | 0.01% | 100% | 99% |
| Overall | 0.00% | -0.08% | -0.08% | 100% | 99% |
| Class D | -0.01% | 0.08% | -0.02% | 101% | 102% |
| Class F | -0.01% | 0.00% | -0.06% | 101% | 102% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.41% | -0.24% | 101% | 91% |
| Class A2 | 0.04% | 0.02% | 0.10% | 100% | 98% |
| Class B | 0.01% | -0.06% | -0.25% | 99% | 89% |
| Class C | -0.01% | -0.13% | -0.15% | 97% | 90% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.13% | -0.15% | 97% | 99% |
| Class D | 0.03% | -0.10% | 0.34% | 99% | 91% |
| Class F | -0.01% | -0.07% | 0.06% | 95% | 81% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.53% | -0.18% | 112% | 78% |
| Class C | 0.04% | -0.07% | -0.23% | 100% | 96% |
| Class E | 0.08% | 0.63% | 0.88% | 99% | 95% |
| Overall | 0.03% | 0.35% | 0.07% | 104% | 88% |
| Class D | 0.08% | 0.65% | 0.64% | 100% | 96% |
| Class F | 0.00% | -0.35% | 0.00% | 100% | 96% |

Results of bypass coding mode for the third and fourth bin of intra_chroma_pred_mode

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.29% | 0.33% | 100% | 98% |
| Class A2 | 0.01% | -0.08% | -0.01% | 100% | 103% |
| Class B | 0.01% | -0.04% | 0.04% | 98% | 99% |
| Class C | 0.01% | -0.02% | 0.00% | 100% | 99% |
| Class E | 0.00% | -0.13% | -0.07% | 100% | 99% |
| Overall | 0.01% | 0.00% | 0.05% | 99% | 100% |
| Class D | -0.01% | 0.10% | -0.02% | 100% | 101% |
| Class F | 0.01% | -0.01% | 0.03% | 104% | 104% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.33% | 0.41% | 88% | 94% |
| Class A2 | 0.04% | 0.15% | 0.12% | 115% | 105% |
| Class B | 0.01% | 0.14% | 0.05% | 109% | 104% |
| Class C | 0.00% | -0.10% | 0.02% | 97% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.11% | 0.13% | 103% | 102% |
| Class D | 0.02% | 0.01% | 0.03% | 99% | 90% |
| Class F | 0.00% | 0.02% | 0.14% | 96% | 83% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.30% | 0.08% | 132% | 90% |
| Class C | 0.00% | 0.17% | -0.03% | 99% | 93% |
| Class E | 0.06% | 0.06% | -0.50% | 99% | 91% |
| Overall | 0.01% | 0.19% | -0.10% | 112% | 92% |
| Class D | 0.01% | -0.33% | 0.47% | 101% | 98% |
| Class F | 0.01% | 0.24% | -0.45% | 99% | 93% |

Conclusion

This contribution proposes a method that remove context derivation dependency for chroma prediction mode when CCLM is enable.

Simulation results for all tests shows negligible coding performance impact on average compared to VTM 5.0 anchor, while no increase in encoding and decoding time. Therefore, it is recommended to adopt the proposed dependency removal method for chroma prediction mode to the next version of VVC working draft.

Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

Patent rights declaration(s)s

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).