| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0420 |
| --- | --- |

| Title: | CE4-related: Reduce scaling operation number for MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Suhong Wang1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 (Peking University1, DJI2) | Email: | suhong.wang@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution proposes to remove the scaling process in MMVD when the starting MV is a temporal motion vector predictor (TMVP), which can reduce worst case number of scaling operation at decoder side when MMVD is enable and keep same scaling operation number for TMVP at both MMVD on and off.

Experimental results show that the proposed method has no performance impact on VTM-5.0 anchor.

Introduction

In current MMVD design, a motion vector selected form merge candidate list is refined by MMVDOffset. When bi-prediction mode is used, MMVD requires additional motion vector scaling process to derive another MMVDOffset value from the signalled MMVDOffset.

Since a bi-prediction TMVP already requires two motion vector scaling operation and bi-prediction TMVP-based MMVD requires another one scaling, current MMVD design requires three motion vector scaling at worst case which imposes burden on motion vector derivation process. Table 1 summarise the number of scaling operation number for different MV type at MMVD on and off. It can be concluded that the motion vector scaling worst case comes from bi-prediction TMVP-based MMVD.

Table 1. Number of scaling processes during merge mode

| MV type | Number of scaling processes (MMVD on) |  | Number of scaling processes (MMVD off) |  |
| --- | --- | --- | --- | --- |
|  | Uni-prediction | Bi-prediction | Uni-prediction | Bi-prediction |
| Spatial MVP | 0 | 1 | 0 | 0 |
| Temporal MVP | 2 | 3 | 1 | 2 |
| HMVP | 0 | 1 | 0 | 0 |
| Pairwise MVP | 0 | 1 | 0 | 0 |

Proposed Method

In order to reduce the worst case complexity, this contribution proposes to remove the scaling process in MMVD when base MV is a bi-prediction TMVP candidate. In that case, the number of scaling operation is reduced to two which is same as MMVD off case. Table 2 summarise the number of scaling operation number for different MV type at MMVD on and off when the proposed method is used.

Table 2. Number of scaling processes during merge mode when the proposed method is used

| MV type | Number of scaling processes (MMVD on) |  | Number of scaling processes (MMVD off) |  |
| --- | --- | --- | --- | --- |
|  | Uni-prediction | Bi-prediction | Uni-prediction | Bi-prediction |
| Spatial MVP | 0 | 1 | 0 | 0 |
| Temporal MVP | 2 | 2 | 1 | 2 |
| HMVP | 0 | 1 | 0 | 0 |
| Pairwise MVP | 0 | 1 | 0 | 0 |

Taking the distance from current picture to two reference pictures into account, flipping process is kept in the proposed method. Specifically, when the starting MV is bi-prediction MVs with both lists point to the same side of the current picture, the MV offsets of the two MVs are the same, while if the two MVs point to different side of the current picture, the MV offsets of the two MVs are opposite. This process can be described as follow:

if 2 MVs point to same side:

MMVDOffset_L0/L1 = MMVDOffset

MMVDOffset_L1/L0 = MMVDOffset

else

MMVDOffset_L0/L1 = MMVDOffset

MMVDOffset_L1/L0 = -MMVDOffset

Experimental Results

The experiments were performed under JVET common test conditions(CTC)[1].

BD-Rate results of the tests on top of the VTM-5.0[2] compared to the anchor with RA and LDB configurations are shown in following tables. The full results are provided in the attached excel sheets.

In particular, for a bi-prediction TMVP candidate in RA CTC case, the distance from current picture to the two reference pictures are mutually opposite numbers according to the coding parameters specified in the JVET CTC RA configurations files. In that case, scaling operation and flipping operation mentioned in Part 2 perform the same thing. For LDB CTC case, the two reference pictures of a bi-prediction TMVP candidate are the same picture which will not introduce scaling operation in MMVD offset derivation process. Therefore, the proposed modification will not introduce any performance change to VTM-5.0 anchor under JVET CTC.

Table 3. Results of proposed method compared to VTM-5.0

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 91% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 89% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 89% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 89% |
| Class F | 0.00% | 0.00% | 0.00% | 95% | 84% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Changes of text(on top of N1001-v8)

8.5.2.7	Derivation process for merge motion vector difference

…

- Otherwise, if Abs( currPocDiffL0 ) is greater than or equal to Abs( currPocDiffL1 ), the following applies:

td = Clip3( −128, 127, currPocDiffL0 )		(8-357)

tb = Clip3( −128, 127, currPocDiffL1 )		(8-358)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-359)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-360)

mMvdL0[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-361)

mMvdL0[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-362)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture, RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture and the merge motion vector is not generated from temporal motion vector predictor, the following applies:

mMvdL1[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 0 ] + 	(8-363)
	128 − ( distScaleFactor * mMvdL0[ 0 ] >= 0 ) )  >>  8 )

mMvdL1[ 1 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL0[ 1 ] + 	(8-364)
	128 − ( distScaleFactor * mMvdL0[ 1 ] >= 0 ) )  >>  8 )

    - Otherwise, the following applies:

mMvdL1[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL0[ 0 ]  :  −mMvdL0[ 0 ]	(8-365)

mMvdL1[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL0[ 1 ]  :  −mMvdL0[ 1 ]	(8-366)

- Otherwise (Abs( currPocDiffL0 ) is less than Abs( currPocDiffL1 )), the following applies:

td = Clip3( −128, 127, currPocDiffL1 )		(8-367)

tb = Clip3( −128, 127, currPocDiffL0 )		(8-368)

tx = ( 16384 + ( Abs( td )  >>  1 ) ) / td		(8-369)

distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 )  >>  6 )	(8-370)

mMvdL1[ 0 ]  =  MmvdOffset[ xCb ][ yCb ][ 0 ]	(8-371)

mMvdL1[ 1 ]  =  MmvdOffset[ xCb ][ yCb ][ 1 ]	(8-372)

    - If RefPicList[ 0 ][ refIdxL0 ] is not a long-term reference picture, RefPicList[ 1 ][ refIdxL1 ] is not a long-term reference picture and the merge motion vector is not generated from temporal motion vector predictor,  the following applies:

mMvdL0[ 0 ] = Clip3( −215, 215 − 1, (distScaleFactor * mMvdL1[ 0 ] +	(8-373)
	 128 − (distScaleFactor * mMvdL1[ 0 ] >= 0) )  >>  8 )

mMvdL0[ 1 ] = Clip3( −215, 215 − 1, , (distScaleFactor * mMvdL1[ 1 ] +	(8-374)
	 128 − (distScaleFactor * mMvdL1[ 1 ] >= 0) )  >>  8 ) )

    - Otherwise, the following applies:

mMvdL10[ 0 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1 )  ?	
	mMvdL1[ 0 ]  :  −mMvdL1[ 0 ]	(8-375)

mMvdL0[ 1 ] = Sign( currPocDiffL0 )  = =  Sign( currPocDiffL1)  ?	
	mMvdL1[ 1 ]  :  −mMvdL1[ 1 ]	(8-376)

…

Conclusion

This proposal suggests disable the scaling process of offset for MMVD when the starting MV is derived from TMVP. Simulation results show that the proposed method has no impact on VTM-5.0 anchor. It is recommended to adopt it to the next VVC working draft.

Reference

- “JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, doc. JVET-N1010, March 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-5.0

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).