| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0425 |
| --- | --- |

| Title: | Non-CE5: Supplementary results on alternative filter sets for Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Patrice Onno Christophe Gisquet Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 jonathan.taquet@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to extend CE5-4 contribution by adding some limitations on the number of filters signaled by alternative luma filter sets, and by not using ALF Luma filters prediction from static filter sets. These limitations enables to reduce the amount of memory that a decoder should use to store the ALF filters coming from multiple APS.

In a first test, and in order to evaluate the gain of ALF Luma filters prediction from static filter sets in the VTM5.0, this contribution removes the use of that prediction.

In a second test, this contribution reports further results of C5-4.3 test, where ALF filters derivation from static filter sets is removed, and where the numbers of filters for the first and second alternative Luma filter sets are respectively limited to 7 and 3. The average Y/Cb/Cr BDR gains over VTM-5.0 are respectively 0.10%/-0.91%/-1.27% for the AI configuration, 0.03%/-1.90%/-2.28% for the RA configuration, -0.11%/-2.38%/-2.66% for the Low Delay B configuration, and -0.22%/-2.53%/-2.67% for the Low Delay P configuration. It is asserted that in Test 2, the memory for storing ALF Luma filters can be reduced by 80% compared to CE5-4.3 test and it is reduced by 60% compared to VTM5.0. Considering the memory for storing all ALF filters (including Chroma), it is asserted that Test 2 enables to reduce the ALF filters’ memory by 45% compared to VTM-5.0.

For comparison purpose, this contribution also provides results in Test 3 where the maximum number of filters in the first alternative Luma filter set is changed (compared to Test 2) and is the same as the maximum number of filters in VTM-5.0’s Luma filter set (i.e. 25 filters), while for the second alternative Luma filter set the maximum number of filters is set to 3. In that configuration, the average Y/Cb/Cr BDR gains over VTM-5.0 are respectively X.XX%/X.XX%/X.XX% for the AI configuration, X.XX%/X.XX%/X.XX% for the RA configuration, -0.13%/-2.31%/-2.51% for the Low Delay B configuration, and -0.20%/-2.53%/-2.60% for the Low Delay P configuration. It is asserted that in Test 3, the memory for storing ALF Luma filters can be reduced by 44% compared to CE5-4.3 test and it is increased by 12% compared to VTM5.0. Considering the memory for storing all ALF filters (including Chroma), it is asserted that Test 3 increases the ALF filters’ memory by 25% compared to VTM-5.0.

# Introduction

During the 13th JVET Meeting, it has been proposed to add a limitation on the number of filters to signal per slice [4], but it was concluded that it is, “generally, not a big deal (low amount of memory saving, while possibly giving up flexibility)”.

During the 14th JVET Meeting, Non-Linear ALF [5] has been adopted. It slightly increases the amount of memory required by ALF with the need for storing filter clipping parameters. In the same meeting, coding tree block based adaptive loop filter [7] has also been adopted. It allows switching between fixed luma filter sets and transmitted luma filter sets coming from possibly more than one APS (up to 6 APS for luma in VTM5.0). During and after the 14th JVET Meeting, some experts asked to take care of the amount of memory for storing the ALF parameters from multiple APS, so it seems that now memory could start to be an issue.

In the 14th JVET Meeting, we also proposed to introduce alternative luma filter sets and alternative chroma filters [6]. It was multiplying the worst case amount of memory needed to store ALF filters by 8 (with maximum 8 alternatives proposed for both luma filter sets and chroma filters). This proposal is under study in CE5-4 [8] were we have reduced the maximum number of alternative luma filter sets to two, as a compromise for encoder complexity, memory use worst case and coding gains. Then the amount of memory needed for luma filters is doubled while it is still multiplied by 8 for chroma filters, but chroma filter memory used is not so much compared to luma filter sets. Worst case memory used for storing ALF parameters as highly increased when comparing the memory used by VTM-3.0 at the time of 13th JVET meeting to the memory used in CE5-4, which includes alternative luma filter set and alternative chroma filers signaling per APS, and, as in VTM-5.0, multiple APS references and Non-Linear ALF.

Also, since the adoption of Coding Tree Block (CTB) based adaptive loop filter [7], it is required to derive the filters for each one of the 25 classes because of the “prediction” from static filters mechanism. The derived filters can be either stored in memory (which is the case in the current VTM-5.0 implementation), if derived at APS decoding time for instance, or they can computed at the time they are used. In the first case you always need to keep 25 filters per filter set in memory event if the number of filters in the APS have been limited, while in the second case, the complexity of the decoding process is increased.

# Proposed changes

In this contribution, we propose limiting the number of filters signaled in APS and the amount of memory to store them, even once they have been decoded. In such objective of limiting the memory usage while not increasing the decoder complexity, we also propose to not use the prediction of Luma filter sets from fixed Luma filter sets, which does not provide any gains in VTM-5.0.

Results of first test are provided below to illustrate that filter prediction from static filter set does not provide any gain in VTM-5.0. Since it does not provide any gain in VTM-5.0, and because it adds complexity in the case where we want to reduce memory usage, we propose to not use/remove it.

To limit, and reduce the number of filter signaled in APS as described in CE5-4.3, we impose a maximum number of alternative filter set equal to 2 and we add a limit on the number of filters signaled in each alternative filter set. In our second test, we are limiting to 7 filters in the first alternative filer set and to 3 filters in the second alternative filter. The provided results illustrate a good memory reduction while preserving the coding efficiency provided by the results of CE5-4.3 where no limitation of the number of filters are imposed.

As a third test, to keep the current “flexibility” of ALF, we also propose to keep 25 filters in the first alternative filter set but limit to 3 the number of filters in the second alternative filter set.

# Experiments

Three experiments have been performed.

Test 1 is informative and shows the results obtained when disabling the prediction by fixed filter set of Luma filter sets in VTM-5.0. Then in both Test 2 and Test 3, the Luma filter set prediction not used.

Test 2 limits to 7 the number of filters in the first alternative Luma filter set, and to 3 the number of filters in the second alternative Luma filter set.

Test 3 does not limit the number of filters in the first alternative Luma filter set, but limits to 3 the number of filters in the second alternative Luma filter set.

# Maximum memory used for storing ALF filters for one APS:

The maximum memory used for storing ALF filters for one APS are reported below for VTM-5.0 and Test 1 (which is the same), for CE5-4.3, for Test 2 and for Test 3.

## VTM-5.0 and Test 1:

(8 bits x 6 coefficients + 2 bits x 6 clipping indexes) x 1 chroma filter

+ (8 bits x 12 coefficients + 2 bits x 12 clipping indexes) x 25 luma filters

= 60 bits + 3000 bits = 3060 bits.

## Test 2

(8 bits x 6 coefficients + 2 bits x 6 clipping indexes) x 1 chroma filter x 8 alternative chroma filters

+ (8 bits x 12 coefficients + 2 bits x 12 clipping indexes) x 7 luma filters x 1 alternative luma filter set

+ (8 bits x 12 coefficients + 2 bits x 12 clipping indexes) x 3 luma filters x 1 alternative luma filter set

= 480 bits + 840 bits + 360 bits = 1680 bits.

## Test 3

(8 bits x 6 coefficients + 2 bits x 6 clipping indexes) x 1 chroma filter x 8 alternative chroma filters

+ (8 bits x 12 coefficients + 2 bits x 12 clipping indexes) x 25 luma filters x 1 alternative luma filter set

+ (8 bits x 12 coefficients + 2 bits x 12 clipping indexes) x 3 luma filters x 1 alternative luma filter set

= 480 bits + 3000 bits + 360 bits = 3840 bits.

## CE5-4.3 (informative, as reference)

(8 bits x 6 coefficients + 2 bits x 6 clipping indexes) x 1 chroma filter x 8 alternative chroma filters

+ (8 bits x 12 coefficients + 2 bits x 12 clipping indexes) x 25 luma filters x 2 alternative luma filter sets

= 480 bits + 6000 bits = 6480 bits.

# Results

The proposed tests are implemented on top of VTM-5.0 [1] and the simulation are performed under common test conditions as described in JVET-N1010 [3]. Results of the CE5-4.3 test are also reported here, as reference for comparisons.

Results also includes a YUV column which is computed as (14*Y+U+V)/16, to provide an overall coding gains approximation.

## Test 1: disable filters derivation from static filter sets (informative)

## Test 2: limit to 7 and 3 filters the first and second alternative luma filter sets

## Test 3: limit to 3 filters the second alternative luma filter sets

## CE5-4.3 (informative, as reference)

# Conclusion

These tests show that even by limiting the amount of memory used for storing ALF parameters, CE5-4.3 still yields interesting coding gains.

Considering the coding gains illustrated by these experiments and reduced memory consumption, we recommend to adopt either Test 2 or Test 3 in the next version of the VVC draft.

# References

- Meeting Report of the 13th JVET Meeting (Marrakech, 9–18 January 2019), JVET-M1000, G. J. Sullivan, J.-R. Ohm

- Versatile Video Coding (Draft 5), Document JVET-N1001_v7, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations”, JVET-N1010

- Adaptive loop filter with a maximum number of luma filters per slice constraint, JVET-M0162, C.-Y. Chen, Z.-Y. Lin, C.-Y. Lai, Y.-W. Huang, S.-M. Lei

- CE5: Results of tests CE5-3.1, CE5-3.2, CE5-3.3 and CE5-3.4 on Non-Linear Adaptive Loop Filter, JVET-N0242, J. Taquet, P. Onno, C. Gisquet, G. Laroche

- Non-CE5: Complementary results of tests CE5-3 on Non-Linear ALF, JVET-N0243, J. Taquet, P. Onno, C. Gisquet, G. Laroche

- CE5: Coding tree block based adaptive loop filter (CE5-4), JVET-N0415, N. Hu, V. Seregin, H. E. Egilmez, M. Karczewicz

- CE5-4: Alternative luma filter sets and alternative chroma filter for ALF, JVET-O0090, J. Taquet, P. Onno, C. Gisquet, G. Laroche

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).