| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0429-v2 |
| --- | --- |

| Title: | Non-CE2: alternative solutions for chroma residual scaling factors derivation for dual tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Peng Yin,(Dolby) Edouard François Franck Hiron (InterDigital) | Email: | pyin@dolby.com tlu@dolby.com edouard.francois@interdigital.com |
| Source: | Dolby, InterDigital |  |  |

_____________________________

# Abstract

In this contribution, two options are proposed to enable chroma residual scaling (CRS) for separate luma/chroma tree (DualTree). In the current version of VTM, chroma residual scaling is disabled for separate luma/chroma tree to avoid introducing pipeline latency resulting from the dependence of CRS on luma. Each of the two proposed options eliminates that latency and enables CRS for DualTree. The two options can also apply on single/shared tree case (SingleTree) to provide a unified CRS approach for both DualTree and SingleTree. An encoder bug fix related to a rounding operation in CRS lambda calculation was found in the original LMCS encoder implementation and is fixed in the tests.

- In Option 1, the index of the CRS factor is explicitly coded/decoded per CTU, thus the chroma-to-luma dependency and pipeline latency are completely removed.

- In Option 2, the CRS factor is derived per VPDU from reconstructed luma samples from a neighboring VPDU, thus removing the pipeline latency.

For enabling CRS in DualTree (with SingleTree remaining the same as the current CRS design in VTM):

- For Option 1 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI: 		Y/U/V {0.05%, -0.17%, -0.31%} EncT:   99% DecT: 99%

  - RA: 	Y/U/V {0.05%, -0.71%, -0.78%} EncT:   101% DecT: 100%

  - LDB:	Y/U/V {0.03%, -0.47%, -0.86%} EncT:   99% DecT: 99%

  - HDR-PQ AI: 	DE/L100/wPSNRY {2.22%, -1.71%, -1.26%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.07%, -1.57%, -1.37%}

- For Option 2 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI:   	Y/U/V { 0.03%, -0.37%, -0.49%} EncT: 101% DecT: 101%

  - RA:   	Y/U/V {0.05%, -1.27%, -1.22%} EncT: 100% DecT: 100%

  - LDB: 	Y/U/V {0.03%, -0.26%, -0.72%} EncT:   99% DecT: 100%

  - HDR-PQ AI:  	DE/L100/wPSNRY {1.94%, -1.66%, -1.23%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.08%, -1.47%, -1.30%}

For unified CRS in both DualTree and SingleTree:

- For Option 1 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI: 		Y/U/V {0.03%, -0.23%, -0.33%} EncT:   99% DecT: 100%

  - RA: 	Y/U/V {0.01%, -0.53%, -0.54%} EncT:   100% DecT: 100%

  - LDB:

  - HDR-PQ AI: 	DE/L100/wPSNRY {2.22%, -1.71%, -1.26%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.01%, -1.49%, -1.30%}

- For Option 2 with the encoder bugfix, the BDRates relative to VTM 5.0 anchor are:

  - AI:   	Y/U/V {0.03%	-0.37%	-0.49%} EncT: 101% DecT: 101%

  - RA:   	Y/U/V {0.08%	-1.24%	-1.22%} EncT: 101% DecT: 100%

  - LDB: 	Y/U/V {0.01%	-0.46%	-0.62%} EncT: 100% DecT: 99%

  - HDR-PQ AI:  	DE/L100/wPSNRY {1.94%, -1.66%, -1.23%}

  - HDR-PQ RA: 	DE/L100/wPSNRY {2.14%, -1.48%, -1.30%}

# Introduction

Chroma residual scaling (CRS) is a component of LMCS. CRS scales chroma residual values by a factor derived from the average value of the corresponding luma prediction block. In VVC draft 5, the average value of the luma prediction block, avgY, is computed using all samples in the prediction block. The chroma scaling factor is then computed by finding the piece index of the LMCS piecewise linear model to which avgY belongs.

CE2-2 investigates chroma residual scaling luma-chroma dependency reduction for the dual-tree case. This contribution proposes two alternate solutions not considered in CE2. Option 1 explicitly signals the CRS index per CTU, thus completely eliminating the luma-chroma dependency and pipeline latency. Option 2 derives the CRS factor for each VPDU using reconstructed luma samples from a neighboring the VPDU, thus eliminating pipeline latency.

# Proposal Option 1: explicit coding of the CRS index

Option 1 explicitly signals the CRS index per CTU (or at finer granularity). At the encoder, the derivation of the index is performed by computing the average value of the CTU original luma samples. The index is then predicted from its two neighboring CTU indexes and coded in a manner similar to the coding of delta QP.

In VTM5, the chroma scaling factor table is built as follows. The slopes of the forward luma mapping function, noted fwdSlope[i], for i=0 to 15, are signalled in the bitstream. The chroma scaling factors are then derived as:

CRS_factor[i] = 64 * (1 << 11) / fwdSlope[i]

As the table fwdSlope may contain redundant values, a compacted chroma residual scaling table is built by removing redundant elements. This compacted table is used at the encoder and decoder side to identify the chroma scaling factor values from the coded/decoded indexes.

# Proposal Option 2: deriving the CRS factor per VPDU

Option 2 derives the chroma residual scaling factor from luma sample values from previously coded VPDU neighbors. As such, Option 2 removes pipeline latency. Option 2 retains the luma-chroma dependency but does not require explicit signalling.

Option 2 is illustrated in Figure 1 with an example of CTU size 128x128 and typical VPDU size 64x64. Each CTU has 4 VPDUs. Inside each VPDU, one chroma scaling factor CRS_factor is derived for use by all chroma TUs inside the current VPDU. The derivation process of CRS_factor is as follows:

- If the top VPDU neighbor is available, get the bottom row of the top VPDU luma reconstructed samples. If the left VPDU neighbor is available, get the rightmost column of the left VPDU luma reconstructed samples. (Note, the neighbor VPDUs may or may not be inside the same CTU but the treatment is the same.)

- Compute the average of the luma samples from step 1. To reduce the computation complexity, a subsampling method is proposed to use only a subset of the luma samples to calculate the average value. For example, if 4 or fewer samples out of 64 are selected from each side, then at most 8 samples would be used in for the chroma scale factor calculation for each VPDU thus making the computation complexity very low.

- The scale factor calculation is the same as the process defined in the current VVC draft specification clause 8.7.5.3. CRS_factor = ChromaScaleCoeff[ idxYInv ] and idxYInv is obtained using the average luma value computed from step 2 by invoking clause 8.8.2.3.

Figure 1. Proposed option2: use luma reconstructed samples in neighbor VPDUs to derive CRS scale factor

To further improve the hardware implementation friendliness of the average computation step (e.g. step 2), the number of samples used in the average computation is constrained to be a power of two, which would allow shift operations instead of division operations to be used.

As illustrated in Figure 2, special handling would be required for non-square VPDUs at picture boundary to use only a power-of-two subset of available samples. For the example shown in Figure 2, the top available neighbor has width TopNbDim and is not a power of two. The left available neighbor has height LeftNbDim and is not a power of two. In this example, LeftNbDim > TopNbDim. Thus, the number of samples to be used for average computation on each side would be computed as: NbSamplesOneDir = 1 << (floorLog2(min(TopNbDim, LeftNbDim)). When subsampling is used, subsampling is also applied within the NbSamplesOneDir samples to make sure the number of selected samples is a power of two.

Figure 2. special handling for non-square VPDUs to enable shift operations instead of division operations to compute average value

# Encoder bugfix related to CRS

An encoder bug related to the CRS was found in the original LMCS encoder implementation. A redundant rounding process was applied for chroma lambda adjustment in CRS. The errant rounding operation results in a loss of precision for floating point lambda values.

The chroma coding performance of VTM5.0 for the bug fix is reported in Table 1 below. (Note that AI coding results are not shown as they are identical to VTM5.0 because CRS is not enabled for dual tree coding in VTM.)

Table 1. Encoder bug fix impact on coding performance

|  |  | Random | access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over | vtm5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -1.04% | -0.34% | 100% | 100% |
| Class A2 | 0.01% | -0.11% | -0.15% | 100% | 100% |
| Class B | 0.01% | -0.31% | -0.34% | 100% | 100% |
| Class C | 0.00% | -0.38% | -0.63% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.44% | -0.38% | 100% | 100% |
| Class D | 0.01% | -0.27% | -0.28% | 100% | 100% |
| Class F | 0.04% | -0.10% | -0.14% | 100% | 100% |
|  |  | Low | delay B |  |  |
|  |  | Over | vtm5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.13% | -0.15% | 100% | 100% |
| Class C | 0.04% | -0.28% | -0.16% | 100% | 100% |
| Class E | -0.01% | 0.22% | -0.37% | 100% | 100% |
| Overall | 0.02% | 0.02% | -0.21% | 100% | 100% |
| Class D | -0.01% | -0.25% | 1.04% | 100% | 100% |
| Class F | 0.04% | -0.04% | -0.08% | 100% | 100% |

# Simulation results

The proposed modifications are implemented on top of VTM5.0. The following main configurations are tested:

- CRS activated in Dual Tree, use proposal in DualTree (intra-tile groups) but use VTM5 design in SingleTree (inter-tile groups)

- CRS activated in Dual Tree, use proposal in DualTree (intra-tile groups) and SingleTree (inter-tile groups)

Tests are reported for SDR and HDR PQ content.

## Option 1: explicit coding of the CRS index

SDR content

Two sub-tests are performed for SDR Option 1:

- SDR1.a: CRS index coded in DualTree

- SDR1.b: CRS index coded in DualTree & SingleTree

Note: for SDR1.b, in order to limit the overhead of CRS index coding in inter-slices, CRS is deactivated for slices of temporal Id larger than or equal to 2 (or pictures with Poc number not multiple of 16). Results with other settings are provided in attached excel files. One CRS index is coded per 2x2 CTUs.

Table 2. Results of SDR1.a: enable option 1 for DualTree

|  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.82% | 0.61% | 98% | 99% |
| Class A2 | -0.10% | 1.32% | 1.44% | 100% | 100% |
| Class B | 0.03% | 0.02% | -0.07% | 97% | 100% |
| Class C | 0.21% | -1.55% | -1.71% | 100% | 102% |
| Class E | 0.17% | -1.14% | -1.54% | 98% | 97% |
| Overall | 0.05% | -0.17% | -0.31% | 99% | 99% |
| Class D | 0.14% | -0.61% | -0.84% | 99% | 99% |
| Class F | 0.32% | -1.43% | -1.56% | 99% | 99% |
|  |  | Random | access |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.34% | -0.25% | 101% | 98% |
| Class A2 | 0.02% | 0.03% | -0.05% | 102% | 102% |
| Class B | 0.03% | -0.33% | -0.42% | 100% | 99% |
| Class C | 0.10% | -2.02% | -2.17% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.71% | -0.78% | 101% | 100% |
| Class D | 0.04% | -1.06% | -1.26% | 98% | 101% |
| Class F | 0.23% | -1.79% | -1.77% | 101% | 99% |
|  |  | Low | delay B |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.04% | -0.38% | 98% | 99% |
| Class C | 0.04% | -0.43% | -0.80% | 100% | 99% |
| Class E | 0.04% | -1.24% | -1.72% | 98% | 100% |
| Overall | 0.03% | -0.47% | -0.86% | 99% | 99% |
| Class D | 0.05% | -1.34% | 0.02% | 101% | 100% |
| Class F | 0.05% | -0.75% | -0.80% | 103% | 99% |

Table 3. Results of SDR1.b: enable option 1 for both DualTree and SingleTree

|  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.67% | 0.52% | 97% | 99% |
| Class A2 | -0.13% | 1.21% | 1.35% | 102% | 102% |
| Class B | 0.03% | -0.05% | -0.10% | 98% | 102% |
| Class C | 0.20% | -1.62% | -1.75% | 101% | 99% |
| Class E | 0.12% | -1.00% | -1.34% | 100% | 99% |
| Overall | 0.03% | -0.23% | -0.33% | 99% | 100% |
| Class D | 0.13% | -0.84% | -1.04% | 99% | 99% |
| Class F | 0.30% | -1.40% | -1.56% | 99% | 101% |
|  |  | Random | access |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.24% | 0.25% | 99% | 101% |
| Class A2 | -0.01% | 0.03% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.20% | -0.26% | 99% | 99% |
| Class C | 0.08% | -1.96% | -1.88% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.53% | -0.54% | 100% | 100% |
| Class D | 0.02% | -0.84% | -0.68% | 99% | 100% |
| Class F | 0.23% | -1.74% | -1.64% | 100% | 99% |
|  |  | Low | delay B |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.06% | 0.64% | 0.60% | 99% | 100% |
| Class E | -0.02% | -1.57% | -0.40% | 101% | 99% |
| Overall |  |  |  |  |  |
| Class D | -0.04% | 0.50% | 1.76% | 102% | 100% |
| Class F | -0.03% | -0.13% | -0.14% | 86% | 87% |

HDR PQ content

The HDR CTC anchor uses Luma-based QP adaptation (LPQ=1) but LMCS is off (LMCS=0).

In the performed tests for HDR, LMCS is activated and LQP is deactivated.

Three sub-tests are performed for HDR PQ Option1:

- HDR1.a: LQP 0, LMCS on / CRS not activated in DualTree

- HDR1.b: LQP 0, LMCS on / CRS idx coded in DualTree

- HDR1.c: LQP 0, LMCS on / CRS idx coded in DualTree & SingleTree for temp. layers <= 1

The first test (HDR1.a) does not use Option 1 but is included to show the performance of LMCS as is for HDR PQ content. The other tests use Option 1 either for DualTree case only (HDR1.b) or for both DualTree and SingleTree cases (HDR1.c).

Note: for HDR1.c, in order to limit the overhead of CRS index coding in inter-slices, CRS is deactivated for slices of temporal Id larger than or equal to 1 (or pictures with Poc number not multiple of 16). Results with other settings are provided in attached excel files. One CRS index is coded per CTU.

Table 4. Results of HDR1.a: LQP on/LMCS on versus LQP off/LMCS off

| HDR1.a | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 8.08% | -2.06% | -1.64% | 14.15% | 8.35% |
| RA | 6.26% | -1.79% | -1.59% | 14.12% | 6.85% |

Table 5. Results of HDR1.b: enable option 1 for DualTree

| HDR1.b | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 2.22% | -1.71% | -1.26% | 6.16% | -0.20% |
| RA | 2.07% | -1.57% | -1.37% | 7.39% | 0.43% |

Table 6. Results of HDR1.c: enable option 1 for DualTree & SingleTree

| HDR1.c | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 2.22% | -1.71% | -1.26% | 6.16% | -0.20% |
| RA | 2.01% | -1.49% | -1.30% | 6.69% | -0.58% |

## Option 2: deriving the CRS factor per VPDU

SDR content

Two sub-tests are performed for SDR Option 2:

- SDR2.a: CRS scale calculated using VPDU neighbor in DualTree

- SDR2.b: CRS scale calculated using VPDU neighbor in DualTree & SingleTree

Table 7. Results of SDR2.a: enable option 2 for DualTree

|  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 1.09% | 0.84% | 98% | 100% |
| Class A2 | -0.21% | 1.85% | 2.19% | 98% | 98% |
| Class B | 0.03% | -0.11% | -0.26% | 105% | 103% |
| Class C | 0.27% | -2.25% | -2.34% | 100% | 99% |
| Class E | 0.18% | -1.94% | -2.42% | 104% | 100% |
| Overall | 0.03% | -0.37% | -0.49% | 101% | 101% |
| Class D | 0.18% | -1.18% | -1.48% | 100% | 103% |
| Class F | 0.44% | -2.05% | -2.24% | 100% | 101% |
|  |  | Random | access |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -1.04% | -0.34% | 101% | 101% |
| Class A2 | 0.01% | -0.11% | -0.15% | 100% | 97% |
| Class B | 0.02% | -0.72% | -0.92% | 99% | 99% |
| Class C | 0.10% | -2.99% | -3.05% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -1.27% | -1.22% | 100% | 100% |
| Class D | 0.10% | -1.76% | -2.01% | 102% | 103% |
| Class F | 0.38% | -2.34% | -2.36% | 102% | 98% |
|  |  | Low | delay B |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.39% | -0.30% | 100% | 102% |
| Class C | 0.04% | -0.19% | -0.62% | 99% | 99% |
| Class E | 0.02% | -1.44% | -1.54% | 95% | 98% |
| Overall | 0.03% | -0.26% | -0.72% | 99% | 100% |
| Class D | 0.06% | -0.23% | 0.34% | 97% | 101% |
| Class F | 0.14% | -1.15% | -0.86% | 95% | 99% |

Table 8. Results of SDR2.b: enable option 2 for both DualTree and SingleTree

|  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 1.09% | 0.84% | 100% | 95% |
| Class A2 | -0.21% | 1.85% | 2.19% | 98% | 95% |
| Class B | 0.03% | -0.11% | -0.26% | 103% | 103% |
| Class C | 0.27% | -2.25% | -2.34% | 100% | 105% |
| Class E | 0.18% | -1.94% | -2.42% | 102% | 102% |
| Overall | 0.03% | -0.37% | -0.49% | 101% | 101% |
| Class D | 0.18% | -1.18% | -1.48% | 99% | 108% |
| Class F | 0.44% | -2.05% | -2.24% | 102% | 107% |
|  |  | Random | access |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -1.04% | -0.51% | 106% | 99% |
| Class A2 | 0.02% | -0.02% | -0.09% | 103% | 92% |
| Class B | 0.03% | -0.76% | -0.86% | 97% | 99% |
| Class C | 0.17% | -2.93% | -3.06% | 100% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | -1.24% | -1.22% | 101% | 100% |
| Class D | 0.12% | -1.87% | -1.96% | 103% | 101% |
| Class F | 0.36% | -2.46% | -2.37% | 100% | 103% |
|  |  | Low | delay B |  |  |
|  |  | Over | VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.13% | -0.13% | 98% | 99% |
| Class C | 0.06% | -0.40% | -0.47% | 103% | 99% |
| Class E | -0.04% | -1.53% | -1.65% | 99% | 98% |
| Overall | 0.01% | -0.46% | -0.62% | 100% | 99% |
| Class D | 0.08% | -0.58% | 0.34% | 102% | 94% |
| Class F | 0.09% | -0.94% | -0.78% | 100% | 99% |

HDR PQ content

Three sub-tests are performed for HDR PQ Option 2:

- HDR2.a: LQP 0, LMCS on / CRS not activated in DualTree: this is identical to test HDR1.a.

- HDR2.b: LQP 0, LMCS on / CRS scale calculated using VPDU neighbor in DualTree

- HDR2.c: LQP 0, LMCS on / CRS scale calculated using VPDU neighbor in DualTree & SingleTree

- Table 9. Results of HDR2.a: LQP on/LMCS on versus LQP off/LMCS off

| HDR2.a/HDR1.a | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 8.08% | -2.06% | -1.64% | 14.15% | 8.35% |
| RA | 6.26% | -1.79% | -1.59% | 14.12% | 6.85% |

- Table 10. Results of HDR2.b: enable option 1 for DualTree

| HDR2.b | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 1.94% | -1.66% | -1.23% | 5.77% | 1.42% |
| RA | 2.08% | -1.47% | -1.30% | 7.30% | 3.24% |

- Table 11. Results of HDR2.c: enable option 1 for DualTree & SingleTree

| HDR2.c | DE100 | PSNR-L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| AI | 1.94% | -1.66% | -1.23% | 5.77% | 1.42% |
| RA | 2.14% | -1.48% | -1.30% | 7.27% | 3.26% |

# Conclusion

In this contribution, two options are proposed to eliminate pipeline dependency for chroma residual scaling for dual tree. The Option 1 eliminates both pipeline latency and luma-chroma dependence by explicitly signalling CRS index values. The Option 2 eliminates pipeline latency by using reconstructed luma values from neighboring VPDUs to derive the chroma scaling factor value. Each option applies to both single and dual tree CRS. A non-normative encoder bug related to a rounding operation for lambda calculation is fixed and applied in experiments.

It is recommended that either Option 1 or Option 2 be adopted in VVC for both single and dual tree.

# Patent rights declaration

Dolby may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).