### Slice header syntax

#### General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if( sps_lmcs_enabled_flag ) { |  |
| slice_lmcs_enabled_flag | u(1) |
| if( slice_lmcs_enabled_flag ) { |  |
| slice_lmcs_aps_id | u(5) |
| slice_chroma_residual_scale_flag | u(1) |
| if( slice_chroma_residual_scale_flag ) |  |
| slice_chroma_residual_scale_grain_minus1 |  |
| } |  |
| if ( entropy_coding_sync_enabled_flag ) |  |
| num_entry_point_offsets | ue(v) |
| if( NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### Coding tree unit syntax

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| xCtb = ( CtbAddrInRs % PicWidthInCtbsY ) << CtbLog2SizeY |  |
| yCtb = ( CtbAddrInRs / PicWidthInCtbsY ) << CtbLog2SizeY |  |
| if( slice_sao_luma_flag ¦ ¦ slice_sao_chroma_flag ) |  |
| sao( xCtb >> CtbLog2SizeY, yCtb >> CtbLog2SizeY ) |  |
| if( slice_alf_enabled_flag ){ |  |
| alf_ctb_flag[ 0 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| if( alf_ctb_flag[ 0 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] ) { |  |
| if( slice_num_alf_aps_ids_luma > 0) |  |
| alf_ctb_use_first_aps_flag | ae(v) |
| if( !alf_ctb_use_first_aps_flag ) { |  |
| if( slice_num_alf_aps_ids_luma > 1 ) |  |
| alf_use_aps_flag | ae(v) |
| if( alf_use_aps_flag ) |  |
| alf_luma_prev_filter_idx_minus1 | ae(v) |
| else |  |
| alf_luma_fixed_filter_idx | ae(v) |
| } |  |
| } |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 1 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) |  |
| alf_ctb_flag[ 2 ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] | ae(v) |
| } |  |
| if( slice_chroma_residual_scale_flag ) { |  |
| if ( ( xCtb >> CtbLog2SizeY ) % ChResScalGrain = = 0 && |  |
| ( yCtb >> CtbLog2SizeY ) % ChResScalGrain = = 0 ) { |  |
| cu_crs_delta_abs | ae(v) |
| if( cu_crs_delta_abs ) |  |
| cu_crs_delta_sign_flag | ae(v) |
| } |  |
| } |  |
| if( slice_type = = I && qtbtt_dual_tree_intra_flag ) |  |
| dual_tree_implicit_qt_split ( xCtb, yCtb, CtbSizeY, 0 ) |  |
| else |  |
| coding_tree( xCtb, yCtb, CtbSizeY, CtbSizeY, 1, 0, 0, 0, 0, 0, SINGLE_TREE ) |  |
| } |  |

#### General slice header semantics

When present, the value of each of the slice header syntax elements slice_pic_parameter_set_id, slice_pic_order_cnt_lsb, and slice_temporal_mvp_enabled_flag shall be the same in all slice headers of a coded picture.

…

slice_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the current slice. slice_ chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is not enabled for the current slice. When slice_ chroma_residual_scale_flag is not present, it is inferred to be equal to 0.

slice_chroma_residual_scale_grain_minus1 specifies the granularity in CTU size for signalling the chroma residual scaling index. When _ slice_chroma_residual_scale_grain_minus1 is not present, it is inferred to be equal to 1.

When slice_chroma_residual_scale_grain_minus1 is not present, it is inferred to be equal to 0, and the variable ChResScalGrain is set equal to 1.

When slice_chroma_residual_scale_grain_minus1 is present, the variable CuCrsDeltaVal is derived as follows:

ChResScalGrain = slice_chroma_residual_scale_grain_minus1 + 1

num_entry_point_offsets is used to specify the variable NumEntryPoints, which specifies the number of entry points in the current slice as follows:

…

#### Luma mapping with chroma scaling data semantics

lmcs_min_bin_idx specifies the minimum bin index used in the luma mapping with chroma scaling construction process. The value of lmcs_min_bin_idx shall be in the range of 0 to 15, inclusive.

…

The variable ChromaScaleCoeff[ i ], with i = 0…15, is derived as follows:

- if ( lmcsCW[ i ] = = 0 ) ChromaScaleCoeff[ i ] is set equal to (1 << 11)

- Otherwise the following applies.

-

ChromaScaleCoeff [ 0 ] = InvScaleCoeff [ 0 ]
idx = 1
for( i = 0; i <= 15; i++ ) {
	remove = 0
	for (j = 0; j < idx; j++)
		if ( ChromaScaleCoeff [i] == InvScaleCoeff [j])
			remove = 1
	if ( remove = = 0 )
		ChromaScaleCoeff [idx++] = InvScaleCoeff [i] 
}

#### Coding tree unit semantics

The CTU is the root node of the coding tree structure.

The array IsInSmr[ x ][ y ] specifying whether the sample at ( x, y ) is located inside a shared merging candidate list region, is initialized as follows for x = 0..CtbSizeY − 1 and y = 0..CtbSizeY − 1:

IsInSmr[ x ][ y ] = FALSE		(7-125)

alf_ctb_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] equal to 1 specifies that the adaptive loop filter is applied to the coding tree block of the colour component indicated by cIdx of the coding tree unit at luma location ( xCtb, yCtb ). alf_ctb_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] equal to 0 specifies that the adaptive loop filter is not applied to the coding tree block of the colour component indicated by cIdx of the coding tree unit at luma location ( xCtb, yCtb ).

When alf_ctb_flag[ cIdx ][ xCtb >> Log2CtbSize ][ yCtb >> Log2CtbSize ] is not present, it is inferred to be equal to 0.

cu_crs_delta_abs specifies the absolute value of the difference CuCrsDeltaVal between the chroma residual scaling index of the current coding unit and its prediction.

cu_crs_delta_sign_flag specifies the sign of CuCrsDeltaVal as follows:

- If cu_crs_delta_sign_flag is equal to 0, the corresponding CuCrsDeltaVal has a positive value.

- Otherwise (cu_crs_delta_sign_flag is equal to 1), the corresponding CuCrsDeltaVal has a negative value.

When cu_crs_delta_sign_flag is not present, it is inferred to be equal to 0.

When cu_crs_delta_abs is present, the variable CuCrsDeltaVal is derived as follows:

CuCrsDeltaVal = cu_crs_delta_abs * ( 1 − 2 * cu_crs_delta_sign_flag )	(7-159)

…

#### Picture reconstruction with luma dependent chroma residual scaling process for chroma samples

Inputs to this process are:

- a location ( xCurr, yCurr ) of the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable nCurrSw specifying the transform block width,

- a variable nCurrSh specifying the transform block height,

- a variable tuCbfChroma specifying the coded block flag of the current chroma transform block,

- an (nCurrSw)x(nCurrSh) array predSamples specifying the chroma prediction samples of the current block,

- an (nCurrSw)x(nCurrSh) array resSamples specifying the chroma residual samples of the current block.

Output of this process is a reconstructed chroma picture sample array recSamples.The reconstructed chroma picture sample recSamples is derived as follows for i = 0..nCurrSw − 1, j = 0..nCurrSh − 1:

- If one of the following conditions is true, recSamples[ xCurr + i ][ yCurr + j ] is set equal to Clip1C( predSamples[ i ][ j ] + resSamples[ i ][ j ] ):

- slice_chroma_residual_scale_flag is equal to 0

- nCurrSw * nCurrSh is less than or euqal to 4

- tu_cbf_cb [ xCurr ][ yCurr ] is equal to 0 and tu_cbf_cr [ xCurr ][ yCurr ] is equal to 0

- Otherwise, the following applies:

- The variable varScale is derived by invoking the derivation process for chroma residual scaling factors as specified in clause 8.8.2.4.

k=0SubWidthC*nCurrSw-1l=0SubHeightC*nCurrSh-1PredMapSamples[ k ][ l ]
		-

-

- The recSamples is derived as follows:

- If tuCbfChroma is equal to 1, the following applies:

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepthC ), ( 1 << BitDepthC ) − 1, resSamples[ i ][ j ] )	(8-1013)

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C( predSamples[ i ][ j ] + 	(8-1014)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tu_cbf is equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1C(predSamples[ i ][ j ] )	(8-1015)

#### Derivation process for chroma residual scaling factors

Inputs to this process are:

- a location ( xCTU, yCTU ) specifying the location of the current CTU of the current picture, in CTU size.

In this process, the chroma residual scaling factor chrResScalFactor[ xCTU ][ yCTU ] of the current CTU is derived.

The predicted parameter crsf_pred is derived as follows:

–	If one or more of the following conditions are true, crsf_pred is set equal to 0:

–	The current CTU is the first CTU in a slice.

–	The current CTU is the first CTU in a brick.

–	Otherwise, crsf_pred is derived as follows.

–	nbPred is set to 0.

–	If ( xCTU, yCTU – 1 ) is inside the current slice and inside the current brick, the following applies

crsf_pred = chrResScalFactor[ xCTU ][ yCTU – 1 ]

nbPred = nbPred + 1

–	If ( xCTU – 1, yCTU ) is inside the current slice and inside the current brick, the following applies

crsf_pred = crsf_pred + chrResScalFactor[ xCTU – 1 ][ yCTU ]

nbPred = nbPred + 1

–	crsf_pred is set to equal to ( crsf_pred + ( nbPred >> 1 ) ) >> nbPred.

chrResScalFactor[ xCTU ][ yCTU ] is derived as follows:

–	If ( xCTU % ChResScalGrain ) = = 0 and  ( yCTU % ChResScalGrain ) = = 0, chrResScalFactor[ xCTU ][ yCTU ] is derived as follows:

–	chrResScalFactor[ xCTU ][ yCTU ] = crsf_pred + CuCrsDeltaVal

–	Otherwise, chrResScalFactor[ xCTU ][ yCTU ] is derived as follows:

–	xCTUref is set equal to ( ChResScalGrain * ( xCTU / ChResScalGrain)  )

–	yCTUref is set equal to ( ChResScalGrain * ( yCTU / ChResScalGrain)  )

–	chrResScalFactor[ xCTU ][ yCTU ] is set equal to chrResScalFactor[ xCTUref ][ yCTUref ]

| Table 9-5 – Association of ctxIdxOffset and syntax elements for each initializationType in the initialization process |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |
|  |  |  | 0 | 1 | 2 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | Table 9-6 | 0 | 9 | 18 |
|  | alf_ctb_use_first_aps_flag | Table 9-7 |  |  |  |
|  | alf_use_aps_flag | Table 9-8 |  |  |  |
|  | cu_crs_delta_abs | Table 9-11 |  |  |  |
| sao( ) | sao_merge_left_flag sao_merge_up_flag | Table 9-9 | 0 | 1 | 2 |
|  | … |  |  |  |  |

| Table 9-11 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| slice_data( ) | end_of_brick_one_bit | FL | cMax = 1 |
| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
|  | alf_ctb_use_first_aps_flag | FL | cMax = 1 |
|  | alf_use_aps_flag | FL | cMax = 1 |
|  | alf_luma_fixed_filter_idx | TB | cMax = 15 |
|  | alf_luma_prev_filter_idx_minus1 | TB | cMax = slice_num_alf_aps_ids_luma − 2 |
|  | cu_crs_delta_abs | 9.5.3.x |  |
|  | cu_crs_delta_sign_flag | FL | cMax = 1 |
| … |  |  |  |

| Table 9-17 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| end_of_brick_one_bit | terminate | na | na | na | na | na |
| alf_ctb_flag[ ][ ][ ] | 0..8 (clause 9.5.4.2.2) | na | na | na | na | na |
| alf_ctb_use_first_aps_flag | 0 | na | na | na | na | na |
| alf_use_aps_flag | 0 | na | na | na | na | na |
| alf_luma_fixed_filter_idx | bypass | bypass | bypass | bypass | bypass | bypass |
| alf_luma_prev_filter_idx_minus1 | bypass | bypass | bypass | bypass | bypass | bypass |
| cu_crs_delta_abs | 0 | 1 | 1 | 1 | 1 | bypass |
| cu_crs_delta_sign_flag | bypass | na | na | na | na | na |
| … |  |  |  |  |  |  |

#### 9.5.3.x	Binarization process for cu_crs_delta_abs

Input to this process is a request for a binarization for the syntax element cu_crs_delta_abs.

Output of this process is the binarization of the syntax element.

The binarization of the syntax element cu_crs_delta_abs is a concatenation of a prefix bin string and (when present) a suffix bin string.

For the derivation of the prefix bin string, the following applies:

- The prefix value of cu_crs_delta_abs, prefixVal, is derived as follows:

prefixVal = Min( cu_crs_delta_abs, 5 )		(9-20)

- The prefix bin string is specified by invoking the TR binarization process as specified in clause 9.5.3.3 for prefixVal with cMax = 5 and cRiceParam = 0.

When prefixVal is greater than 4, the suffix bin string is present and it is derived as follows:

- The suffix value of cu_crs_delta_abs, suffixVal, is derived as follows:

suffixVal = cu_crs_delta_abs − 5		(9-21)

- The suffix bin string is specified by invoking the k-th order EGk binarization process as specified in clause 9.5.3.5 for suffixVal with the Exp-Golomb order k set equal to 0.