| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0438 |
| --- | --- |

| Title: | Non-CE4: Cleanup for affine AMVR enabling flag in SPS level |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geonjung Ko, Dongcheol Kim, Jason Jung, Ju-Hyung (John) Son, Jin Sam Kwak | Tel: Email: | +82-31-712-0523 greg.ko@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

# Abstract

This contribution presents a syntax change to remove unnecessary signalling of sps_affine_amvr_enabled_flag when sps_affine_enabled_flag is equal to 0. When sps_affine_enabled_flag is equal to 0, the coding unit does not use affine motion compensation and the affine AMVR enabling flag does not have impact on the decoding process. Therefore, if affine motion compensation is not enabled by SPS level flag, it is proposed not to signal the affine AMVR enabling flag. It reportedly shows 0.00%/0.00%/0.00% (Y/U/V) average BD-rate change with RA and LDB configurations.

# Introduction

In the current VVC [1], there are affine-related enabling flags in SPS level, and such flags include sps_affine_amvr_enabled_flag and sps_affine_enabled_flag as shown in Table 1.

Table 1. Sequence parameter set RBSP syntax of VVC Draft 5

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| // … | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) |  |
| sps_affine_type_flag | u(1) |
| // … |  |
| } |  |

sps_affine_amvr_enabled_flag is used only when parsing amvr_flag in CU level and it is effective only if the CU uses affine motion compensation. Therefore, when affine motion compensation is not enabled in SPS level by sps_affine_enabled_flag, it is not needed to parse sps_affine_amvr_enabled_flag.

# Proposed specification text

(Changes are highlighted in yellow and red.)

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| // … | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| sps_dmvr_enabled_flag | u(1) |
| sps_mmvd_enabled_flag | u(1) |
| sps_isp_enabled_flag | u(1) |
| sps_mrl_enabled_flag | u(1) |
| sps_mip_enabled_flag | u(1) |
| sps_cclm_enabled_flag | u(1) |
| if( sps_cclm_enabled_flag && chroma_format_idc = = 1 ) |  |
| sps_cclm_colocated_chroma_flag | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_sbt_enabled_flag | u(1) |
| if( sps_sbt_enabled_flag ) |  |
| sps_sbt_max_size_64_flag | u(1) |
| sps_affine_enabled_flag | u(1) |
| if( sps_affine_enabled_flag ) { |  |
| sps_affine_type_flag | u(1) |
| sps_affine_amvr_enabled_flag | u(1) |
| } |  |
| // … |  |
| } |  |

7.4.3.3	Sequence parameter set RBSP semantics

sps_affine_amvr_enabled_flag equal to 1 specifies that adaptive motion vector difference resolution is used in motion vector coding of affine inter mode. sps_affine_amvr_enabled_flag equal to 0 specifies that adaptive motion vector difference resolution is not used in motion vector coding of affine inter mode. When not present, the value of sps_affine_amvr_enabled_flag is inferred to be equal to 0.

# Experimental results

The proposed method was implemented on top of VTM-5.0 and tested under the common test conditions [2]. Table 2 and Table 3 summarize experimental results for random access and low delay B configurations, respectively.

Table 2. Experimental results of the proposed method with RA configurations

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 3. Experimental results of the proposed method with LDB configurations

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

# Conclusion

This contribution proposes not to signal sps_affine_amvr_enabled_flag if affine motion compensation is disabled in SPS level. The proposed method has negligible impact on BD-rate and encoding/decoding time under the common test conditions. It is suggested to adopt the proposed method in the next version of VVC draft text and the reference software.

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-N1001, 14th meeting, Geneva, CH, 19–27 Mar. 2019.

[2] F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th meeting, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).