| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0447-v1 |
| --- | --- |

| Title: | Enabling tools for small blocks in small pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Fabrice Le Léannec, Franck Galpin, Tangi Poirier 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 philippe.bordes@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

In VVC, many tools have been disabled for lower block sizes, mainly to prevent the burden worst case of large pictures that could be fully encoded with 8x8 or 4x4 CUs for example. This has a non-desired consequence of degrading the relative coding performance of VVC for sequences with small picture sizes (class D or class C) in comparison with other state-of-art codecs (HEVC, AV1).

It is proposed some means to make the enabling of some coding tools depending not on CU size only but on picture size too.

# Introduction - Problem Statement

The VVC has been developed by putting more emphasis on large pictures (class-B and class-A). The required test sequences of CTCs contain a majority of large sequences (RA: 11/15, LDB: 8/12) whereas the small picture sizes represent a minority (class C) or are even optional (class D).

This has been motivated by the increasing capability of the end devices (TVs, smartphones…) and the growing of available bandwidth forecast (optical fiber, 5G…) for the end user. However, there may still remain some use cases that will require efficient video compression for lower picture resolution (e.g. video streaming with dynamic network adaptation).

In VVC, several coding tools are disabled based on minimum/maximum CU size conditions. The Table 1 lists some minimal CU size or number of samples thresholds for enabling one coding tool. The rationale of these limitations is to prevent the burden worst case of large pictures that could be fully encoded with 8x8 or 4x4 CUs. For example, 4K picture coded with 8x8 CUs would require the decoding of 129600 luma blocks per frame period.

Table 1: summary of tool enabling conditions based on CU size, for several VVC tools (VTM-5.0).

|  |  | Tool is disabled if CU size is below these limits |  |  |
| --- | --- | --- | --- | --- |
|  | Tool | num. luma samples | width | Height |
| 1 | Long-tap filters | <= 16 |  |  |
| 2 | Bi-prediction | <= 32 |  |  |
| 3 | Inter-prediction | <= 16 |  |  |
| 4 | DMVR | < 128 | 8 | 8 |
| 5 | TMVP | <= 32 |  |  |
| 6 | BDOF |  | 4 | 8 |
| 7 | Affine |  | <= 16 | <= 16 |
| 8 | Triangle | <= 64 |  |  |
| 9 | BPWA (GBI) | < 256 |  |  |

For small picture sizes, the validity of this cautious limitation is more questionable, since the number of maximum 8x8 CUs per picture is reduced dramatically (e.g. 1560 8x8 luma CUs for class D).

One consequence of these fixed threshold values is the relative reduced performance of the current VVC with small picture sizes compared to larger picture sizes, as depicted in Table 2. Other state-of-art video codecs (e.g. AV1) do not have such gap between large and small pictures coding performance [2].

Table 2: summary of VTM-5.0 and AV1 BD-rate performance compared to HEVC.

|  | VVC and AV1 vs HEVC (RA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM-5.0 vs HM-16.19 |  |  |  |  | AV1 vs HM-16.18 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -34.92% | -39.81% | -43.03% |  |  | -20.75% | -22.43% | -28.96% |  |  |
| Class A2 | -39.04% | -37.01% | -30.74% |  |  | -17.83% | -21.08% | -16.18% |  |  |
| Class B | -32.76% | -37.86% | -40.19% |  |  | -19.75% | -32.38% | -29.50% |  |  |
| A+B | -35.06% | -38.16% | -38.39% |  |  | -19.50% | -26.58% | -25.72% |  |  |
| Class C | -27.86% | -27.10% | -31.11% |  |  | -17.65% | -26.69% | -26.48% |  |  |
| Class D | -26.07% | -24.83% | -28.17% |  |  | -17.66% | -30.94% | -30.04% |  |  |
| C+D | -26.96% | -25.97% | -29.64% |  |  | -17.66% | -28.82% | -28.26% |  |  |

# Experiments

The performance of VTM-5.0 for classes C and D can be increased by modifying the tool activation thresholds for small pictures (Table 3) as depicted in Table 4.

Table 3: summary of proposed enabling conditions based on CU size, for small picture sizes and for several VVC tools (VTM-5.0).

|  |  | Tool is disabled if CU size is below these limits |  |  |
| --- | --- | --- | --- | --- |
|  | Tool | num. luma samples | width | Height |
| 1 | Long-tap filters |  |  |  |
| 2 | Bi-prediction | <= 16 |  |  |
| 3 | Inter-prediction |  |  |  |
| 4 | DMVR | <= 64 |  |  |
| 5 | TMVP |  |  |  |
| 6 | BDOF |  | 4 | 4 |
| 7 | Affine |  | <= 8 | <= 8 |
| 8 | Triangle | <= 32 |  |  |
| 9 | BPWA (GBI) | 128 (test-1) or 64 (test-2) |  |  |

Table 4: summary of BD-rate gains of proposed threshold modifications (VTM-5.0).

|  | Proposal difference with VTM-5.0 (test-1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.21% | -0.52% | -0.51% | 101% | 103% | -0.15% | -0.15% | -0.23% | 101% | 102% |
| Class C | -0.62% | -1.05% | -1.14% | 101% | 100% | -0.50% | -0.80% | -1.13% | 103% | 107% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -1.12% | -1.34% | -1.48% | 101% | 100% | -1.30% | -1.61% | -0.96% | 102% | 102% |
| Class F |  |  |  |  |  |  |  |  |  |  |

|  | Proposal difference with VTM-5.0 (test-2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.69% | -1.30% | -1.32% | 102% | 103% | -0.63% | -0.89% | -0.97% | 104% | 105% |
| Class E |  |  |  |  |  | -0.09% | 0.21% | -0.28% | 102% | 104% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -1.18% | -1.77% | -1.55% | 102% | 100% | -1.46% | -2.06% | -1.12% | 103% | 102% |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Proposal

One proposes several methods for selecting the CU size thresholds (Table 1 or Table 3) enabling the coding tools {…}={long-tap filters, bi-prediction, intr-prediction, DMVR, TMVP, BDOF, Affine, Triangle, BPWA}:

Method 1:

Encode a flag sps_tools_enabled_for small_blocks_flag in SPS:

sps_tools_enabled_for_small_blocks_flag equal to 1 specifies that the CU size conditions for enabling the coding tools {…} are given by Table 3. sps_tools_enabled_for_small_blocks_flag equal to 0 specifies that the CU size conditions for enabling the coding tools {…} are given by Table 1.

Method 2:

The Table 1 or Table 3 is selected as a function of the CTU size (1<<(log2_ctu_size_minus2+2)). If CTU size is inferior or equal to 64, then use Table 3, else use Table 1.

Method 3:

The Table 1 or Table 3 is selected as a function of the profile, tier and level combination.

# Conclusion

In VVC, many tools have been disabled for lower block sizes. This has a non-desired consequence of degrading the relative coding performance of VVC for sequences with small picture sizes (class D or class C).

It is proposed some means to make the enabling of some coding tools depending not on CU size only but on picture size too.

# References

- A.Segall,V.Baroncini, J.Boyce, J.Chen, T.Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC,” JVET-H1002, 8th Meeting: Macao, CN, 18–24 Oct.2017.

- Y.Chen, E.François, F.Galpin, R.Jullian, M.Kerdranvat, “Comparative study of video coding solutions VVC, AV1, EVC versus HEVC,” JVET-N0605, 14th Meeting: Geneva, CH, 19–27 March 2019.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).