| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0456 |
| --- | --- |

| Title: | Non-CE4: Grouping the syntax elements of motion vector differences |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang, Yan Zhang, Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | {yjchang, yzh, chunchic, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to reorganize the MVD syntax elements into two groups depending on whether an MVD syntax element is context coded or bypass coded aiming at improving the CABAC engine’s throughput. The grouped design is implemented on top of VTM 5.0. The simulation results under Common Test Conditions compared to VTM-5.0 shows no coding efficiency change while improving CABAC throughput.

# Introduction

In context-based adaptive binary arithmetic coding (CABAC), there are two coding modes. One is named as bypass mode, and the other one is named as context mode. In bypass mode, there is no context update process. Therefore, it can achieve higher data throughput than context mode by exploiting hardware or software level parallelism. Such benefit grows if a bunch of bypass bins can be processed together.

During the coding process of an Affine inter coded block, the related context is updated for as many as 50 times which are the most among all coding modes such that the worst case of CABAC throughput falls into Affine inter mode. The dramatical increase in context coded bins is caused by additional MVD information that is required by Affine inter mode in which up to 6 MVDs need to be signalled for 3 CPMVs in the case of 6-parameter affine model and B-Slice. These 6 MVDs signaling lead to 24 times of context update which is almost 50% of the total context updates in the Affine inter mode. Compared to the regular inter mode, the Affine inter mode takes more than 1.6 times of context updates. The number would go up to 8 times of context updates as compared to the IBC mode, that requires the least times of context updates among different inter modes. It is for this reason that Affine inter mode is of most critical to the throughput of CABAC engine.

The CABAC engine throughput would be even worse in Affine inter mode because the syntax elements of one MVD with context mode and bypass mode are sequentially processed, and thus all syntax elements with bypass mode cannot be processed together for six MVDs. More specifically, abs_mvd_greater0_flag and abs_mvd_greater1_flag are context coded while abs_mvd_minus2 and mvd_sign_flag are bypass coded as shown in Table 1 for one MVD. With this, due to the necessity of coding 6 MVDs at most for Affine inter mode, the CABAC engine has to switch between context mode and bypass mode several times which is detrimental to the throughput of the CABAC engine.

Table 1: Syntax table for MVD signaling for one MVD

| mvd_coding( x0, y0, refList ,cpIdx ) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ] ) |  |
| abs_mvd_minus2[ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ] ) |  |
| abs_mvd_minus2[ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ] | ae(v) |
| } |  |
| } |  |

# Technical Description

To improve the throughput of the CABAC engine, a grouping method can be applied to the context coded MVD syntax elements, abs_mvd_greater0_flag and abs_mvd_greater1_flag, to group all the MVDs of a given reference list in an Affine coded Coding Unit (CU) as shown in Table 2. A similar grouping method is also applied to bypass coded MVD syntax elements: abs_mvd_minus2 and mvd_sign_flag as shown in Table 3. With the grouping of the context coded MVD syntax elements as well as the grouping of the bypass coded MVD syntax elements, the MVD syntax elements coded in CABAC mode and bypass mode can be separately processed so that the CABAC engine will switch between CABAC mode and bypass mode less frequently and hence the CABAC throughput could be improved.

Table 2: Signaling of context coded MVD syntax elements in the grouping method

| mvd_coding_group_ctx( x0, y0, refList) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ][ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ][ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ][ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ][ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ][ 0 ] ) |  |
| abs_mvd_greater1_flag[ 1 ][ 0 ] | ae(v) |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ){ |  |
| abs_mvd_greater0_flag[ 0 ][ 1 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ][ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ][ 1 ] ) |  |
| abs_mvd_greater1_flag[ 0 ][ 1 ] | ae(v) |

| if( abs_mvd_greater0_flag[ 1 ][ 1 ] ) |  |
| --- | --- |
| abs_mvd_greater1_flag[ 1 ][ 1 ] | ae(v) |
| } |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 1 ){ |  |
| abs_mvd_greater0_flag[ 0 ][ 2 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ][ 2 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ][ 2 ] ) |  |
| abs_mvd_greater1_flag[ 0 ][ 2 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ][ 2 ] ) |  |
| abs_mvd_greater1_flag[ 1 ][ 2 ] | ae(v) |
| } |  |
| } |  |

Table 3: Signaling of bypass coded MVD syntax elements in grouping method

| mvd_coding_ group_bypass( x0, y0, refList) { | Descriptor |
| --- | --- |
| if( abs_mvd_greater0_flag[ 0 ][ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ][ 0 ] ) |  |
| abs_mvd_minus2[ 0 ][ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ][ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ][ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ][ 0 ] ) |  |
| abs_mvd_minus2[ 1 ][ 0 ] | ae(v) |
| mvd_sign_flag[ 1 ][ 0 ] | ae(v) |
| } |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) { |  |
| if( abs_mvd_greater0_flag[ 0 ][ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ][ 1 ] ) |  |
| abs_mvd_minus2[ 0 ][ 1 ] | ae(v) |
| mvd_sign_flag[ 0 ][ 1 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ][ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ][ 1 ] ) |  |
| abs_mvd_minus2[ 1 ][ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ][ 1 ] | ae(v) |
| } |  |
| } |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 1 ) { |  |
| if( abs_mvd_greater0_flag[ 0 ][ 2 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ][ 2 ] ) |  |
| abs_mvd_minus2[ 0 ][ 2 ] | ae(v) |
| mvd_sign_flag[ 0 ][ 2 ] | ae(v) |
| } |  |

| if( abs_mvd_greater0_flag[ 1 ][ 2 ] ) { |  |
| --- | --- |
| if( abs_mvd_greater1_flag[ 1 ][ 2 ] ) |  |
| abs_mvd_minus2[ 1 ][ 2 ] | ae(v) |
| mvd_sign_flag[ 1 ][ 2 ] | ae(v) |
| } |  |
| } |  |

# Experimental results

The proposed technique has been implemented on top of VTM-5.0. The experimental results under RA and LDB configurations following common test condition [1] are reported in Table 4.

Table 4: BD rate results under CTC

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 107% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 102% | 104% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 90% |
| Class E | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 91% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

In this contribution, a reorganization of MVD syntax structure is proposed: the context coded MVD syntax elements for multiple MVDs are grouped so as to the bypass coded MVD syntax elements. With this new organizing method for MVD syntax elements, it is claimed the worst case throughput of the CABAC engine can be improved.

# Specification text

#### 7.3.7.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { |  |
| --- | --- |
| …… |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| if( NumRefIdxActive[ 0 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l0[ x0 ][ y0 ] | ae(v) |
| mvd_coding_group_ctx( x0, y0, 0) |  |
| mvd_coding( x0, y0, 0, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 0, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 0, 2 ) |  |
| mvp_l0_flag[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| …… |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( NumRefIdxActive[ 1 ] > 1 && !sym_mvd_flag[ x0 ][ y0 ] ) |  |
| ref_idx_l1[ x0 ][ y0 ] | ae(v) |
| if( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) { |  |
| …… |  |
| } else { |  |
| if( sym_mvd_flag[ x0 ][ y0 ] ) { |  |
| …… |  |
| } else { |  |
| mvd_coding_group_ctx( x0, y0, 1) |  |
| mvd_coding( x0, y0, 1, 0 ) |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) |  |
| mvd_coding( x0, y0, 1, 1 ) |  |
| if(MotionModelIdc[ x0 ][ y0 ] > 1 ) |  |
| mvd_coding( x0, y0, 1, 2 ) |  |
| mvp_l1_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| } else { |  |
| …… |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L1 ) { |  |
| mvd_coding_group_bypass( x0, y0, 0) |  |
| } |  |
| if( inter_pred_idc[ x0 ][ y0 ] != PRED_L0 ) { |  |
| if( !( mvd_l1_zero_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI ) && !sym_mvd_flag[ x0 ][ y0 ]){ |  |
| mvd_coding_group_bypass( x0, y0, 1) |  |
| } |  |
| } |  |
| if( ( sps_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 0 && ( MvdL0[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL0[ x0 ][ y0 ][ 1 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 0 ] != 0 ¦ ¦ MvdL1[ x0 ][ y0 ][ 1 ] != 0 ) ) ¦ ¦ ( sps_affine_amvr_enabled_flag && inter_affine_flag[ x0 ][ y0 ] = = 1 && ( MvdCpL0[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 0 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 1 ] [ 1 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL0[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 0 ] != 0 ¦ ¦ MvdCpL1[ x0 ][ y0 ][ 2 ] [ 1 ] != 0 ) ) { |  |
| amvr_flag[ x0 ][ y0 ] | ae(v) |
| if( amvr_flag[ x0 ][ y0 ] ) |  |
| amvr_precision_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| …… |  |
| } |  |

#### 7.3.7.8.1 Motion vector difference syntax for CABAC mode

| mvd_coding_group_ctx( x0, y0, refList) { | Descriptor |
| --- | --- |
| abs_mvd_greater0_flag[ 0 ][ 0 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ][ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ][ 0 ] ) |  |
| abs_mvd_greater1_flag[ 0 ][ 0 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ][ 0 ] ) |  |
| abs_mvd_greater1_flag[ 1 ][ 0 ] | ae(v) |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ){ |  |
| abs_mvd_greater0_flag[ 0 ][ 1 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ][ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ][ 1 ] ) |  |
| abs_mvd_greater1_flag[ 0 ][ 1 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ][ 1 ] ) |  |
| abs_mvd_greater1_flag[ 1 ][ 1 ] | ae(v) |
| } |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 1 ){ |  |
| abs_mvd_greater0_flag[ 0 ][ 2 ] | ae(v) |
| abs_mvd_greater0_flag[ 1 ][ 2 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 0 ][ 2 ] ) |  |
| abs_mvd_greater1_flag[ 0 ][ 2 ] | ae(v) |
| if( abs_mvd_greater0_flag[ 1 ][ 2 ] ) |  |
| abs_mvd_greater1_flag[ 1 ][ 2 ] | ae(v) |
| } |  |
| } |  |

#### 7.3.7.8.2 Motion vector difference syntax for bypass mode

| mvd_coding_ group_bypass( x0, y0, refList) { | Descriptor |
| --- | --- |
| if( abs_mvd_greater0_flag[ 0 ][ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ][ 0 ] ) |  |
| abs_mvd_minus2[ 0 ][ 0 ] | ae(v) |
| mvd_sign_flag[ 0 ][ 0 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ][ 0 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ][ 0 ] ) |  |
| abs_mvd_minus2[ 1 ][ 0 ] | ae(v) |
| mvd_sign_flag[ 1 ][ 0 ] | ae(v) |
| } |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 0 ) { |  |
| if( abs_mvd_greater0_flag[ 0 ][ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ][ 1 ] ) |  |
| abs_mvd_minus2[ 0 ][ 1 ] | ae(v) |
| mvd_sign_flag[ 0 ][ 1 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ][ 1 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ][ 1 ] ) |  |
| abs_mvd_minus2[ 1 ][ 1 ] | ae(v) |
| mvd_sign_flag[ 1 ][ 1 ] | ae(v) |
| } |  |
| } |  |
| if( MotionModelIdc[ x0 ][ y0 ] > 1 ) { |  |
| if( abs_mvd_greater0_flag[ 0 ][ 2 ] ) { |  |
| if( abs_mvd_greater1_flag[ 0 ][ 2 ] ) |  |
| abs_mvd_minus2[ 0 ][ 2 ] | ae(v) |
| mvd_sign_flag[ 0 ][ 2 ] | ae(v) |
| } |  |
| if( abs_mvd_greater0_flag[ 1 ][ 2 ] ) { |  |
| if( abs_mvd_greater1_flag[ 1 ][ 2 ] ) |  |
| abs_mvd_minus2[ 1 ][ 2 ] | ae(v) |
| mvd_sign_flag[ 1 ][ 2 ] | ae(v) |
| } |  |
| } |  |

# References

[1]   F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).