| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0457-v2 |
| --- | --- |

| Title: | CE4-related: LIC flag inherited per component |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, F.Urban, F.Le Léannec, F.Galpin, T.Poirier 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 philippe.bordes@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

It is proposed to inherit the LIC-flag separately for each component. At the parsing stage, when LIC-flag is explicitly coded, the 3 LIC-flag-per-comp[c] are initialized to LIC-flag. After the LIC parameters have been derived, the value of LIC-flag-per-comp[c] are up-dated if LIC parameters are defaults.

It is reported the proposed method reduces the number of times the process for LIC parameters derivation is invoked by 13.7% , 21.8%, 20.7% (RA) and 13.9%, 23.4%, 22.7% (LDB) for Y,Cb,Cr components in average respectively, and the number of times the derived LIC parameters are defaults is reduced by 39.4% , 48.8%, 47.9% (RA) and 34.2%, 43%, 42.8% (LDB) for Y,Cb,Cr components in average respectively. The BD-rate gains and the encoder/decoder times are un-changed compared to CE4-3.2a.

# Introduction - Problem Statement

In CE4-3 [1], one single Local Illumination Compensation (LIC) flag is coded per CU with AMVP and Affine uni-directional modes. When LIC flag is true, then the LIC process applies for the 3 components Y, Cb, Cr. At the encoder, LIC may be selected by the RDO encoder even if the derived LIC parameters may be LIC-defaults (scale=1, offset=0) for one or more components.

In merge, the LIC flag may be inherited successively by several CUs (Figure 1), even if the derived LIC parameters are defaults. This may burden the encoder and decoder with un-necessary LIC process.

Figure 1: the LIC flag may be inherited successively by several CUs.

# Proposal

It is proposed to inherit the LIC-flag separately for each component. At the parsing stage, when LIC-flag is explicitly coded, the 3 LIC-flag-per-comp[c] are set to LIC-flag. For all CUs with LIC enabled, after the LIC parameters (LIC-param[c]) have been derived, the value of LIC-flag-per-comp[c] is up-dated as follows and stored in place of regular LIC-flag:

LIC-flag-per-comp[c] = ( LIC-param[c] == LIC-default ) ? false : true

When LIC flag is inherited (merge mode), the value LIC-flag-per-comp[c] is inherited for the component ‘c’.

The proposed method has been implemented on top of CE4-3.2a (LIC with min-max method)[1][2]. The BD-rate gains and the encoder/decoder times in RA and LDB configurations are un-changed compared to CE4-3.2a (Table 3). However, the number of times the process for LIC parameters derivation is invoked is reduced by 13.7% , 21.8%, 20.7% (RA) and 13.9%, 23.4%, 22.7% (LDB) for Y,Cb,Cr components in average respectively, and the number of times derived LIC parameters are defaults is reduced by 39.4% , 48.8%, 47.9% (RA) and 34.2%, 43%, 42.8% (LDB) for Y,Cb,Cr components in average respectively.

Table 1: number of blocks with LIC enabled.

|  | CE4-3.2a (RA) |  |  | CE4-3.2a (RA) + proposed LIC inherited per comp. |  |  | reduction of num blocks with LIC % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 1917273 | 1917273 | 1917273 | 1667662 | 1550767 | 1589495 | -13.0% | -19.1% | -17.1% |
| Class A2 | 1151451 | 1151451 | 1151451 | 1009788 | 901379 | 908438 | -12.3% | -21.7% | -21.1% |
| Class B | 1860501 | 1860501 | 1860501 | 1582395 | 1395705 | 1416488 | -14.9% | -25.0% | -23.9% |
| Class C | 597591 | 597591 | 597591 | 509785 | 472143 | 466741 | -14.7% | -21.0% | -21.9% |
| Overall | 5526816 | 5526816 | 5526816 | 4769630 | 4319994 | 4381162 | -13.7% | -21.8% | -20.7% |
| Class D | 148053 | 148053 | 148053 | 134312 | 128858 | 129046 | -9.3% | -13.0% | -12.8% |
| Class F | 639999 | 639999 | 639999 | 526405 | 485566 | 478758 | -17.7% | -24.1% | -25.2% |
|  | CE4-3.2a (LDB) |  |  | CE4-3.2a (LDB) + proposed LIC inherited per comp. |  |  | reduction of num blocks with LIC % |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 2910630 | 2910630 | 2910630 | 2475214 | 2161056 | 2192526 | -15.0% | -25.8% | -24.7% |
| Class C | 930597 | 930597 | 930597 | 831316 | 781958 | 777668 | -10.7% | -15.0% | -16.4% |
| Class E | 22326 | 22326 | 22326 | 18498 | 16164 | 15633 | -17.1% | -27.6% | -29.0% |
| Overall | 3863553 | 3863553 | 3863553 | 3325028 | 2959178 | 2985827 | -13.9% | -23.4% | -22.7% |
| Class D | 212151 | 212151 | 212151 | 193027 | 184670 | 185467 | -9.0% | -13.0% | -12.6% |
| Class F | 1055724 | 1055724 | 1055724 | 885613 | 817119 | 802175 | -16.1% | -22.6% | -24.0% |

Table 2: number of blocks with default LIC parameters.

|  | CE4-3.2a (RA) |  |  | CE4-3.2a (RA) + proposed LIC inherited per comp. |  |  | reduction of num blocks with LIC default % |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A1 | 206849 | 206849 | 206849 | 113499 | 100334 | 104000 | -45.1% | -51.5% | -49.7% |
| Class A2 | 157916 | 157916 | 157916 | 106182 | 84363 | 85565 | -32.8% | -46.6% | -45.8% |
| Class B | 271619 | 271619 | 271619 | 165726 | 137886 | 139837 | -38.0% | -49.2% | -48.5% |
| Class C | 77014 | 77014 | 77014 | 46568 | 42510 | 42004 | -39.5% | -44.8% | -45.5% |
| Overall | 713398 | 713398 | 713398 | 431975 | 365093 | 371406 | -39.4% | -48.8% | -47.9% |
| Class D | 14982 | 14982 | 14982 | 10421 | 9911 | 9904 | -30.4% | -33.8% | -33.9% |
| Class F | 99925 | 99925 | 99925 | 50231 | 44542 | 43856 | -49.7% | -55.4% | -56.1% |
|  | CE4-3.2a (LDB) |  |  | CE4-3.2a (LDB) + proposed LIC inherited per comp. |  |  | reduction of num blocks with LIC default % |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class B | 393257 | 393257 | 393257 | 252807 | 213863 | 215286 | -35.7% | -45.6% | -45.3% |
| Class C | 100274 | 100274 | 100274 | 72252 | 67557 | 67294 | -27.9% | -32.6% | -32.9% |
| Class E | 4526 | 4526 | 4526 | 2656 | 2260 | 2206 | -41.3% | -50.1% | -51.3% |
| Overall | 498057 | 498057 | 498057 | 327715 | 283680 | 284786 | -34.2% | -43.0% | -42.8% |
| Class D | 19634 | 19634 | 19634 | 15451 | 14780 | 14787 | -21.3% | -24.7% | -24.7% |
| Class F | 151807 | 151807 | 151807 | 74141 | 66224 | 65323 | -51.2% | -56.4% | -57.0% |

Table 3: summary of CE4-3.2a (LIC-with-Min-Max-method) (left), and proposed method (right).

|  | VVC CE4-3.2a (RA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE4-3.2a |  |  |  |  | CE4-3.2a + proposed LIC inherited per comp. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.58% | -0.55% | 123% | 99% | -0.59% | -0.58% | -0.49% | 123% | 99% |
| Class A2 | -0.35% | -0.24% | -0.43% | 112% | 100% | -0.35% | -0.34% | -0.37% | 112% | 100% |
| Class B | -0.43% | -0.30% | -0.52% | 116% | 101% | -0.42% | -0.27% | -0.48% | 115% | 101% |
| Class C | -0.31% | -0.39% | -0.41% | 118% | 101% | -0.32% | -0.34% | -0.44% | 118% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.41% | -0.37% | -0.48% | 117% | 100% | -0.41% | -0.36% | -0.45% | 117% | 100% |
| Class D | -0.15% | -0.17% | 0.01% | 114% | 102% | -0.12% | -0.10% | -0.02% | 111% | 97% |
| Class F |  | -0.52% |  | -0.34% |  | -0.44% |  | 114% |  | 101% |  | -0.53% |  | -0.34% |  | -0.38% |  | 115% |  | 102% |  |

|  | VVC CE4-3.2a (LDB) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE4-3.2a |  |  |  |  | CE4-3.2a + proposed LIC inherited per comp. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.41% | 0.04% | -0.29% | 126% | 100% | -0.41% | -0.01% | -0.46% | 125% | 99% |
| Class C | -0.34% | -0.43% | -0.63% | 127% | 103% | -0.33% | -0.13% | -0.46% | 127% | 103% |
| Class E | -0.09% | 0.04% | -0.25% | 101% | 100% | -0.07% | 0.11% | -0.04% | 101% | 102% |
| Overall | -0.31% | -0.12% | -0.40% | 119% | 101% | -0.30% | -0.02% | -0.36% | 119% | 101% |
| Class D | -0.11% | -0.08% | 0.33% | 118% | 96% | -0.14% | 0.15% | 0.22% | 117% | 97% |
| Class F | -1.56% | -1.74% | -1.81% | 119% | 102% | -1.47% | -1.52% | -1.65% | 119% | 104% |

# Conclusion

It is proposed to inherit the LIC-flag separately for each component. In RA configuration, it is reported the proposed method reduces the number of times the process for LIC parameters derivation is invoked by 13.7% , 21.8%, 20.7% (RA) and 13.9%, 23.4%, 22.7% (LDB) for Y,Cb,Cr components in average respectively, and the number of times the derived LIC parameters are defaults is reduced by 39.4% , 48.8%, 47.9% (RA) and 34.2%, 43%, 42.8% (LDB) for Y,Cb,Cr components in average respectively. The BD-rate gains and the encoder/decoder times are un-changed compared to CE4-3.2a.

# References

- H.Haito and C.Chen, “Description of Core Experiment 4 (CE4): Inter prediction,” JVET-N1024, 14th Meeting: Geneva, CH, 19–27 March 2019.

- A.Filippov, V.Rufitskiy, E.Alshina, “CE4-3.2a and CE4-3.2b: Unidirectional local illumination compensation with affine prediction and simplified linear model parameter derivation based on min / max method,” JVET-O0086, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).