| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0463_v1 |
| --- | --- |

| Title: | Non-CE8: Intra prediction mode of BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dongcheol Kim, Geonjung Ko, Ju-Hyung (John) Son, Jin Sam Kwak (WILUS), Jaehong Jung (Gaudio Lab) | Tel: Email: | +82-31-712-0523 dongcheol.Kim@wilusgroup.com |
| Source: | WILUS Inc., Gaudio Lab |  |  |

_____________________________

- Abstract

In this contribution, for the BDPCM block, it is proposed to set IntraPredModeY as either mode 18 or mode 50 depending on the block’s BDPCM prediction direction, which can be used to derive the MPM list in neighbor intra blocks. It is reportedly shown that the method provides BD-rate by -0.12%, 0.04%, -0.08% in Class F and -0.18%, -0.06%, -0.07% in TGM under AI configuration, and -0.09%, 0.08%, 0.10% in Class F and -0.18%, -0.02%, -0.13% in TGM under RA configuration. Therefore, it is recommended to adopt the proposed method.

Introduction

Block-based Delta Pulse Code Modulation (BDPCM) is adopted in VTM-5.0 [1].

If intra_bdpcm_flag is equal to 1 and intra_bdpcm_dir_flag is equal to 0, the BDPCM prediction direction is horizontal. If intra_bdpcm_flag is equal to 1 and intra_bdpcm_dir_flag is equal to 1, the BDPCM prediction direction is vertical.

In the current spec., if intra_bdpcm_flag is equal to 1, predModeIntra is derived as follows:

predModeIntra = intra_bdpcm_dir_flag ? 50: 18.

Then, if intra_bdpcm_flag is equal to 1, IntraPredModeY is set to INTRA_PLANAR mode which can be used to derive the MPM list for neighbor intra blocks.

However, the two BDPCM prediction directions are same as the horizontal and the vertical modes of the conventional intra prediction modes. Therefore, it would be beneficial to map IntraPredModeY to its corresponding BDPCM prediction modes instead of the INTRA_PLANAR mode.

Proposed Solutions

In this contribution, it is proposed that IntraPredModeY is remapped to mode 18 or mode 50 depending on the syntax element intra_bdpcm_dir_flag as follows.

IntraPredModeY = intra_bdpcm_dir_flag ? 50 : 18

Experimental Results

The simulations are performed according to common test conditions (CTC) as described in [3]. Class F & TGM in AI and Class F & TGM in RA results are reported.

Table 1: Results of proposed method over VTM5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | -0.12% | 0.04% | -0.08% |  | #NUM! |
| TGM | -0.18% | -0.06% | -0.07% |  | #NUM! |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | -0.09% | 0.08% | 0.10% |  | #NUM! |
| TGM | -0.18% | -0.02% | -0.13% |  | #NUM! |

Conclusion

In this contribution, it is proposed that IntraPredModeY is mapped to mode 18 or mode 50 depending on the syntax element intra_bdpcm_dir_flag. It is reportedly shown that the method provides BD-rate by -0.12%, 0.04%, -0.08% in Class F and -0.18%, -0.06%, -0.07% in TGM under All Intra (AI) configuration and -0.09%, 0.08%, 0.10% in Class F and -0.18%, -0.02%, -0.13% in TGM under Random Access (RA) configuration. Therefore, it is recommended to adopt the proposed method to VVC

References

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- “JVET common test conditions and software reference configurations”, JVET-N1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, 14th JVET meeting, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Gaudio Lab may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).