| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0466 |
| --- | --- |

| Title: | Non-CE6 : interaction between MTS and LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, F. Urban, K. Naser, T. Poirier 975 Avenue des Champs Blancs, 35510 Cesson-Sévigné | Tel: Email: | +33256856196 fabrice.leleannec@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to limit the usage of LFNST according to the primary transform used for a given block. Two options are proposed. First option disallows LFNST for a block with MTS index different from 0 (MTS_DCT2_DCT2). Second option forbids LFNST if MTS index is different from 0 and different from 2 (MTS_DST7_DST7). These 2 restrictions are added to already existing LFNST restrictions in VTM-5.0.

Optionally, the MTS-LFNST interaction option used is signaled in the SPS.

The first option reportedly brings -0.04% Luma BD-rate gain in RA and 0.07% Luma BD-rate loss in AI over VTM-5.0. The second option reportedly brings 0.00% Luma BD-rate in RA and 0.00% in AI.

It is suggested to adopt the first option in RA coding test conditions and the second option in AI.

# Introduction

In VTM-5.0, the usage of LFNST is allowed under the following conditions.

- The CU prediction mode is intra

- The transform skip mode flag is equal to 0

- ISP is not used

- MIP intra prediction mode is not used

- the considered CU is not a chroma CU in separated tree mode with width or height strictly lower than 4

- no non-zero parsed quantized coefficient exists with scanning position greater or equal to 8, in a square luma TB with size 4x4 or 8x8

- no non-zero parsed quantized coefficient exists with scanning position greater or equal to 8 or 16 (depending on the block size), in a luma TB with size 4x4 or at least 8x8

- no non-zero parsed quantized coefficient exists with scanning position greater or equal to 8 or 16 (depending on the block size), in a chroma TB with size 4x4 or at least 8x8

It has been observed that LFNST is very rarely used for MTS indices different from MTS_DCT2_DCT2 and MTS_DST7_DST7. Based on this observation, it is proposed to infer the LFNST index to zero, according to the value of the MTS index.

# Proposed restrictions on LFNST usage

Some restrictions, in addition to the already existing restrictions mentioned in previous section, are proposed to limit the use of LFNST.

Two options are proposed.

- Option 1: LFNST is inferred to 0 if the MTS index of the Luma TU contained in the considered CU is different from 0 (MTS_DCT2_DCT2).

- Option 2: LFNST is inferred to 0 if the MTS index of the Luma TU contained in the considered CU is different from 0 (MTS_DCT2_DCT2) and is different from 2 (MTS_DST7_DST7)

Additionally, the above inferred of LFNST index to 0 occurs if the tu_cbf_luma flag of the luma TU is non-zero. Indeed, in case the Luma residual is zero, LFNST may be used for chroma and cannot be inferred to 0 based on a non-available MTS index.

Optionally, the MTS-LFNST interaction option used is signaled in the SPS with 2 bits. The decoded mts_lfnst_interaction parsed value has the following meaning:

- 0: MTS-LFNST interaction is not used

- 1: MTS-LFNST interaction is used with first proposed option

- 3: MTS-LFNST interaction is used with second proposed option

# Modified decoding process

The coding_unit syntax table of JVET-N1001 (VVC draft 5) is modified according to the following Table 1. The sequence parameter set syntax table of JVET-N1001 is modified according to Table 2.

Table 1: proposed option 1 modification to the decoding process of the lfnst_idx syntax element.

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| ….. // INTRA preciction syntax |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| ….. // INTER preciction syntax |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 && ! ( tu_cbf_luma[ x0 ][ y0 ] && tu_mts_idx[ x0 ][ y0 ] !=0) ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Table 2: proposed modification to the decoding process of the lfnst_idx syntax element (with SPS modification)

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| ….. // INTRA preciction syntax |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| ….. // INTER preciction syntax |  |
| } |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] ) { |  |
| if( cbWidth <= MaxSbtSize && cbHeight <= MaxSbtSize ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ¦ ¦ allowSbtVerQ ¦ ¦ allowSbtHorQ ) |  |
| cu_sbt_flag | ae(v) |
| } |  |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| numSigCoeff = 0 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && numZeroOutSigCoeff = = 0 && ! ( tu_cbf_luma[ x0 ][ y0 ] && !( (mts_lfnst_interaction&1) && tu_mts_idx[ x0 ][ y0 ]==0 ) && !( (mts_lfnst_interaction&2) && tu_mts_idx[ x0 ][ y0 ]==2 ) ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Table 3:Proposed optional modification to the Sequence Parameter Set syntax table of JVET-N1001

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …. |  |
| sps_lfnst_enabled_flag | u(1) |
| if(sps_lfnst_enabled_flag ) |  |
| mts_lfnst_interaction | u(2) |
| …… |  |
| rbsp_trailing_bits( ) |  |
| } |  |

# Experimental Results

The performances of the proposed option 1 are shown by the following Table 3. The performances of the proposed option 2 are shown by the following Table 4.

Table 4: performance obtained with the proposed option 1 for MTS-LFNT interaction (without SPS modification)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.15% | 0.20% | 95% | 101% |
| Class A2 | 0.03% | 0.11% | 0.05% | 95% | 100% |
| Class B | 0.07% | 0.08% | 0.16% | 94% | 100% |
| Class C | 0.08% | -0.04% | 0.09% | 93% | 100% |
| Class E | 0.08% | 0.04% | -0.01% | 93% | 101% |
| Overall | 0.07% | 0.06% | 0.10% | 94% | 100% |
| Class D | 0.08% | 0.20% | -0.10% | 94% | 101% |
| Class F | 0.04% | 0.07% | 0.12% | 93% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.14% | 0.09% | 99% | 101% |
| Class A2 | -0.01% | 0.02% | -0.03% | 99% | 101% |
| Class B | -0.04% | -0.06% | -0.11% | 99% | 100% |
| Class C | -0.04% | -0.11% | 0.01% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.07% | -0.02% | 99% | 100% |
| Class D | 0.06% | -0.04% | 0.38% | 99% | 101% |
| Class F | 0.03% | -0.10% | -0.04% | 99% | 99% |

Table 5: performance obtained with the proposed option 1 for MTS-LFNT interaction (with SPS modification)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.15% | 0.20% | 93% | 100% |
| Class A2 | 0.03% | 0.11% | 0.05% | 96% | 100% |
| Class B | 0.07% | 0.08% | 0.16% | 94% | 103% |
| Class C | 0.09% | -0.03% | 0.10% | 95% | 101% |
| Class E | 0.08% | 0.04% | -0.01% | 94% | 100% |
| Overall | 0.07% | 0.06% | 0.11% | 94% | 101% |
| Class D | 0.08% | 0.20% | -0.10% | 95% | 100% |
| Class F | 0.04% | 0.07% | 0.12% | 96% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.14% | 0.09% | 102% | 106% |
| Class A2 | -0.01% | 0.02% | -0.03% | 103% | 106% |
| Class B | -0.04% | -0.06% | -0.11% | 99% | 109% |
| Class C | -0.04% | -0.11% | 0.01% | 98% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.07% | -0.02% | 100% | 106% |
| Class D | 0.07% | -0.04% | 0.38% | 97% | 101% |
| Class F | 0.03% | -0.10% | -0.03% | 98% | 103% |

Table 6: performance obtained with the proposed option 2 for MTS-LFNT interaction

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.01% | 101% | 103% |
| Class C | 0.00% | 0.01% | 0.00% | 100% | 102% |
| Class E | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Class D | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | -0.04% | 106% | 107% |
| Class A2 | 0.01% | 0.05% | 0.08% | 105% | 109% |
| Class B | 0.00% | 0.01% | -0.06% | 102% | 109% |
| Class C | -0.01% | 0.02% | 0.03% | 102% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 103% | 107% |
| Class D | 0.00% | 0.04% | 0.05% | 101% | 103% |
| Class F | 0.01% | 0.01% | 0.01% | 101% | 102% |

## Encoder-Only results

To assess the benefit of the proposed normative modification, Table 5 shows the results obtained if the restriction onto the LFNST usage are only set on the encoder side. The encoder-only version of option 1 consists in limiting the RD search for LFNST to cases where MTS index is equal to 0. However, the lfnst_idx syntax element is signaled and parsed by the decoder whatever the MTS index. As shown by Table 5, such approach provides 0.09% and 0.07% loss, respectively in AI and RA.

Table 7: option 1 results with encoder-only modification of the VTM-5.0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.05% | 0.18% | 94% | 101% |
| Class A2 | 0.04% | 0.06% | 0.06% | 95% | 101% |
| Class B | 0.09% | 0.06% | 0.12% | 94% | 103% |
| Class C | 0.12% | 0.07% | 0.11% | 95% | 103% |
| Class E | 0.13% | 0.09% | 0.15% | 95% | 101% |
| Overall | 0.09% | 0.07% | 0.12% | 95% | 102% |
| Class D | 0.16% | 0.24% | -0.05% | 96% | 102% |
| Class F | 0.07% | 0.09% | 0.05% | 97% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.18% | 0.08% | 104% | 108% |
| Class A2 | 0.05% | 0.06% | 0.05% | 103% | 106% |
| Class B | 0.08% | 0.05% | -0.13% | 99% | 109% |
| Class C | 0.11% | 0.03% | -0.29% | 99% | 104% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.00% | -0.09% | 101% | 107% |
| Class D | 0.14% | 0.04% | 0.32% | 98% | 100% |
| Class F | 0.06% | 0.03% | 0.02% | 99% | 102% |

# Conclusion

This contribution proposes to restrict the usage of LFNST, as a function of the MTS information attached to the Luma TU of a CU, where relevant.

- Option 1: LFNST is disallowed in case the MTS index of the luma TU is different from 0. Otherwise, same rules as in VTM-5.0 apply.

- Option 2: LFNST is disallowed in case the MTS index of the luma TU is different from 0 and different from 2. Otherwise, same rules as in VTM-5.0 apply.

It is reported that 0.00% luma BD-rate loss over VTM-5.0 is obtained in AI configuration with option 2, with no significant impact on coding and decoding time, despite the reduced RD search on the encoder side.

With option 1, -0.04% gain is obtained over VTM-5.0 in RA configuration, with negligible impact on overall coding and decoding time.

Optionally, an SPS-level mts-lfnst-interaction syntax element is signaled, which is used on the decoder to infer the non-usage of LNST, according to the two proposed options.

It is suggested to adopt the proposed mts-lfnst-interaction tool into future VTM-6.0, with option 2 activated in AI and option 1 activated in RA.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).