| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0472-v5 |
| --- | --- |

| Title: | “Non-CE6: Simplified LFNST signalling” |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jason Jung Dongcheol Kim Geonjung Ko Ju-Hyung (John) Son Jin Sam Kwak | Tel: Email: | +82-31-712-0523 jason.jung@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

In this contribution, a simplified LFNST (Low Frequency Non-Separable Transform) signalling method is proposed where one of dependencies on the number of significant coefficients is removed. In VTM-5.0, a CU-level LFNST index can be conditionally signalled when the number of significant coefficients in the CU exceeds a predefined threshold. Therefore, a counter is needed to count the number of significant coefficients in the whole CU region and it may increase the processing cost for residual coding. It is proposed to code LFNST index based on position of the last significant coefficient, instead of the number of significant coefficients. Experimental results show that compared to VTM-5.0, the proposed method can achieve 0.00% and -0.01% luma BD-rates for AI and RA configurations, respectively. As a second test, it is reported that compared to CE6-2.1a, which is the adoption in this meeting cycle, the proposed method on top of CE6-2.1a achieves 0.00% and -0.01% luma BD-rates for AI and RA configurations, respectively. It is asserted that the proposed method makes residual coding process much simpler than that of the current specification without coding efficiency loss on both VTM-5.0 and CE6-2.1a.

Introduction

LFNST is applied to intra-coded blocks where only the top-left coefficients are transformed using the intra mode dependent non-separable transform kernels. A CU-level LFNST index, lfnst_idx specifies whether LFNST is applied or not and which kernel is used if it is applied. Regarding signalling conditions for lfnst_idx, there are two dependencies of the number of significant coefficients. First, lfnst_idx can be signalled if the number of significant coefficients inside the CU (numSigCoeff) is greater than a threshold. Second, lfnst_idx can be signalled if the number of significant coefficients outside the LFNST top-left corner region (numZeroOutSigCoeff) is equal to 0. Therefore, two counters are needed in residual coding and they increase processing cost for residual coding.

Proposed method

In the proposed method, the condition of counting numSigCoeff is replaced with checking the position of the last significant coefficient. If all scan positions of the last significant coefficient (lastScanPos) within the CU are less than 1, lfnst_idx is not signalled and it can be inferred to 0. Otherwise, lfnst_idx can be signalled according to other conditions.

All coefficients in the transform block need to be considered for counting the number of significant coefficients, while the scan position of the last significant coefficient needs to be checked once per transform block. Therefore, the proposed method makes residual coding much simpler.

The proposed spec text changes are as follows.

7.3.7.5  Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| ⋯ |  |
| if( !pcm_flag[ x0 ][ y0 ] ) { |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA && general_merge_flag[ x0 ][ y0 ] = = 0 ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| ⋯ |  |
| numSigCoeff = 0 |  |
| lfnstLastScanPos = 1 |  |
| numZeroOutSigCoeff = 0 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !intra_mip_flag[ x0 ][ y0 ] ) { |  |
| if( ( numSigCoeff > ( ( treeType = = SINGLE_TREE ) ? 2 : 1 ) ) && lfnstLastScanPos = = 0 && numZeroOutSigCoeff = = 0 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

7.3.7.11  Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| ⋯ |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| ⋯ |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ] ) { |  |
| lfnstLastScanPos = lfnstLastScanPos && ( lastScanPos < 1 ) |  |
| } |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| ⋯ |  |
| if( ( i < lastSubBlock ) && ( i > 0 ) ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| ⋯ |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| ⋯ |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| ⋯ |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) { |  |
| numSigCoeff++ |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |
| numZeroOutSigCoeff++ |  |
| } // if( transform_skip_flag[ x0 ][ y0 ] |  |
| ⋯ |  |
| } // if( sig_coeff_flag[ x0 ][ y0 ] |  |
| ⋯ |  |
| } // 1st pass |  |
| ⋯ |  |
| } // Sub-block loop |  |
| } |  |

Experimental results

The proposed method is implemented on top of VTM-5.0 [1] and tested under the common test condition (CTC) [2]. Table 1 shows summarized experimental results of the proposed method compared to VTM-5.0 with AI and RA configurations. Detailed results are available in the attached excel sheet.

Table 1. Coding performance of the proposed method compared to VTM-5.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.08% | -0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.06% | -0.08% | 100% | 99% |
| Class B | 0.00% | -0.02% | 0.06% | 100% | 100% |
| Class C | 0.00% | -0.03% | 0.13% | 100% | 100% |
| Class E | 0.00% | 0.04% | 0.01% | 100% | 99% |
| Overall | 0.00% | 0.02% | 0.03% | 100% | 100% |
| Class D | -0.02% | 0.20% | 0.05% | 100% | 100% |
| Class F | -0.03% | 0.06% | -0.02% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | -0.04% | 99% | 100% |
| Class B | 0.02% | 0.08% | -0.10% | 100% | 100% |
| Class C | -0.07% | 0.02% | -0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | -0.06% | 100% | 100% |
| Class D | 0.00% | 0.37% | -0.08% | 100% | 100% |
| Class F | -0.01% | 0.04% | 0.10% | 100% | 100% |

As a second test, the proposed method is implemented on top of CE6-2.1a [3], which is the adopted version in this meeting, and tested under the CTC. Table 2 shows summarized experimental results of the proposed method compared to CE6-2.1a with AI and RA configurations. Detailed results are available in the attached excel sheet.

Table 2. Coding performance of the proposed method compared to CE6-2.1a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE6-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.09% | -0.03% | 100% | 99% |
| Class A2 | 0.00% | 0.04% | -0.01% | 100% | 99% |
| Class B | 0.00% | 0.03% | 0.02% | 100% | 100% |
| Class C | 0.00% | 0.08% | 0.01% | 101% | 100% |
| Class E | 0.01% | 0.07% | -0.07% | 100% | 99% |
| Overall | 0.00% | 0.06% | -0.01% | 100% | 100% |
| Class D | 0.01% | 0.07% | 0.05% | 100% | 100% |
| Class F | 0.01% | -0.01% | -0.09% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over CE6-2.1a |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.09% | 0.00% | 100% | 100% |
| Class A2 | 0.01% | 0.04% | 0.11% | 100% | 100% |
| Class B | 0.00% | 0.02% | 0.16% | 100% | 100% |
| Class C | -0.02% | 0.13% | 0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.09% | 100% | 100% |
| Class D | 0.00% | 0.01% | -0.06% | 100% | 100% |
| lass F | -0.07% | 0.07% | 0.21% | 100% | 100% |

Conclusion

This contribution proposes to replace LFNST signalling condition of counting the number of significant coefficients with the scan position of the last significant coefficient. It is reportedly shown that the proposed method does not degrade the coding performance and makes residual coding process much simpler. It is recommended to adopt the proposed method to next version of the reference software and text specification.

Reference

- VTM-5.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, CH, 19-27 Mar. 2019

- M. Siekmann, H.Schwarz and D. Marpe, “CE6-2.1: Simplification of Low Frequency Non-Separable Transform”, JVET-O0094, 15th JVET meeting, Gothenburg, SE, 3-12 July 2019

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).