| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0xxx-v1 |
| --- | --- |

| Title: | AHG11: SCC test sequences for VVC standardization and suggestion for class F update |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information, proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu | Tel: Email: | {xiaozhongxu, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution provides test results of a set of potential SCC test sequence candidates. In addition, suggestions for an update of sequences in class F are made.

Sequence description

The following sequences in YUV format are tested. The length of the sequence is 10 seconds.

| Sequence | Format | Frame rate | Number of frames |
| --- | --- | --- | --- |
| MissionControlClip2_2560x1440_60p_8b420.yuv | YUV 420/444 | 60 fps | 600 |
| MissionControlClip3_1920x1080_60p_8b420.yuv | YUV 420/444 | 60 fps | 600 |
| Basketball_Screen_2560x1440_60p_8b420.yuv | YUV 420/444 | 60 fps | 600 |
| Shark1_1920x1088_30p_8b420.yuv | YUV 420 | 30fps | 300 |
| GlassHalf_3840x2160_24p_10b420.yuv | YUV 420/444 | 24fps | 240 |
| Hero_2048x856_24p_10b420.yuv | YUV 420/444 | 24fps | 240 |

The screen shot of these sequences are shown below:

| MissionControlClip2 |  |
| --- | --- |
| MissionControlClip3 |  |
| Basketball_Screen |  |
| Shark1 |  |
| GlassHalf |  |
| Hero |  |

Note that the sources of these tested sequences are listed as follows:

| Sequence | Source |
| --- | --- |
| MissionControlClip2_2560x1440_60p_8b420.yuv | HEVC SCC test sequence |
| MissionControlClip3_1920x1080_60p_8b420.yuv | HEVC SCC test sequence |
| Basketball_Screen_2560x1440_60p_8b420.yuv | HEVC SCC test sequence |
| Shark1_1920x1088_30p_8b420.yuv | HEVC 3DV test sequences |
| GlassHalf_3840x2160_24p_10b420.yuv | Blender Foundation [1] |
| Hero_2048x856_24p_10b420.yuv | Blender Foundation [1] |

Objective Evaluation

RD performance of RA configuration for this test sequence is evaluated by VTM-5.0 [1]. The default configurations provided in the cfg files for class F in the software are used. 4 QPs 22-27-32-37 are tested.

For sequences in 420 format:

|  | QP | Bitrate (Kbps) | Y (dB) | U (dB) | V (dB) |
| --- | --- | --- | --- | --- | --- |
| Basketball_Screen | 22 | 4359.31 | 48.01 | 50.11 | 50.08 |
|  | 27 | 2501.20 | 44.75 | 46.71 | 46.74 |
|  | 32 | 1576.70 | 41.31 | 44.05 | 44.17 |
|  | 37 | 1008.00 | 37.68 | 41.96 | 42.12 |
| MissionControlClip2 | 22 | 4299.34 | 49.19 | 50.66 | 51.26 |
|  | 27 | 2429.87 | 45.74 | 47.48 | 48.04 |
|  | 32 | 1423.20 | 42.09 | 44.99 | 45.69 |
|  | 37 | 879.61 | 38.53 | 42.95 | 43.97 |
| MissionControlClip3 | 22 | 2445.48 | 49.32 | 49.90 | 50.37 |
|  | 27 | 1468.36 | 45.84 | 46.40 | 47.15 |
|  | 32 | 930.64 | 42.04 | 43.82 | 44.96 |
|  | 37 | 606.03 | 38.07 | 41.94 | 43.26 |
| Shark1 | 22 | 4927.45 | 44.14 | 48.73 | 48.25 |
|  | 27 | 2312.63 | 41.10 | 46.64 | 45.99 |
|  | 32 | 1163.50 | 38.24 | 45.08 | 44.18 |
|  | 37 | 575.30 | 35.48 | 43.82 | 42.60 |
| GlassHalf | 22 | 9264.8632 | 50.8858 | 50.6911 | 51.5475 |
|  | 27 | 5329.3496 | 46.86 | 47.4311 | 48.3983 |
|  | 32 | 3329.9896 | 43.1133 | 45.1421 | 46.1309 |
|  | 37 | 2069.5448 | 39.5667 | 42.7276 | 43.7856 |
| Hero | 22 | 1386.1304 | 50.8701 | 51.5628 | 52.595 |
|  | 27 | 722.5736 | 47.0798 | 48.7456 | 50.0406 |
|  | 32 | 410.3944 | 43.6687 | 46.4707 | 48.0223 |
|  | 37 | 234.5264 | 40.4144 | 44.4613 | 46.1803 |

For sequences in 444 format:

|  | QP | Bitrate (Kbps) | Y (dB) | U (dB) | V (dB) |
| --- | --- | --- | --- | --- | --- |
| Basketball_Screen | 22 | 5852.35 | 47.96 | 50.68 | 50.77 |
|  | 27 | 3222.89 | 44.71 | 47.27 | 47.20 |
|  | 32 | 1912.05 | 41.27 | 43.55 | 43.43 |
|  | 37 | 1147.99 | 37.65 | 40.14 | 40.07 |
| MissionControlClip2 | 22 | 6731.24 | 48.01 | 51.50 | 52.49 |
|  | 27 | 3567.80 | 44.57 | 48.68 | 49.48 |
|  | 32 | 1894.42 | 41.08 | 45.79 | 46.55 |
|  | 37 | 1034.49 | 37.90 | 43.11 | 43.93 |
| MissionControlClip3 | 22 | 2428.34 | 49.78 | 51.60 | 51.68 |
|  | 27 | 1515.88 | 46.37 | 47.61 | 47.78 |
|  | 32 | 988.29 | 42.53 | 43.52 | 43.97 |
|  | 37 | 640.77 | 38.40 | 39.96 | 40.41 |
| Shark1 | 22 |  |  |  |  |
|  | 27 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 37 |  |  |  |  |
| GlassHalf | 22 | 11120.54 | 50.97 | 53.44 | 54.29 |
|  | 27 | 6146.41 | 47.00 | 49.25 | 50.34 |
|  | 32 | 3645.09 | 43.24 | 45.86 | 47.10 |
|  | 37 | 2169.81 | 39.66 | 42.62 | 43.90 |
| Hero | 22 | 1585.08 | 50.91 | 53.70 | 54.46 |
|  | 27 | 804.29 | 47.13 | 50.69 | 51.78 |
|  | 32 | 439.04 | 43.71 | 47.86 | 49.27 |
|  | 37 | 11120.54 | 50.97 | 53.44 | 54.29 |

The RD curves are presented as follows:

Fig. 1 Rate-Distortion curves of proposed sequence (RA) in YUV420 format

Fig. 2 Rate-Distortion curves of proposed sequence (RA) in YUV444 format

Replace outdated Class F sequences

The current Class F sequences have been used by the community for a number of years, however they are now becoming outdated in that they neither have the resolutions, nor the frame rates current systems use.

In a previous JVET meeting [2], a new gaming sequence (AOV) was included in the test set in both 420 and 444 formats.

Sequences in the existing class F are listed as follows:

| Category | Resolution | Sequence name | Fps | No. of Frames |
| --- | --- | --- | --- | --- |
| Class F | 832x480 1920x1080 1280x720 1280x720 | BasketballDrillText ArenaOfValor SlideEditing SlideShow | 50 60 30 20 | 500 600 300 200 |

The other old sequences still need to be replaced. We propose to replace class F sequences with the following, both 4:2:0 and YUV 4:4:4 formats are available.

The proposed new class F will be as follows:

| Category | Resolution | Sequence name | bitdepth | Fps | No. of Frames |
| --- | --- | --- | --- | --- | --- |
| Class F | 1920x1080 1920x1080 1920x1080 1920x1080 2560x1440 3840x2160 | FlyingGraphics Desktop ChineseEditing ArenaOfValor MissionControlClip2 GlassHalf | 8 8 8 8 8 10 | 60 60 60 60 60 24 | 300 600 600 600 600 240 |

Note that all the sequences suggested have 444 version available.

Comments on the selected sequences:

| Sequences | Properties |
| --- | --- |
| FlyingGraphics | Moving graphics with various patterns |
| Desktop | English text editing, computer programs |
| ChineseEditing | Chinese text editing |
| ArenaOfValor | Gaming scene |
| MissionControlClip2 | Mixed contents |
| GlassHalf | Animation |

Conclusions

In this contribution, a few existing sequences with computer generated content are tested using VTM-5. RD curves of these sequences are shown. It is recommended to replace the old class F with some updated sequences in this contribution.

References

[1] F. Siddi, T. Roosendaal, “Blender Foundation/Animation Studio test sequences”, JVET-L0547, Macao, CN, Oct. 2018

[2] J. Ye, et al, “Tencent test sequences, and Class F test set restructure”, JVET-K0294, Ljubljana, SI, July 2018

Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.