| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0483-v3 |
| --- | --- |

| Title: | AHG11: SCC test sequences for VVC standardization and suggestion for class F update |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu (Tencent) Alexis Tourapis (Apple) Jizheng Xu (ByteDance) | Tel: Email: | {xiaozhongxu, xlxiangli, shanl} @tencent.com atourapis@apple.com xujizheng@bytedance.com |
| Source: | Tencent, Apple, ByteDance |  |  |

_____________________________

- Abstract

This contribution provides test results of a set of potential SCC test sequence candidates. In addition, suggestions for an update of sequences in class F are made.

Sequence description

The following sequences in YUV format are tested. The length of the sequence is 10 seconds.

| Sequence | Resolution | Format | Frame rate | BitDepth | Number of frames |
| --- | --- | --- | --- | --- | --- |
| MissionControlClip2 | 2560x1440 | YUV 420/444 | 60 fps | 8 | 600 |
| MissionControlClip3 | 1920x1080 | YUV 420/444 | 60 fps | 8 | 600 |
| Basketball_Screen | 2560x1440 | YUV 420/444 | 60 fps | 8 | 600 |
| Shark1 | 1920x1088 | YUV 420 | 30fps | 8 | 300 |
| GlassHalf | 3840x2160 | YUV 420/444 | 24fps | 10 | 240 |
| Hero | 2048x856 | YUV 420/444 | 24fps | 10 | 240 |

The screen shot of these sequences are shown below:

| MissionControlClip2 |  |
| --- | --- |
| MissionControlClip3 |  |
| Basketball_Screen |  |
| Shark1 |  |
| GlassHalf |  |
| Hero |  |

Note #1: the sources of these tested sequences are listed as follows:

| Sequence | Source |
| --- | --- |
| MissionControlClip2 | HEVC SCC test sequence |
| MissionControlClip3 | HEVC SCC test sequence |
| Basketball_Screen | HEVC SCC test sequence |
| Shark1 | HEVC 3DV test sequences |
| GlassHalf | Blender Foundation [1] |
| Hero | Blender Foundation [1] |

Note #2: For blender foundation sequences (GlassHalf and Hero), the 4:2:0 format was converted from its original 4:4:4 using ffmpeg software with the following command (assuming resolution is 1920x1080, input is input.yuv in 4:4:4 and output file is output.yuv in 4:2:0):

ffmpeg -s 1920x1080 -pix_fmt yuv444p -i input.yuv -pix_fmt yuv420p output.yuv

Note #3: For blender foundation sequences (GlassHalf and Hero), the values of contents are in video range (64 to 940 for luma, 64 to 960 for chroma).

Objective Evaluation

RD performance of RA configuration for this test sequence is evaluated by VTM-5.0 [1]. The default configurations provided in the cfg files for class F in the software are used. 4 QPs 22-27-32-37 are tested.

For sequences in 420 format:

|  | QP | Bitrate (Kbps) | Y (dB) | U (dB) | V (dB) |
| --- | --- | --- | --- | --- | --- |
| Basketball_Screen | 22 | 4359.31 | 48.01 | 50.11 | 50.08 |
|  | 27 | 2501.20 | 44.75 | 46.71 | 46.74 |
|  | 32 | 1576.70 | 41.31 | 44.05 | 44.17 |
|  | 37 | 1008.00 | 37.68 | 41.96 | 42.12 |
| MissionControlClip2 | 22 | 4299.34 | 49.19 | 50.66 | 51.26 |
|  | 27 | 2429.87 | 45.74 | 47.48 | 48.04 |
|  | 32 | 1423.20 | 42.09 | 44.99 | 45.69 |
|  | 37 | 879.61 | 38.53 | 42.95 | 43.97 |
| MissionControlClip3 | 22 | 2445.48 | 49.32 | 49.90 | 50.37 |
|  | 27 | 1468.36 | 45.84 | 46.40 | 47.15 |
|  | 32 | 930.64 | 42.04 | 43.82 | 44.96 |
|  | 37 | 606.03 | 38.07 | 41.94 | 43.26 |
| Shark1 | 22 | 4927.45 | 44.14 | 48.73 | 48.25 |
|  | 27 | 2312.63 | 41.10 | 46.64 | 45.99 |
|  | 32 | 1163.50 | 38.24 | 45.08 | 44.18 |
|  | 37 | 575.30 | 35.48 | 43.82 | 42.60 |
| GlassHalf | 22 | 9264.8632 | 50.8858 | 50.6911 | 51.5475 |
|  | 27 | 5329.3496 | 46.86 | 47.4311 | 48.3983 |
|  | 32 | 3329.9896 | 43.1133 | 45.1421 | 46.1309 |
|  | 37 | 2069.5448 | 39.5667 | 42.7276 | 43.7856 |
| Hero | 22 | 1386.1304 | 50.8701 | 51.5628 | 52.595 |
|  | 27 | 722.5736 | 47.0798 | 48.7456 | 50.0406 |
|  | 32 | 410.3944 | 43.6687 | 46.4707 | 48.0223 |
|  | 37 | 234.5264 | 40.4144 | 44.4613 | 46.1803 |

For sequences in 444 format:

|  | QP | Bitrate (Kbps) | Y (dB) | U (dB) | V (dB) |
| --- | --- | --- | --- | --- | --- |
| Basketball_Screen | 22 | 5852.35 | 47.96 | 50.68 | 50.77 |
|  | 27 | 3222.89 | 44.71 | 47.27 | 47.20 |
|  | 32 | 1912.05 | 41.27 | 43.55 | 43.43 |
|  | 37 | 1147.99 | 37.65 | 40.14 | 40.07 |
| MissionControlClip2 | 22 | 6731.24 | 48.01 | 51.50 | 52.49 |
|  | 27 | 3567.80 | 44.57 | 48.68 | 49.48 |
|  | 32 | 1894.42 | 41.08 | 45.79 | 46.55 |
|  | 37 | 1034.49 | 37.90 | 43.11 | 43.93 |
| MissionControlClip3 | 22 | 2428.34 | 49.78 | 51.60 | 51.68 |
|  | 27 | 1515.88 | 46.37 | 47.61 | 47.78 |
|  | 32 | 988.29 | 42.53 | 43.52 | 43.97 |
|  | 37 | 640.77 | 38.40 | 39.96 | 40.41 |
| Shark1 | 22 |  |  |  |  |
|  | 27 |  |  |  |  |
|  | 32 |  |  |  |  |
|  | 37 |  |  |  |  |
| GlassHalf | 22 | 11120.54 | 50.97 | 53.44 | 54.29 |
|  | 27 | 6146.41 | 47.00 | 49.25 | 50.34 |
|  | 32 | 3645.09 | 43.24 | 45.86 | 47.10 |
|  | 37 | 2169.81 | 39.66 | 42.62 | 43.90 |
| Hero | 22 | 1585.08 | 50.91 | 53.70 | 54.46 |
|  | 27 | 804.29 | 47.13 | 50.69 | 51.78 |
|  | 32 | 439.04 | 43.71 | 47.86 | 49.27 |
|  | 37 | 11120.54 | 50.97 | 53.44 | 54.29 |

The RD curves are presented as follows:

Fig. 1 Rate-Distortion curves of proposed sequence (RA) in YUV420 format

Fig. 2 Rate-Distortion curves of proposed sequence (RA) in YUV444 format

Replace outdated Class F sequences

The current Class F sequences have been used by the community for a number of years, however they are now becoming outdated in that they neither have the resolutions, nor the frame rates current systems use.

In a previous JVET meeting [2], a new gaming sequence (AOV) was included in the test set in both 420 and 444 formats.

Sequences in the existing class F are listed as follows:

| Category | Resolution | Sequence name | Fps | No. of Frames |
| --- | --- | --- | --- | --- |
| Class F | 832x480 1920x1080 1280x720 1280x720 | BasketballDrillText ArenaOfValor SlideEditing SlideShow | 50 60 30 20 | 500 600 300 200 |

The other old sequences still need to be replaced. We propose to replace class F sequences with the following, both 4:2:0 and YUV 4:4:4 formats are available.

The proposed new class F will be as follows:

| Category | Resolution | Sequence name | bitdepth | Fps | No. of Frames |
| --- | --- | --- | --- | --- | --- |
| Class F | 1920x1080 1920x1080 1920x1080 1920x1080 2560x1440 3840x2160 | FlyingGraphics Desktop ChineseEditing ArenaOfValor MissionControlClip2 GlassHalf | 8 8 8 8 8 10 | 60 60 60 60 60 24 | 300 600 600 600 600 240 |

Note that all the sequences suggested have 444 version available.

Comments on the selected sequences:

| Sequences | Properties |
| --- | --- |
| FlyingGraphics | Moving graphics with various patterns |
| Desktop | English text editing, computer programs |
| ChineseEditing | Chinese text editing |
| ArenaOfValor | Gaming scene |
| MissionControlClip2 | Mixed camera/computer contents |
| GlassHalf | Animation |

Conclusions

In this contribution, a few existing sequences with computer generated content are tested using VTM-5. RD curves of these sequences are shown. It is recommended to replace the old class F with some updated sequences in this contribution.

Acknowledgements

The authors would like to thank Blender Foundation for making the test sequences available.

References

[1] F. Siddi, T. Roosendaal, “Blender Foundation/Animation Studio test sequences”, JVET-L0547, Macao, CN, Oct. 2018

[2] J. Ye, et al, “Tencent test sequences, and Class F test set restructure”, JVET-K0294, Ljubljana, SI, July 2018

Copy rights declaration(s)

The authors of this contribution do not hold any copy rights of the materials tested. All the sequences are acquired from available resources that were provided for standardization testing purposes.