| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0488-v1 |
| --- | --- |

| Title: | Parameterizable cubemap for 360 degree video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal, Draft text |  |  |
| Author(s) or Contact(s): | Johannes Sauer Max Bläser | Tel: Email: | +49241-80-27678 sauer@ient.rwth-aachen.de |
| Source: | RWTH Aachen University |  |  |

_____________________________

# Abstract

This document describes a parameterizable cubemap, which is able to mimic the behaviour of other cubemap formats, e.g. CMP, EAC based on its configuration. The parameters of the cubemap are signaled with an SEI message. Using this, the specific mapping function of a cubemap format does no longer have to be hard coded, i.e. it is sufficient to implement this format instead of implementing CMP, EAC, ….

# Introduction

The basis of all cubemap formats is the conventional cubemap (CMP). CMP has the problem that the distribution of used samples on the sphere is not uniform. For this reason other formats have been proposed such as EAC or HEC. CMP can be transformed into another format by applying a function on the used sample position, which makes the distribution of used samples on the sphere more uniform. We will call such functions sample position adjustment functions (SPAF). 
For example the SPAF for EAC is [JVET-M1004-v1]:

x=tan(x'*π/4.0)

HEC uses the same mapping function for the u direction all faces, but changes the mapping function of the equatorial faces to [JVET-M1004-v1]:

v=v'1+0.4(1-u'2)(1-v'2)

It should be noted that two mapping functions are required, a forward and a matching backward mapping function. Given above were the respective forward mapping functions.

# Parameterizable cubemap

In order to obtain a parameterizable cubemap we perform a taylor approximation of the forward SPAF, i.e. we define the forward mapping function as:

fx= n=0∞anx-bn

It is sufficient to signal the chosen taylor approximation coefficients in order for the decoder/renderer to support the cubemap format.

The matching backward SPAF can be obtained by inverting the forward SPAF. This can be done numerically: the forward SPAF is sampled sufficiently dense, then the backward SPAF can be obtained piece-wise by interpolation. We used Barycentric Rational Interpolation to do so:
https://www.boost.org/doc/libs/1_65_0/libs/math/doc/html/math_toolkit/interpolate/barycentric.html

## Approximation of CMP

The sample position adjustment function for CMP is the identity function. The approximation is perfect. The evaluation point b=0 was used. The coefficients are: {0, 1}

## Approximation of EAC

A 9th order approximation of the EAC sample position adjustment function can be obtained with the coefficients of {0, 0.7853981633974483, 0, 0.16149102437656154, 0, 0.03984631312308352, 0, 0.00994872753755221, 0, 0.002486838364204076 }. The evaluation point b=0 was used.

# Signaling of parametrizable cubemap:

The signaling of the cubemap to sphere SPAF is sufficient, since the inverse function can be derived from it. A possible signaling for the required coefficients, as well as the layout of the packed cube faces is shown in Table 1. It should be noted that a more elaborate signaling could include different SPAF functions for the different axis of a face and could be face dependent.

## Unit face packing

In order for a decoder/render to correctly generate a viewport the relation of the different cube faces to each other has to be known. This can be done by mapping the packed cube faces to a unit cube layout, for which the relation of the cube faces is known. For example this unit cube could be [JVET-M1004-v1]:

Figure 4. Coordinates definition for CMP

Table 1. Face index of CMP

| Face index | Face label | Notes |
| --- | --- | --- |
| 0 | PX | Front face with positive X axis value |
| 1 | NX | Back face with negative X axis value |
| 2 | PY | Top face with positive Y axis value |
| 3 | NY | Bottom face with negative Y axis value |
| 4 | PZ | Right face with positive Z axis value |
| 5 | NZ | Left face with negative Z axis value |

It is sufficient to know the id of a packed cube face as well as its orientation and mirroring to map it to the unit cube.

## Syntax:

Table 2: Syntax for signaling of the cubemap packing format and the SPAF coefficients.

| cube_packing( ) { | Descriptor |
| --- | --- |
| 360_video_geometry_type | u(3) |
| if( 360_video_geometry_type == FORMAT_360_PARCMP) { |  |
| for( i = 0; i < num_cube_face_equal_to_6; i++ ) { |  |
| unit_face_idx[ i ] | u(3) |
| coded_face_width[ i ] | u(16) |
| coded_face_height[ i ] | u(16) |
| coded_face_top[ i ] | u(16) |
| coded_face_left[ i ] | u(16) |
| cube_face_transform_type[ i ] | u(3) |
| } |  |
| SPAF_num_coeffs | ue(v) |
| SPAF_coeffs_divisor | ue(v) |
| for( i = 0; i < SPAF_num_coeffs; i++ ) { |  |
| SPAF_coeffs_dividend[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

## Semantics

360_video_geometry_type[ i ] specifies the used cubemap format.

Note: only the syntax for the proposed parameterizable cubemap is shown (FORMAT_360_PARCMP)

unit_face_idx[ i ] specifies the index of the corresponding unit cube face.

coded_face_width[ i ], coded_face_height[ i ], coded_face_top[ i ], and coded_face_left[ i ] specify the width, height, the top luma sample row, and the left-most luma sample column, respectively, of the i-th packed cube face.

coded_face_width[ i ], coded_face_height[ i ], coded_face_top[ i ], and coded_face_left[ i ] are indicated in relative picture sample units. coded_face_width[ i ], coded_face_height[ i ], coded_face_top[ i ], and coded_face_left[ i ] shall represent integer horizontal and vertical coordinates of luma sample units within the cropped decoded pictures.

cube_face_transform_type[ i ] specifies the rotation and mirroring to be applied to the i-th packed cube face to remap to the i-th unit cube face. When cube_face_transform_type[ i ] specifies both rotation and mirroring, rotation applies before mirroring. The values of cube_face_transform_type[ i ] are specified in Table 2.

Table 2 – cube_face_transform_type[ i ] values

| Value | Description |
| --- | --- |
| 0 | no transform |
| 1 | mirroring horizontally |
| 2 | rotation by 180 degrees (anticlockwise) |
| 3 | rotation by 180 degrees (anticlockwise) before mirroring horizontally |
| 4 | rotation by 90 degrees (anticlockwise) before mirroring horizontally |
| 5 | rotation by 90 degrees (anticlockwise) |
| 6 | rotation by 270 degrees (anticlockwise) before mirroring horizontally |
| 7 | rotation by 270 degrees (anticlockwise) |

SPAF_num_coeffs specifies the number of used SPAF coefficients.

SPAF_coeffs_divisor specifies the divisor used for obtaining the SPAF coefficients.

SPAF_coeffs_divident[ i ] specifies the dividend for the i-th SPAF coefficient.

The variables SPAF_coefficient[ i ] are derived as follows:

for( i = 0; i < SPAF_num_coeffs; i++) {
		SPAF_coefficient[ i ] = SPAF_coeffs_divident[ i ] / SPAF_coeffs_divisor
	}

# Results

The implemented parameterizable cubemap only support a single SPAF, i.e. both components (u,v) use the same mapping function. It can mimic CMP or EAC but not HEC, which requires two mapping functions. The parameterizable cubemap was tested for mimicking CMP (Table 3) and EAC (Table 4). Signaling (SEI message) has not been implemented yet.

There are some losses for the approximation of EAC. EAC was approximated using a 9th order taylor series. There has not been time yet to investigate the source of the losses.

|  | End-to-end WS-PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class S1 | 0.00% | 0.00% | 0.00% | - | 100% |
| Class S2 | 0.00% | 0.00% | 0.00% | - | 100% |
| Overall | 0.00% | 0.00% | 0.00% | - | 100% |

Table 3: Approximation of CMP with parameterizable cubemap

|  | End-to-end WS-PSNR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class S1 | 0.39% | 0.19% | 0.32% | - | 99% |
| Class S2 | 0.37% | 0.01% | -0.09% | - | 99% |
| Overall | 0.38% | 0.12% | 0.15% | - | 99% |

Table 4: Approximation of EAC with parameterizable cubemap

# Conclusions

It is suggested to add the parameterizable cubemap to 360Lib for further study. We further propose to add a simple SEI message to VVC, which covers cubemap formats. The above syntax can be a basis for this.

# Notes

The current implementation approximates the forward mapping function and interpolates the backward mapping functions. Likely this should be changed. As it is the backward mapping function which is required for viewport generation, it is the backward mapping function which should be specified with the parameters.

Codec WS-PSNR has not been implemented yet. The Codec WS-PSNR results should be not considered.

# Patent rights declaration(s)

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.