| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0494 |
| --- | --- |

| Title: | AHG12: On filtering of independently coded region |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to changes to the filtering of independently coded regions in order to allow drift free filtering and is twofold:

The first aspect is to reduce the number of in-loop filter affected samples at boundaries of independently coded regions by disabling certain in-loop filters. Two options are given to reduce the filter reach, i.e. the in-loop filter affected samples, to either 3 or 2 samples.

The second aspect is to apply padding in order to replace in-loop filter affected samples at the boundaries of independent regions for inter-prediction.

# Introduction

For use cases such as Region-Of-Interest (ROI) coding, independently coded regions are an important tool. Using independent layers comes at disadvantages regarding coding efficiency or output synchronization. The more viable solution for this use case envisioned for VVC is picture sub-divisioning with boundary padded tiles/bricks. The related notes state: “The group was inclined to adopt [TileBoundaryPadding], but felt that VTM software should be available before action”, which was submitted to this meeting.

In HEVC, the related SEI message (Temporal Motion Constrained Tile Sets) contains an indication with respect to filtering across boundaries of independent regions. More specifically, the flag mc_exact_sample_value_match_flag indicates whether samples of an independent region are derived incorrectly when samples outside the independent region are not decoded or decoding context changes. In other words, when filtering is applied to mitigate boundary artefacts of independently coded regions, drift might occur in certain application scenarios, i.e. only decoding the ROI.

The current specification of VVC contains three in-loop filters: Deblocking, SAO and ALF. The following table specifies the number of luma samples at a filtered block boundary that are affected by sample values of the neighbouring block through the in-loop filter process if enabled across tiles/bricks:

| In-loop filter | Worst case filter reach in samples |
| --- | --- |
| DB – strong short filter | 3 |
| DB – weak short filter | 2 |
| DB – long filter | 7 |
| SAO | 1 |
| ALF | 3 |

For HEVC, the HM encoder implements the motion constraints for MCTS adaptively to the filter settings to enable drift free changing of decoding context. For instance, when filtering across tile boundaries is enabled, the affected samples are not used for inter-prediction. Such a restriction incurrs a coding efficiency penatly, it would be beneficial to restrict the in-loop filters to reduce the amount of affected samples.

For the alternative approach to encoder-side constraints in VVC, i.e. independent tile boundary padding, analog behavior to HEVC could be envisioned that would consist of also replacing the samples affected by filtering through padded samples to prevent drift. As for the MCTS approach, it would be beneficial to restrict the in-loop filter to reduce the amount of affected and therefore padded samples.

# Proposal

This contribution proposes changes to the coding of independently coded regions to allow boundary filtering without occurrence of drift. This proposal is two-fold:

- Restrict the available in-loop filters at independent tile boundaries to limit in-loop filter reach

- Padding of the in-loop filter affected samples on independent tile boundaries for inter-prediction

The proposal would allow benefitting from the coding efficiency gains of padding when compared to encoder-side constraints while still enable drift-free filtering.

## In-loop filter restrictions

It is proposed to restrict the in-loop filter at independently coded region boundaries to reduce the number of affected samples. Two options are proposed as follows:

Restrict in-loop filter reach to 3 samples:

Disable long deblocking filter on independently coded region boundaries.

Restrict in-loop filter reach to 2 samples:

Disable ALF, long deblocking filter and strong short deblocking filter on independently coded region boundaries.

Without the two above changes, in-loop filter reach remains to be the current maximum of 7 samples.

## Padding tile boundaries

The in-loop filter affected samples are illustrated in Figure 1 between two dashed lines around the boundary of two independently coded regions within a picture. In order to avoid drift when decoding context changes, padding sample values outside an independent tile when performing inter-prediction for said tile is not sufficient as described above.

Figure 1: Boundary between two independent regions.

Instead, it is proposed to carry out padding of sample values for inter-prediction on the boundary between the samples of the independently coded region that are affected by in-loop filters and those that are not affected by the in-loop filters as illustrated for Tile 0 in Figure 2.

Figure 2: Boundary padding of in-loop filter affected samples for inter-prediction of tile 0.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).