| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0497 |
| --- | --- |

| Title: | AHG17: On layer grouping for independent regions and ARC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Sühring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a mechanism grouping of layers to address two aspects:

- Layered Independent Regions jointly decodable as single layer

- Indication of layers that can be used for ARC or open GOP switching

The contribution proposes a layer group definition to be carried in the VPS with an indication of associated layers and where all or only one layer needs to be decoded.

# Introduction

During the 14th JVET Meeting in Geneva, two approaches for independent regions were discussed. The first one is similar to the principle in HEVC with MCTS, where tiles or group of tiles are encoded in such a way that they are independent of samples from other tiles or group of tiles. A second approach was considered that relay on layers, where each layer is independent.

It is asserted that although the second approach can work for certain use cases, it requires an output synchronization module on system side that many devices may not have. In fact, the layered approach is in principle the same as the multi-bitstream simulcast approach. Implementations in the scope of MPEG-OMAF have shown that synchronizing a large amount of output pictures of several bitstreams is challenging or even infeasible due to API limitation on many devices, which is not envisioned to change in the near future.

Still, using layers as a multiplexing mechanism for independent regions has its merits, such as an easy extraction procedure in the spirit of sub-bitstream extraction for HEVC layered extensions or a simpler conformance handling by using independent single-layer HRD models. Following the discussion in JVET-O0492 on bitstream merging and conformance, it could be envisioned that independent regions are offered as separate layers but are encoded in such a way that they can actually be decoded as a single bitstream under some conditions, e.g. when similar GOP structures are used, they are encoded as MCTS or tile boundary extension is used and so on.

# Proposal

This contribution proposes a grouping mechanism that indicates how many and which streams are grouped together into a so called Layer Group to be processed jointly by a decoder as a single layer, i.e. a decoder set by external means to decode a specific Layer Group encounters a NAL unit of said Layer Group, it decodes the NAL unit as if they had the same LayerId value.

Similarly, the Layer Group concept can be used for ARC/RPR. In a multi-layer approach, it may be the case that not all layers can be used for ARC/RPR, but that switching from one layer to another without an IDR is constrained so that it can only be done between identified layers in a Layer Group. Therefore, a grouping mechanism is required that indicates which group of layers have been encoded to allow for switching by using ARC/RPR and that only a single layer per access unit (e.g. the highest layer_id present) is to be decoded. Since in this case, again, the layers within a layer group are considered as if they belonged to the same layer, the marking process for another layer of the same Layer Group might be affected by another layer within the same Layer Group.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_5bits | u(5) |
| […] |  |
| sps_layer_group_enabled_flag | u(1) |
| if ( sps_layer_group_enabled_flag ) { |  |
| sps_num_layer_groups_minus1 | ue(v) |
| for( i = 0; i < sps_num_layer_groups_minus1 + 1; i++ ) { |  |
| sps_num_layers_in_layer_group_minus1[ i ] | ue(v) |
| sps_exclusive_layer_in_layer_group[ i ] | u(1) |
| for( j = 0; j < sps_num_layers_in_layer_group_minus1[ i ]+ 1; j++ ) |  |
| sps_layer_id_in_layer_group[ i ][ j ] | u(7) |
| } |  |
| } |  |
| […] |  |
| } |  |

sps_layer_group_enabled_flag equal to 1 specifies the presence of the syntax elements sps_num_layer_groups_minus1, sps_num_layers_in_layer_group_minus1[ i ], sps_exclusive_layer_in_layer_group[ i ] and sps_layer_id_in_layer_group[ i ][ j ].

sps_num_layer_groups_minus1 plus 1 specifies the number of Layer Groups within the bitstream. When not present, the value of sps_num_layer_groups_minus1 is inferred to be equal to -1.

sps_num_layers_in_layer_group_minus1[ i ] plus 1 specifies the number of layers that belong the i-th Layer Group.

sps_exclusive_layer_in_layer_group[ i ] equal to 1 specifies that only NAL units of one layer of the layers belonging to the i-th Layer Group are decoded per access unit when decoding the i-th Layer Group. sps_exclusive_layer_in_layer_group[ i ] equal to 0 specifies that all NAL units with LayerId values of all layers belonging to the i-th Layer Group are decoded per access unit when when decoding the i-th Layer Group.

sps_layer_id_in_layer_group[ i ][ j ] specifies the value of the layer_id of the j-th layer belonging to the i-th Layer Group.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).