| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0501_v2 |
| --- | --- |

| Title: | Non-CE4: Temporal merge modes simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Franck Galpin, Fabrice Le Léannec, Tangi Poirier, Antoine Robert | Tel: Email: | +33.2.56.85.62.76 franck.galpin@InterDigital.com fabrice.leleannec@InterDigital.com tangi.poirier@InterDigital.com antoine.robert@InterDigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution reports the result of simplifying the temporal merge modes construction by isolating the temporal predictors in a specific list. A second test also removes some temporal candidates to simplify further the decoding process. The results show a loss of 0.03% in RA and 0.22% in LDB for test 1, and 0.07% for RA and 0.33% for LDB for the second test.

# Introduction

In current VTM-5.0, the temporal candidates are spread over several merge lists (regular merge, mmvd, ciip, sub-block, triangle). It is proposed to group some temporal candidates into a specific list, reducing the worst-case decoder complexity and allowing a more predictable memory caching. It also allows to solve the consistency issue in the coding of the sub-block and affine flag as explained in JVET-O0500.

# Problem definition

The current VTM-5.0 merge modes contain the following temporal predictors:

- In regular merge mode, the TMVP candidate

- In MMVD mode, a potential candidate based on the TMVP candidate

- In CIIP merge mode, a potential candidate based on the TMVP candidate

- In subblock merge mode, the SbTMVP candidate

- In triangle mode, potential candidates based on the TMVP candidate

The construction of the merge lists, in the worst case, requires the caching of both the current CU neighbors and an area in the temporal reference buffer.

Moreover, the position of the SbTMVP candidate in the subblock list makes the use of the affine flag inconsistent between merge and AMVP modes.

# Proposed solutions

## Test 1

A first solution to limit the memory requirement of the temporal buffer and to solve the subblock/affine coding consistency is to put the SbTMVP candidate outside the subblock list. It is put in a “list” with only 1 element.

This way the decoder can consistently put in cache the temporal buffer when the SbTMVP is selected, and the subblock mode becomes the affine mode, consistently with the AMVP affine mode.

It slightly changes the coding of the sub-block candidates index:

- When the SbTMVP candidate exists and is selected, the coding is the same (1 bin equal to 1 to code the SbTMVP flag or 1 bin equal to 1 to indicate the use of the first candidate)

- When the SbTMVP candidate exists and is not selected, the coding is the same (1 bin equal to 0 to code the SbTMVP flag or 1 bin equal to 0 to indicate the use of further candidate)

- When the SbTMVP candidate does not exist, 1 bin indicates that the SbTMVP is not selected. Note that the condition on the SbTMVP existence are partially solved during the parsing (block size, existence of an inter reference frame).

In this case the syntax is changed as:

if( SbTMVP_possible && cbWidth >= 8 && cbHeight >= 8 )

merge_subblock_flag[ x0 ][ y0 ]

if( ! merge_subblock_flag[ x0 ][ y0 ] &&MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 )

merge_affine_flag[ x0 ][ y0 ]

The SbTMVP_possible condition depends of the SPS flag value of the SbTMVP mode and the type of the reference picture used.

The CABAC coding of the merge_affine_flag is left unchanged as it is now semantically consistent with the coding of the  inter_affine_flag.

## Test 2

In a second solution, the TMVP candidate of the regular merge is added to the proposed temporal candidate list after the SbTMVP candidate. Moreover, temporal candidates are removed from other merge lists, namely the TMVP candidate in the MVMD mode and the CIIP merge list.

# Experimental Results

The encoding/decoding time are not fully reliable for both tests. Especially, theoretical encoder is decreased since some merge candidates are moved from the lists.

## Test 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | -0.04% | 104% | 99% |
| Class A2 | 0.02% | -0.02% | -0.01% | 104% | 101% |
| Class B | 0.05% | 0.06% | -0.12% | 101% | 104% |
| Class C | 0.03% | 0.00% | -0.23% | 100% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | 0.00% | -0.11% | 102% | 99% |
| Class D | 0.02% | 0.06% | 0.06% | 99% | 91% |
| Class F | 0.04% | 0.10% | 0.05% | 102% | 110% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.21% | 0.12% | 0.14% | 103% | 90% |
| Class C | 0.18% | -0.05% | -0.24% | 101% | 99% |
| Class E | 0.30% | 0.18% | -0.03% | 103% | 125% |
| Overall | 0.22% | 0.08% | -0.03% | 102% | 101% |
| Class D | 0.12% | -0.20% | 0.60% | 97% | 89% |
| Class F | 0.17% | 0.14% | -0.29% | 101% | 119% |

## Test 2

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.10% | 0.03% | 106% | 99% |
| Class A2 | 0.01% | -0.03% | -0.06% | 106% | 98% |
| Class B | 0.04% | 0.05% | -0.07% | 103% | 101% |
| Class C | 0.12% | -0.06% | -0.07% | 103% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | -0.02% | -0.05% | 104% | 97% |
| Class D | 0.18% | 0.35% | 0.23% | 103% | 89% |
| Class F | 0.09% | 0.07% | 0.06% | 106% | 107% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.20% | 0.11% | -0.22% | 104% | 89% |
| Class C | 0.36% | 0.11% | -0.08% | 104% | 98% |
| Class E | 0.52% | 0.14% | 0.21% | 103% | 121% |
| Overall | 0.33% | 0.12% | -0.06% | 104% | 99% |
| Class D | 0.32% | -0.46% | 0.49% | 100% | 88% |
| Class F | 0.15% | 0.40% | 0.14% | 105% | 119% |

# Conclusion

This contribution proposes to reduce the memory bandwidth requirements during the merge candidate’s derivation at decoder side by putting some temporal candidates in their own list. It is proposed to further study the benefit of such approach for the merge process.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).