| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 Jul. 2019 | Document: JVET-O0502-v3 |
| --- | --- |

| Title: | Non-CE3: Proposed ISP cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Valeri George, Gayathri Venugopal, Jens Brandenburg, Benjamin Bross, Tung Nguyen, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution removes the MPM only and PDPC restrictions and applies always the cubic filter on CUs using ISP. Results show a gain of 0.10% and 0.06% for AI and RA respectively. Encoding and decoding run-times are unaffected. The removal of the mentioned restrictions simplifies the design and harmonizes it with other intra coding tools.

# Introduction

In VTM-5.0 the Intra Sub-Partitions (ISP) coding mode can only code MPMs, the PDPC filter is not applied on the sub-partitions and the decision between the Gaussian and the cubic filter is determined according to the sub-partition size and the intra mode. This contribution proposes the following changes to unify the intra mode coding and prediction stages between ISP and non-ISP blocks:

- Remove the MPM restriction so that ISP can code all modes. The signalling of the new available intra modes is exactly the same as with non-ISP blocks. This change requires modifying the encoder search, given that now ISP can code 67 intra modes instead of only 6. The removal of the MPM restriction was proposed during the last meeting in JVET-N0437: Non-CE3: On allowing non-MPM modes for ISP and non-zero reference line. A further study of the topic was recommended.

- Apply PDPC on the sub-partitions if their corresponding width and height is at least 4 samples long (and if all the other regular conditions of PDPC apply).

- Use always the cubic filter in the prediction stage of the sub-partitions (like MRL and BDPCM).

# Experimental results

According to the common test conditions [2], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with the VTM-5.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler. SCC TGM sequences have also been simulated.

Table I: Results for the AI configuration

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | -0.09% | 98% | 100% |
| Class A2 | -0.06% | -0.08% | -0.15% | 99% | 97% |
| Class B | -0.08% | -0.15% | -0.14% | 100% | 99% |
| Class C | -0.14% | -0.20% | -0.15% | 101% | 97% |
| Class E | -0.18% | -0.27% | -0.20% | 101% | 99% |
| Overall | -0.10% | -0.14% | -0.14% | 100% | 98% |
| Class D | -0.16% | -0.05% | -0.32% | 102% | 97% |
| Class F | -0.13% | -0.05% | -0.05% | 101% | 100% |
| Class TGM | -0.11% | -0.09% | -0.09% | 100% | 102% |

Table II: Results for the RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.24% | 0.02% | 100% | 101% |
| Class A2 | -0.02% | 0.02% | 0.05% | 100% | 99% |
| Class B | -0.05% | -0.11% | -0.15% | 100% | 100% |
| Class C | -0.12% | -0.46% | -0.35% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.20% | -0.13% | 100% | 100% |
| Class D | -0.05% | 0.02% | -0.28% | 100% | 100% |
| Class F | -0.14% | -0.30% | -0.07% | 100% | 99% |
| Class TGM | -0.05% | -0.04% | -0.08% | 99% | 100% |

# Draft changes

Changes are highlighted in yellow.

## MPM restriction removal

if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) )

intra_luma_ref_idx[ x0 ][ y0 ]

if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) )

intra_subpartitions_mode_flag[ x0 ][ y0 ]

if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY )

intra_subpartitions_split_flag[ x0 ][ y0 ]

if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 )

intra_luma_mpm_flag[ x0 ][ y0 ]

if( intra_luma_mpm_flag[ x0 ][ y0 ] ) {

if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 )

intra_luma_not_planar_flag[ x0 ][ y0 ]

if( intra_luma_not_planar_flag[ x0 ][ y0 ] )

intra_luma_mpm_idx[ x0 ][ y0 ]

} else

intra_luma_mpm_remainder[ x0 ][ y0 ]

## PDPC enabling

When all of the following conditions are true, the position-dependent prediction sample filtering process specified in clause 8.4.5.2.14 is invoked with the intra prediction mode predModeIntra, the transform block width nTbW, the transform block height nTbH, the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1, the reference sample width refW, the reference sample height refH, the reference samples p[ x ][ y ], with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and the colour component index cIdx as inputs, and the output is the modified predicted sample array predSamples:

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0

- nTbW is greater than or equal to 4 and nTbH is greater than or equal to 4 or cIdx is not equal to 0

- refIdx is equal to 0 or cIdx is not equal to 0

- BdpcmFlag[ xTbCmp ][ xTbCmp ] is equal to 0

- One of the following conditions is true:

- predModeIntra is equal to INTRA_PLANAR

- predModeIntra is equal to INTRA_DC

- predModeIntra is equal to INTRA_ANGULAR18

- predModeIntra is equal to INTRA_ANGULAR50

- predModeIntra is less than or equal to INTRA_ANGULAR10

- predModeIntra is greater than or equal to INTRA_ANGULAR58

## Interpolation filter selection

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0.

- refFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is greater than or equal to INTRA_ANGULAR34 and nTbW is less than or equal to 8

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and predModeIntra is less than INTRA_ANGULAR34 and nTbH is less than or equal to 8.

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 8-10.

- The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- minDistVerHor is greater than intraHorVerDistThres[ nTbS ] and refFilterFlag is equal to 0

- Otherwise, filterFlag is set equal to 0.

# Conclusion

It is observed that the proposed simplification brings coding gain benefit of 0.10% gain in AI and 0.06% gain in RA without increasing the encoding run-time and unifying the existing design.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).