#### 8.5.6.3 Fractional sample interpolation process

#### 8.5.6.3.1 General

Inputs to this process are:

- a luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture,

- a variable sbWidth specifying the width of the current coding subblock,

- a variable sbHeight specifying the height of the current coding subblock,

- a motion vector offset mvOffset,

- a refined motion vector refMvLX,

- the selected reference picture sample array refPicLX,

- the bidirectional optical flow flag bdofFlag,

- a variable cIdx specifying the colour component index of the current block.

Outputs of this process are:

- an (sbWidth +bdofOffset)x(sbHeight +bdofOffset) array predSamplesLX of prediction sample values.

The bidirectional optical flow boundary offset bdofOffset is derived as follows:

bdofOffset = bdofFlag  ?  2  :  0		(8-720)

- If cIdx is equal to 0, the following applies:

  - Let ( xIntL, yIntL ) be a luma location given in full-sample units and ( xFracL, yFracL ) be an offset given in 1/16-sample units. These variables are used only in this clause for specifying fractional-sample locations inside the reference sample arrays refPicLX.

  - For each luma sample location ( xL = 0..sbWidth − 1 +bdofOffset, yL = 0..sbHeight − 1 +bdofOffset ) inside the prediction luma sample array predSamplesLX, the corresponding prediction luma sample value predSamplesLX[ xL ][ yL ] is derived as follows:

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = xSb + ( refMvLX[ 0 ]  >>  4 ) + xL		(8-721)

yIntL = ySb + ( refMvLX[ 1 ]  >>  4 ) + yL		(8-722)

xFracL = refMvLX[ 0 ] & 15		(8-723)

yFracL = refMvLX[ 1 ] & 15		(8-724)

  - If bdofFlag is equal to TRUE and one or more of the following conditions are true, the prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma integer sample fetching process as specified in clause 8.5.6.3.3 with ( xIntL, yIntL ), ( xFracL, yFracL ) and refPicLX as inputs:

    - xL is equal to 0.

    - xL is equal to sbWidth + 1.

    - yL is equal to 0.

    - yL is equal to sbHeight + 1.

  - Otherwise, the following applies:

    - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

    - The list padVal[ dir ] is derived as follows for dir = 0..1:

      - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xL : yL)	(8-725)

      - If disp is less than 0, padVal[ dir ] is set equal to disp.

      - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth : sbHeight ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth : sbHeight ) − 1).

      - Otherwise, padVal[ dir ] is set equal to 0.

    - The prediction luma sample value predSamplesLX[ xL ][ yL ] is derived by invoking the luma sample 8-tap interpolation filtering process as specified in clause 8.5.6.3.2 with ( xIntL − bdofFlag, yIntL − bdofFlag), ( xFracL, yFracL ), refPicLX, sbWidth, sbHeight, ( xSb, ySb ) and padVal as inputs.

- Otherwise (cIdx is not equal to 0), the following applies:

  - Let ( xIntC, yIntC ) be a chroma location given in full-sample units and ( xFracC, yFracC ) be an offset given in 1/32 sample units. These variables are used only in this clause for specifying general fractional-sample locations inside the reference sample arrays refPicLX.

  - For each chroma sample location ( xC = 0..sbWidth − 1, yC = 0.. sbHeight − 1 ) inside the prediction chroma sample arrays predSamplesLX, the corresponding prediction chroma sample value predSamplesLX[ xC ][ yC ] is derived as follows:

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows: [Ed. (SL): Shall we make it consistent: /2 or /SubWidthC and /SubHeightC?]

xIntC = ( xSb / SubWidthC ) + ( mvLX[ 0 ]  >>  5 ) + xC	(8-726)

yIntC = ( ySb / SubHeightC ) + ( mvLX[ 1 ]  >>  5 ) + yC	(8-727)

xFracC = mvLX[ 0 ] & 31		(8-728)

yFracC = mvLX[ 1 ] & 31		(8-729)

  - The motion vector mvLX is set equal to ( refMvLX − mvOffset ).

  - The list padVal[ dir ] is derived as follows for dir = 0..1:

    - The variable disp is derived as follows:

disp = ( refMvLX[ dir ]  >>  4) − ( mvLX[ dir ]  >>  4) + ( dir = = 0 ? xC : yC)	(8-730)

    - If disp is less than 0, padVal[ dir ] is set equal to disp.

    - Otherwise, if disp is greater than ( dir  = = 0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1, padVal[ dir ] is set equal to disp − ( ( dir  = =  0  ?  sbWidth / SubWidthC : sbHeight / SubWidthC ) − 1).

    - Otherwise, padVal[ dir ] is set equal to 0.

  - The prediction sample value predSamplesLX[ xC ][ yC ] is derived by invoking the process specified in clause 8.5.6.3.4 with ( xIntC, yIntC ), ( xFracC, yFracC ), refPicLX, and padVal as inputs.

##### 8.5.6.3.2 Luma sample interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL,

- a variable sbWidth specifying the width of the current subblock,

- a variable sbHeight specifying the height of the current subblock,

- a luma location ( xSb, ySb ) specifying the top-left sample of the current subblock relative to the top-left luma sample of the current picture,

–	a list padVal[ dir ] with dir = 0,1 specifying reference sample padding direction and amount.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-13.

The luma interpolation filter coefficients fPadL[ 0 ] are set equal to fL[ xFracL ] and fPadL[ 1 ] are set equal to fL[ yFracL ] and modified depending on padVal[ dir ] as follows for dir being equal to 0 and 1:

–	If padVal[ dir ] is equal to − 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 2 ] = fPadL[ dir ][ 2 ] + fPadL[ dir ][ 1 ] + fPadL[ dir ][ 0 ]	(8-731)

fPadL[ dir ][ 0 ] = 0		(8-732)

fPadL[ dir ][ 1 ] = 0		(8-733)

–	Otherwise, if padVal[ dir ] is equal to − 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 1 ] = fPadL[ dir ][ 1 ] + fPadL[ dir ][ 0 ]		(8-734)

fPadL[ dir ][ 0 ] = 0		(8-735)

–	Otherwise, if padVal[ dir ] is equal to 1, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 6 ] = fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ]		(8-736)

fPadL[ dir ][ 7 ] = 0		(8-737)

–	Otherwise, if padVal[ dir ] is equal to 2, fPadL[ dir ] is modified as follows:

fPadL[ dir ][ 5 ] = fPadL[ dir ][ 5 ] + fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ] 	(8-738)

fPadL[ dir ][ 6 ] = 0		(8-739)

fPadL[ dir ][ 7 ] = 0		(8-740)

When MotionModelIdc[ xSb ][ ySb ] is greater than 0 [Ed. (JC): the condidtion of affine block seems unnecessary since 4x4 normal inter block does not exist anymore] and sbWidth is equal to 4 and sbHeight is equal to 4, fPadL[ dir ] is modified as follows for dir being equal to 0 and 1:

fPadL[ dir ][ 1 ] = fPadL[ dir ][ 0 ] + fPadL[ dir ][ 1 ]		(8-741)

fPadL[ dir ][ 0 ] = 0		(8-742)

fPadL[ dir ][ 6 ] = fPadL[ dir ][ 6 ] + fPadL[ dir ][ 7 ]		(8-743)

fPadL[ dir ][ 7 ] = 0		(8-744)

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(8-745) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(8-746)

The predicted luma sample value predSampleLXL is derived as follows:

–	If both xFracLand yFracL are equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3	(8-747)

–	Otherwise, if xFracL is not equal to 0 and yFracL is equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yInt3 ]   >>  shift1	(8-748)

–	Otherwise, if xFracL is equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 1 ][ i ] * refPicLXL[ xInt3 ][ yInti ]   >>  shift1	(8-749)

–	Otherwise, if xFracL is not equal to 0 and yFracL is not equal to 0, the value of predSampleLXL is derived as follows:

- The sample array temp[ n ] with n = 0..7, is derived as follows:

temp[ n ] =  i=07fPadL[ 0 ][ i ] * refPicLXL[ xInti ][ yIntn ]   >>  shift1	(8-750)

- The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL =  i=07fPadL[ 1 ][ i ] * temp[ i ]   >>  shift2	(8-751)

Table 8-13 – Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p.

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

##### 8.5.6.3.3 Luma integer sample fetching process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

xOffset = xFrac > 7 ? 1 : 0

yOffset = yFrac > 7 ? 1 : 0

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(8-752)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL − 1 + xOffset) : xIntL − 1  + xOffset)

yInt = Clip3( 0, picH − 1, yIntL − 1 + yOffset)		(8-753)

The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt ][ yInt ] << shift3		(8-754)

##### 8.5.6.3.4 Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthC − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthC ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

The chroma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p equal to xFracC or  yFracC are specified in Table 8-14.

The chroma interpolation filter coefficients fPadC[ 0 ] are set equal to fC[ xFracL ] and fPadC[ 1 ] are set equal to fC[ yFracL ] and modified depending on padVal[ dir ] as follows for dir being equal to 0 and 1:

–	If padVal[ dir ] is equal to − 1, fPadC[ dir ] is modified as follows:

fPadC[ dir ][ 1 ] = fPadC[ dir ][ 1 ] + fPadC[ dir ][ 0 ]		(8-755)

fPadC[ dir ][ 0 ] = 0		(8-756)

–	Otherwise, if padVal[ dir ] is equal to 1, fPadC[ dir ] is modified as follows:

fPadC[ dir ][ 2 ] = fPadC[ dir ][ 2 ] + fPadC[ dir ][ 3 ]		(8-757)

fPadC[ dir ][ 3 ] = 0		(8-758)

The variable xOffset is set equal to ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY ) / SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(8-759)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(8-760)

The predicted chroma sample value predSampleLXC is derived as follows:

–	If both xFracC and yFracC are equal to 0, the value of predSampleLXC is derived as follows:

predSampleLXC = refPicLXC[ xInt1 ][ yInt1 ] << shift3	(8-761)

–	Otherwise if xFracC is not equal to 0 and yFracC is equal to 0, the value of predSampleLXC is derived as follows:

predSampleLXC =  i=03fPadC[ 0 ][ i ] * refPicLXC[ xInti ][ yInt1 ]   >>  shift1	(8-762)

–	Otherwise if xFracC is equal to 0 and yFracC is not equal to 0, the value of predSampleLXC is derived as follows:

predSampleLXC =  i=03fPadC[ 1 ][ i ] * refPicLXC[ xInt1 ][ yInti ]   >>  shift1 	(8-763)

–	Otherwise if xFracC is not equal to 0 and yFracC is not equal to 0, the value of predSampleLXC is derived as follows:

- The sample array temp[ n ] with n = 0..3, is derived as follows:

temp[ n ] =  i=03fPadC[ 0 ][ i ] * refPicLXC[ xInti ][ yIntn ]   >>  shift1	(8-764)

- The predicted chroma sample value predSampleLXC is derived as follows:

predSampleLXC =( fPadC[ 1][ 0 ] * temp[ 0 ] +
					 fPadC[ 1 ][ 1 ] * temp[ 1 ] +
					 fPadC[ 1 ][ 2 ] * temp[ 2 ] +		(8-765)
					 fPadC[ 1 ][ 3 ] * temp[ 3 ] ) >> shift2