| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0511 |
| --- | --- |

| Title: | AHG12: On tiles and bricks partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naël Ouedraogo Eric Nassor | Email:Email: | nael.ouedraogo@crf.canon.fr eric.nassor@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes to replace the tile grid by a one-row tile partitioning and then to divide each tile column into bricks. The scan order of the bricks in the picture depends on the slice mode. In raster scan slice mode, this scan follows the raster scan of a picture. In rectangular slice mode, this scan follows the raster scan within tile columns. For raster scan mode, bricks are constrained to form a grid.

It is asserted that this partitioning approach reduces the complexity of the partitioning because there is only one possible tile and brick partitioning for a given pattern of rectangular regions in one frame and the brick scan order is independent of the partitioning.

Syntax elements related to tiles rows have been removed from PPS. The PPS includes new syntax elements to describe patterns of bricks split.. It is reported that the syntax is more compact for repetitive brick split patterns.

# Introduction

Since the introduction of bricks in VVC, the partitioning of a frame in regions is not unique. For example, a frame is divided into 3x3 rectangular regions as represented in the figures below (tile boundaries in blue and brick boundaries in black).

Figure 1: Examples of tile and brick partitioning of a frame in nine regions

The frame on the left is divided into 3x3 tile grid. The frame on the center is divided into 2x3 tile grid with two bricks in each tile of the bottom tile row. The frame on the right is divided into a 1x3 tile grid with 3 bricks per tile. These examples show that different partitionings can be used with the same effect on the prediction dependencies. The main difference between the three partitionings is the scanning order of the bricks in the frames (c.f. brick scan order index in the figure).

We think that all these alternatives of partitionings and their impact on the brick scan order provide an unnecessary complexity.

# Proposal

## Main principles

The proposal consists in replacing the tile grid partitioning by a one-row partitioning. In other words, each frame of the video sequence is divided into tile columns. Each tile column is then divided into bricks.

As a result, the brick and tile partitioning of a frame is unique as represented in the Figure below.

Figure 2: Tile and brick partitioning of the previous example

The scan order of the bricks in raster scan slice mode is raster-scan of a picture. Each brick is processed horizontally across the tile boundaries to avoid processing the frame vertically. To avoid complex scanning patterns, in raster scan mode it is a requirement that the partitions in bricks form a grid.

Figure 3: Scan order of bricks in raster-scan slice mode

In the raster-scan slice mode, a slice contains a sequence of bricks in a brick raster-scan of a picture from one or several tiles of the picture.

The brick scan order for rectangular slice mode is the same as a 1xN tile grid. It consists in scanning the bricks in raster-scan within tile columns. In other words, the bricks are processed vertically from the leftmost to the rightmost tile.

In the rectangular slice mode, a slice contains a number of bricks of a picture that collectively form a rectangular region of the picture. The bricks within a rectangular slice are in the order of brick raster-scan within tile column of the slice.

It is a requirement that top and left boundaries of a slice must be shared with previously decoded slices.

Figure 4: Examples of Scan order of bricks in rectangular mode

In both raster-scan and regular slice mode, the slice may contain one or more bricks from different tiles.

## PPS Syntax

Syntax elements that relate to tile rows have been removed from the PPS. PPS includes new syntax elements that specify patterns of brick split. When there is only one brick pattern, it applies to all the tile columns. Otherwise, for each tile column, a brick_layout index refers to one of the brick split pattern to indicate the partitioning in bricks of the tile.

When the slice mode is raster scan, it is a requirement that bricks form a grid. This is can be expressed by the requirement that num_brick_patterns_minus1 is equal 0.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| […] |  |
| if( !single_brick_in_pic_flag ) { |  |
| uniform_tile_spacing_flag | u(1) |
| if( uniform_tile_spacing_flag ) { |  |
| tile_cols_width_minus1 | ue(v) |
| tile_rows_height_minus1 | ue(v) |
| } else { |  |
| num_tile_columns_minus1 | ue(v) |
| num_tile_rows_minus1 | ue(v) |
| for( i = 0; i < num_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i < num_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| num_brick_patterns_minus1 | ue(v) |
| for( i = 0; i < num_brick_patterns_minus1+1; i++ ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| brick_splitting_present_flag | u(1) |
| for( i = 0; brick_splitting_present_flag && i < NumTilesInPic; i++ ) { |  |
| if(!uniform_tile_spacing_flag ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| if(num_brick_patterns_minus1 > 0) |  |
| brick_layout[i] | u(v) |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus1[ i ] | ue(v) |
| for( j = 0; j < num_brick_rows_minus1[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| […] |  |
| } |  |

num_brick_patterns_minus1 plus 1 is equal to the number of brick splitting patterns described in the PPS. The value of num_brick_patterns_minus1 should be in the range of 0 to num_tile_columns_minus1.

uniform_brick_spacing_flag[ i ] equal to 1 specifies that horizontal brick boundaries are distributed uniformly across the tile for the i-th brick pattern and signaled using the syntax element brick_height_minus1[ i ]. uniform_brick_spacing_flag[ i ] equal to 0 specifies that horizontal brick boundaries may or may not be distributed uniformly across the tile for the i-th brick pattern and signaled using the syntax element num_brick_rows_minus1[ i ] and a list of syntax elements brick_row_height_minus1[ i ][ j ].

brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the tile for the i-th brick pattern in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to PicHeightInCtbY − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal PicHeightInCtbY − 1.

num_brick_rows_minus1[ i ] plus 1 specifies the number of bricks partitioning the i-th brick pattern when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus1[ i ] shall be in the range of 1 to PicHeightInCtbY − 1, inclusive. When not present, the value of num_brick_rows_minus1[ i ] is inferred to be equal to Ceil( PicHeightInCtbY / ( brick_height_minus1[ i ] + 1 ) ).

brick_row_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th brick in a tile for the i-th brick pattern in units of CTBs.

brick_layout[ i ] is the index of the brick pattern that specifies the brick partitioning of the i-th tile. When not present, brick_layout[ i ] is inferred equal to 0. The length of brick_layout[ i ] is equal to Ceil( Log2( num_brick_patterns_minus1 + 1 ) ) bits.

### Conclusion

To sum up, we think that our design is less complex for two reasons: First, we obtain a single partitioning for a given pattern of regions instead of multiple partitionings as with tile grid followed by brick partitioning.

Secondly, the scan order is defined independently of the partitioning which is not the case with current VVC.

Finally, the proposed syntax is more compact for repetitive brick split patterns.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).