| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0516 |
| --- | --- |

| Title: | Non-CE4 : BCW inheritance for pairwise |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. Le Léannec, F. Galpin | Email: | antoine.robert@interdigital.com tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com franck.galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

The merge pairwise candidate is the only remaining merge candidate without BCW weights in VTM-5.0. This contribution describes three simple BCW weights inheritance methods depending on the inter direction of the merge candidates used to compute the pairwise merge candidate.

For the 2 first proposed methods, some luma BD-rate loss is obtained in RA (0.03% and 0.02%) and some gain is obtained in LDB (-0.01% and -0.02%). For the third proposed method, the BD-rate performances are the same as VTM-5.0 in both RA and LDB and small gains are obtained with the constructed affine BCW inheritance fix. The encoding and decoding times are unchanged compared to VTM-5.0 with all methods.

# Introduction

During Geneva JVET meeting, a new BCW weights derivation process for the constructed affine candidates JVET-N0481 [1] has been adopted. Consequently, in the merge and affine merge lists, the only candidate that does not inherit BCW weights is the merge pairwise candidate.

In order to harmonize the design of the current VVC, it is proposed to inherit BCW weights for the pairwise merge candidate so that every merge candidate can inherit BCW from MV predictors.

# Description

## Pairwise candidate construction

The merge pairwise candidate is constructed as an average of the two first candidates of the merge list, denoted Cand0 and Cand1 respectively in the following. The same process is performed for each reference picture list independently as described in Figure 1.

Figure 1: Pairwise candidate construction for reference frame list Lx

Then, depending on Cand0 and Cand1 uni-directional or bi-directional mode, the reference frame(s) used by the merge pairwise candidate can be deduced as presented in Table 1.

Table 1: Reference frames usage for pairwise candidate (grey is for bi-directional candidate.
a0, a1, b0, b1 represent the reference frame indices assigned to each candidate in L0 and L1.
red, yellow and green boxes respectively correspond to the proposed method 1, 2 and 3.)

|  | Cand0 |  | Cand1 |  | Pairwise |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | L0 | L1 | L0 | L1 | L0 | L1 |
| Reference frames | a0 | a1 | b0 | b1 | a0 | a1 |
|  | a0 | a1 | b0 | / | a0 | a1 |
|  | a0 | a1 | / | b1 | a0 | a1 |
|  | a0 | / | b0 | b1 | a0 | b1 |
|  | / | a1 | b0 | b1 | b0 | a1 |
|  | a0 | / | b0 | / | a0 | / |
|  | a0 | / | / | b1 | a0 | b1 |
|  | / | a1 | b0 | / | b0 | a1 |
|  | / | a1 | / | b1 | / | a1 |

## Proposed BCW weights inheritance

BCW weights can only be applied to a bi-directional pairwise candidate (shown in grey in Table 1) and can only be inherited from a bi-directional input candidate (Cand0 and/or Cand1).

### Method 1: Full BCW weights inheritance

The full BCW weights inheritance is applied on all bi-directional pairwise candidate created from at least one bi-directional candidate (red box on Table 1).

As shown in Table 1, when Cand0 is bi-directional, then its reference picture indices are assigned to the resulting pairwise candidate. In method 1, the pairwise candidate inherits the BCW weights of Cand0 in these cases.

Moreover, method 1 also comprises the following second step.
When Cand1 is bi-directional (and Cand0 is uni-directional) and when the reference picture indices used by Cand0 and Cand1 on the commonly used reference picture lists are the same (a0=b0 or a1=b1 on the two last lines of the red box of Table 1 resp.), the two reference pictures indices of Cand1 are used by the pairwise candidate. In proposed method 1, the BCW weights of Cand1 are assigned to the pairwise candidate in such case.

### Method 2: Restricted BCW weights inheritance

In order to limit the number of reference picture index comparisons involved in the second step of the previous method, this second step is removed in the method 2 proposed, called restricted BCW weights inheritance method.

It is restricted to Cand0 directionality. When Cand0 is bi-directional, the pairwise candidate inherits the BCW weights of Cand0 (yellow box on Table 1).

### Method 3: Fully restricted BCW weights inheritance

In a even more restricted method 3 for BCW weights inheritance, the pairwise candidate inherits BCW weights of Cand0 only when both input candidates are bi-directional and use the same BCW weights (green box on Table 1).

# Simulation results

The simulations are conducted using the VTM configuration as described in JVET-N1010 [2]. VTM-5.0 reference software is used with VTM configuration files.

## Full BCW weights inheritance (Method 1)

Table 2 and Table 3 show the simulation results of method 1 of the proposed BCW weights inheritance in random access and low delay B configurations. The proposed BCW weights inheritance slightly decreases the BD-rate performances in random access (0.03%) but increases it a bit in low delay B (-0.01%) with the same encoding/decoding time as VTM-5.0.

Table 2: Simulation results of the full BCW weights inheritance for pairwise candidate in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.19% | 0.15% | 101% | 101% |
| Class A2 | 0.05% | 0.04% | 0.05% | 100% | 101% |
| Class B | 0.04% | 0.04% | -0.07% | 99% | 99% |
| Class C | 0.00% | 0.07% | -0.15% | 99% | 97% |
| Overall | 0.03% | 0.00% | -0.03% | 100% | 99% |
| Class D | 0.03% | 0.05% | 0.11% | 99% | 98% |
| Class F (optional) | 0.00% | 0.08% | 0.04% | 100% | 97% |

Table 3: Simulation results of the full BCW weights inheritance for pairwise candidate in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.02% | 0.20% | -0.17% | 101% | 101% |
| Class C | 0.00% | -0.20% | -0.05% | 100% | 101% |
| Class E | -0.03% | -0.44% | -0.65% | 100% | 100% |
| Overall | -0.01% | -0.09% | -0.25% | 101% | 101% |
| Class D | 0.01% | -0.10% | 0.69% | 100% | 99% |
| Class F (optional) | -0.12% | -0.17% | -0.34% | 101% | 102% |

## Restricted BCW weights inheritance (Method 2)

Table 4 and Table 5 show the simulation results of method 2 of the proposed BCW weights inheritance in random access and low delay B configurations. The proposed BCW weights inheritance slightly decreases the BD-rate performances in random access (0.02%), a little bit less than the first method, but increases it in low delay B (-0.02%), a little bit more than the first method, with the same complexity in encoding/decoding time.

Table 4: Simulation results of the restricted BCW weights inheritance for pairwise candidate in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.13% | 0.12% | 101% | 101% |
| Class A2 | 0.05% | 0.07% | 0.06% | 100% | 100% |
| Class B | 0.02% | 0.00% | -0.03% | 100% | 100% |
| Class C | -0.01% | 0.01% | -0.05% | 101% | 101% |
| Overall | 0.02% | -0.01% | 0.01% | 100% | 101% |
| Class D | 0.02% | 0.11% | -0.04% | 101% | 100% |
| Class F (optional) | 0.00% | 0.06% | -0.01% | 99% | 96% |

Table 5: Simulation results of the restricted BCW weights inheritance for pairwise candidate in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.03% | 0.05% | -0.18% | 100% | 99% |
| Class C | 0.01% | 0.16% | -0.14% | 100% | 100% |
| Class E | -0.07% | -0.22% | -0.64% | 100% | 100% |
| Overall | -0.02% | 0.02% | -0.28% | 100% | 99% |
| Class D | -0.03% | 0.11% | 0.49% | 99% | 99% |
| Class F (optional) | -0.04% | -0.13% | -0.50% | 100% | 99% |

## Fully restricted BCW weights inheritance (Method 3)

Table 6 and Table 7 show the simulation results of method 3 of the proposed BCW weights inheritance in random access and low delay B configurations. The proposed BCW weights inheritance keeps exactly the BD-rate performances in random access and in low delay B with the same complexity in encoding/decoding time.

Table 6: Simulation results of the fully restricted BCW weights inheritance for pairwise candidate in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.09% | 101% | 101% |
| Class A2 | 0.03% | -0.02% | 0.03% | 100% | 101% |
| Class B | 0.01% | -0.02% | -0.10% | 100% | 100% |
| Class C | -0.02% | -0.06% | 0.01% | 101% | 101% |
| Overall | 0.00% | -0.04% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.09% | -0.03% | 101% | 100% |
| Class F (optional) | -0.02% | 0.07% | 0.01% | 99% | 97% |

Table 7: Simulation results of the fully restricted BCW weights inheritance for pairwise candidate in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | 0.36% | -0.09% | 100% | 98% |
| Class C | 0.04% | 0.08% | -0.04% | 100% | 101% |
| Class E | -0.03% | -0.03% | -0.30% | 100% | 101% |
| Overall | 0.00% | 0.17% | -0.13% | 100% | 100% |
| Class D | 0.01% | -0.31% | 0.28% | 99% | 97% |
| Class F (optional) | 0.05% | -0.25% | 0.20% | 99% | 99% |

In VTM-5.0, there exists a bug, already solved in the VTM-5.1 [3], in the previously adopted JVET-N0481 [2]. This is an initialization problem where BCW weights of the different explored positions for the constructed affine model are not all initially set to the default value. This bug impacts BCW inheritance for all other modes and the proposed solutions.

Table 8 and Table 9 show the simulation results of method 3 of the proposed BCW weights inheritance in random access and low delay B configurations over a VTM-5.0 with a fix for the constructed affine BCW inheritance [3]. The proposed BCW weights inheritance keeps the same trend as without the fix in random access (close to 0%) and in low delay B, the performances seem a bit better and may lead to small gains. The complexity in encoding/decoding time remains the same as the VTM-5.0.

Table 8: Simulation results of the fully restricted BCW weights inheritance for pairwise candidate in random access
over a VTM-5.0 with a fix for the constructed affine BCW inheritance

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over fixed VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | -0.08% | 101% | 100% |
| Class C | 0.00% | -0.05% | 0.09% | 101% | 99% |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.08% | -0.02% | 101% | 101% |
| Class F (optional) |  |  |  |  |  |

Table 9: Simulation results of the fully restricted BCW weights inheritance for pairwise candidate in low delay B
over a VTM-5.0 with a fix for the constructed affine BCW inheritance

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over fixed VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |
| Class C | 0.01% | 0.08% | -0.19% | 101% | 100% |
| Class E | -0.04% | -0.39% | -0.20% | 101% | 101% |
| Overall |  |  |  |  |  |
| Class D | -0.04% | 0.07% | -0.31% | 101% | 100% |
| Class F (optional) |  |  |  |  |  |

# Conclusion

To be able to set BCW weights to the merge pairwise candidate, the only remaining merge candidate without BCW inheritance, this contribution describes three simple BCW weights inheritance methods. In the method 1, BCW weights inheritance depends on the inter directions of both input candidates and their reference pictures index. In the method 2, BCW weights inheritance depends only on the inter direction of the first input candidate. And in a method 3, BCW weights inheritance depends on the inter directions of both input candidates and their BCW weights.

The two first methods decrease the BD-rate performances in RA configuration (0.03% and 0.02% resp.) by remaining close to those of the VTM-5.0. But in LDB configuration, the BD-rate performances are increased (-0.01% and -0.02% resp.). With the third method, the BD-rate performances remain exactly the same as VTM-5.0 in both configurations and can reach small gains with the constructed affine BCW inheritance fix. And in any cases, encoding and decoding complexity are identical to VTM-5.0.

# References

[1] J. Chen, R.-L. Liao, Y. Ye, “Non-CE2: GBi inheritance for constructed affine merge candidate”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N0481, 14th Meeting, Geneva, CH, March 2019.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-N1010, 14th Meeting, Geneva, CH, March 2019.

[3] B. Heng, “Bug in GBI inheritance for constructed affine merge candidate (JVET-N0481)”, Ticket #312, https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/312

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).