| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0519 |
| --- | --- |

| Title: | Switching between CABAC context coded bins and EP coded bins |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This document describes a modified coding method which introduces a change to entropy coding.  This change allows for a simplification in coefficient coding and removes the need to use cabac_zero_word padding. The modification works by dynamically swapping between EP and CABAC bin coding.   Two different implementations are described, the second of which is designed to limit the number of exchanges between CABAC and EP bin coding.  Results are given which show gains in low QP conditions (-0.42% for class A1 in All Intra) without harming performance in CTC conditions.

# Introduction

Section 7.4.3.11 of the specification [1] states the following:

Let NumBytesInVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )	(7-39)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

Thus the number of times DecodeBin (in terms of the specification this includes CABAC, EP, and TRM) be called is dependent on a static quantity which may be computed at the beginning of the picture / slice and a dynamic quantity based on the number of bytes in the NAL unit. Cabac_zero_word syntax is used to ensure this limit. This provides a decoder a limit from which to work.

To help reduce the number of CABAC codec bins, JVET-L0274[3] introduced limits of 32 (4x4) and 8 (2x2) to the number of bins that could be coded for sub-TU blocks.  This added a local level reduction, but it was still possible to reach the top-level limit, causing cabac_zero_words to be required.

In addition, JVET-N0280[4] introduced a limit to the number of CABAC bins that could be coded of two times the area of the transform unit for TS blocks. Again, this helped, but did not prevent cabac_zero_words being required.

# Proposed implementation

Instead of using cabac_zero_words, wherein bytes are added to the end of the slice to maintain a bit/bin ratio, it is proposed that cabac bins themselves are automatically changed to EP bins to maintain this ratio.

Two implementations have been developed, as described below.  Both implementations have removed the limits imposed in the coefficient coding documents JVET-L0274 and JVET-N0280 (other aspects of those proposals have been maintained). This has subsequently simplified the specification.

Both implementations make use of both the static and dynamic parts of the constraint given above to monitor the number of CABAC bins that can be decoded.  The dynamic component allows for 4  3 CABAC bins to be decoded for every bit read from the NAL unit.  Thus the counter used to monitor the coding process is incremented by 4 for every BIT read and decremented by 3 for every BIN decoded (CABAC, EP or TRM).  The static component allows the initial value of the counter to be set to 3 * ( RawMinCuBits * PicSizeInMinCbsY )  32 at the beginning of every picture (or similar, if there is finer granularity in the constraints).

i.e. for CABAC coding, for every renomalisation, the counter is incremented by 4; for every flag decode, the counter is decremented by 3.

i.e. for EP coding, the counter is effectively incremented by 1.

In addition, this scheme provides an alternative to CABAC skip mode described in JVET-N0207, with the possibility of reducing the number of CABAC coded bins to a smaller proportion of the bitstream, with numerous operating points (i.e. not just coding all CABAC coded bins as EP bins).

## Implementation 1 – immediate switch

In this first implementation, the value of the counter is checked each time a CABAC bin is to be coded.  If the value is greater or equal to 3 the bin is coded using CABAC, if not the bin is coded using EP.  Thus, in effect, CABAC usage is rationed to ensure the constraint given in section 7.4.3.11 is always met.

## Implementation 2 – group switch

The second implementation modifies the first as follows, noting that JVET-N0280 introduced the two method scheme for coding TU.

For a TS-coded TU the counter is checked before coding any coefficient flags to ensure the counter’s value is 15 times the area of the block (i.e. it can decode 5 flags per coefficient – the maximum requirement).  If this is the case CABAC coding will be used for all the coefficient flags within the TU, if not EP coding will be used for all the coefficient flags.

For non TS-coded TUs, the counter is checked before decoding any coefficient flags to ensure the counter’s value is 12 times the area of the block (i.e. it can decode 4 flags per coefficient – significance, gt1, gt2 and parity).  If this is the case CABAC coding will be used for all the coefficient flags within the TU, if not EP coding will be used for all the coefficient flags.

This second implementation thus reduces the number of times the counter needs to be checked, and reduces the number of times and places that the decoder switches from CABAC to EP decoding.

Following TU coding, the immediate switch method is re-enabled for all subsequent CABAC coded symbols. Potentially switch points could be only available at the start of a CU/TU/PU.

# Results

In both implementations, the existing cabac_zero_word scheme was present in the system, but was not required, indicating that the schemes were successful.

Results for some of the low-QP simulations are still running and may be updated in a later revision.

## Implementation 1 – immediate switch

|  | All Intra - CTC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.10% | -0.02% | 102% | 101% |
| Class A2 | -0.02% | -0.08% | -0.07% | 99% | 100% |
| Class B | -0.02% | -0.15% | -0.02% | 100% | 101% |
| Class C | -0.06% | -0.14% | -0.09% | 100% | 101% |
| Class E | -0.04% | -0.09% | -0.09% | 99% | 100% |
| Overall | -0.03% | -0.08% | -0.05% | 100% | 101% |
| Class D | -0.06% | 0.05% | -0.12% | 100% | 101% |
| Class F | -0.25% | -0.20% | -0.33% | 102% | 101% |

|  | Random Access - CTC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0-Benchmark-VTM-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.34% | 0.04% | 100% | 100% |
| Class A2 | 0.00% | -0.04% | -0.04% | 99% | 99% |
| Class B | 0.00% | -0.08% | -0.19% | 100% | 100% |
| Class C | -0.05% | -0.23% | -0.14% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.16% | -0.10% | 100% | 100% |
| Class D | 0.00% | -0.09% | 0.04% | 100% | 100% |
| Class F | -0.29% | -0.27% | -0.02% | 100% | 100% |

|  | All Intra – Low QP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0-Benchmark-LOWQP-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.60% | -0.63% | 104% | 105% |
| Class A2 | -0.39% | -0.58% | -0.58% | 104% | 107% |
| Class B | -0.06% | -0.12% | -0.16% | 103% | 106% |
| Class C | 0.05% | -0.03% | -0.04% | 106% | 107% |
| Class E | 0.02% | -0.12% | -0.14% | 102% | 104% |
| Overall | -0.14% | -0.26% | -0.28% | 104% | 106% |
| Class D | 0.03% | -0.05% | -0.03% | 102% | 104% |
| Class F | -0.81% | -0.75% | -0.76% | 102% | 103% |

|  | Random Access – Low QP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0-Benchmark-LOWQP-JVET-M1010 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.35% | -0.53% | -0.59% | 101% | 102% |
| Class C | -0.25% | -0.40% | -0.36% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.25% | -0.34% | -0.36% | 102% | 103% |
| Class F | -3.42% | -0.70% | -0.78% | 101% | 101% |

## Implementation 2 – group switch

|  | All Intra - CTC |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM5.0-Benchmark-VTM-JVET-M1010 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | -0.01% | 0.10% |  | -0.02% |  | 101% |  | 101% |  |
| Class A2 | -0.02% | -0.08% |  | -0.07% |  | 101% |  | 101% |  |
| Class B | -0.02% | -0.15% |  | -0.02% |  | 101% |  | 102% |  |
| Class C | -0.06% | -0.14% |  | -0.09% |  | 102% |  | 102% |  |
| Class E | -0.04% | -0.09% |  | -0.09% |  | 100% |  | 100% |  |
| Overall | -0.03% | -0.08% |  | -0.05% |  | 101% |  | 101% |  |
| Class D | -0.06% | 0.05% |  | -0.12% |  | 101% |  | 102% |  |
| Class F | -0.25% | -0.20% |  | -0.33% |  | 101% |  | 102% |  |
|  | Random Access - CTC |  |  |  |  |  |  |  |  |
|  | Over VTM5.0-Benchmark-VTM-JVET-M1010-Cluster1 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.01% | -0.34% |  | 0.04% |  | 101% |  | 100% |  |
| Class A2 | 0.00% | -0.04% |  | -0.04% |  | 101% |  | 100% |  |
| Class B | 0.00% | -0.08% |  | -0.19% |  | 101% |  | 101% |  |
| Class C | -0.05% | -0.23% |  | -0.14% |  | 101% |  | 101% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.16% |  | -0.10% |  | 101% |  | 101% |  |
| Class D | 0.00% | -0.09% |  | 0.04% |  | 100% |  | 101% |  |
| Class F | -0.29% | -0.27% |  | -0.02% |  | 101% |  | 100% |  |
|  | All Intra – Low QP |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0-Benchmark-LOWQP-JVET-M1010 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class B | -0.06% |  | -0.12% |  | -0.16% |  | 102% |  | 104% |  |
| Class C | 0.05% |  | -0.03% |  | -0.04% |  | 105% |  | 101% |  |
| Class E | 0.02% |  | -0.12% |  | -0.14% |  | 102% |  | 102% |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class D | 0.03% |  | -0.05% |  | -0.03% |  | 105% |  | 103% |  |
| Class F | -0.81% |  | -0.75% |  | -0.76% |  | 104% |  | 102% |  |
|  | Random Access – Low QP |  |  |  |  |  |  |  |  |  |
|  | Over VTM5.0-Benchmark-LOWQP-JVET-M1010 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class A2 | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class B | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class C | -0.24% |  | -0.39% |  | -0.35% |  | 101% |  | 102% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |
| Class D | -0.25% |  | -0.34% |  | -0.35% |  | 102% |  | 100% |  |
| Class F | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  |

# Conclusion

A modified coding technique is described which introduces a change to entropy coding and simplifications to coefficient coding. The technique essentially rations the use of CABAC bins to ensure that the constraint given in section 7.4.3.11 of the specification is never broken.  Two implementations are described and give similar results with no net loss in CTC and gains in both AI and RA at low QP.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu

- “CE7: Transform coefficient coding with reduced number of regular coded bins (tests 7.1.3a, 7.1.3b)”, JVET-L0274, H.Schwarz, T.Nguyen, D.Marpe, T.Wiegand, M.Karczewicz, M.Coban, J.Dong

- “CE8: Residual Coding for Transform Skip Mode (CE8-4.3a, CE8-4.3b, CE8-4.4a, and CE8-4.4b)”, JVET-N0280, B.Bross, T.Nguyen, H.Schwarz, D.Marpe, T.Wiegand