| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0520 |
| --- | --- |

| Title: | GT1 flag coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Steve Keating Karl Sharman | Email: | adrian.browne@sony.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

For some sequences it has been observed that at certain QPs there is a peak in the number of GT1 flags coded but which predominantly indicated a value of 0.

This contribution describes the introduction of a GT1_GROUP flag into coefficient coding.  This flag is set to indicate the presence of a value greater than one in a transform unit block.  The GT1_GROUP flag is only coded if a statistical measure indicates a very high probability this flag is unset.  When this flag is present and unset, there is no need to code a GT1 flag for each significant coefficient.

For the above cases, it is reported that the introduction of the flag results in a significant reduction (~40%) in the number of GT1 flags that must be coded.  This reduces the workload on the entropy decoder, can reduce the amount of cabac_zero_word padding, and can result in BD-rate reductions for some sequences.

# Introduction

It was first observed during low-QP simulations some sequences require the addition of considerable amounts of cabac_zero_word padding, e.g. DaylightRoad2 at QP=17, requires 1,606,905 bytes of padding, approximately 1.83% of the total bytes required for the sequence.

When statistics for this sequence at QP=17 were compared with the average statistics for all RA sequences at QPs 2, 7, 12, 17, 22, 27, 32 and 37 an obvious difference was seen in the statistics for the GT1 flag.  For DaylightRoad2 at QP=17, the CABAC ratio for GT1 flags was 17.6%, whereas the average over the normal and low QP CTCs (and splitting simulations into 1s GOPs) was 61.5%.  This was caused by the ratio of bins coded with a value of 1 being 4.1% for DaylightRoad2 versus 26.12% for the average. i.e. a significant number of flags are being coded with the value of 0. This is shown in the Figure 1 below.  Figures 2 and 3 show that other sequences exhibit the same characteristics.

Figure 1: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for DaylightRoad2 (Random Access).

Figure 2: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for CatRobot (Random Access).

Figure 3: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for BQTerrace (Random Access).

On further investigation, similar trends were observed with other sequences, especially for random-access, but also occasionally in intra simulations.  It is hypothesised that this occurs for highly predictable areas, where only low levels of non-correlated noise exists, which at certain QPs will get quantized to ±1.  In addition it was noticed that dependent quantization also encouraged this behaviour, however, this may be more to do with encoder RDO techniques than dependent quantization itself.

To address this problem of coding lots of GT1 flags with the same, highly probable value of 0, 2 implementations are proposed.

# Proposed Implementations

The proposed implementations introduce a GT1 group flag to signal that at least one of a group of coefficients has a magnitude of greater than one.  Thus, if this flag is coded and is 0, all the coefficients have a magnitude of no more than one, and no GT1 flags need to be coded for the group.

As not all sequences and conditions benefit from this type of coding, the decision on whether to code a GT1 group flag is based on a cumulative statistical measure which determines the probability that the group does not contain any values of magnitude greater than one.

In the proposed architecture this is implemented by reusing the same CABAC contexts used to code the GT1 group flag.  The CABAC contexts have been modified to allow the state of each GT1 group context to be read by the coefficient coder and to be updated without producing any output.

Two different implementations are proposed – one which operates on TUs and one which operates on sub-TUs.  Both implementations impose preconditions which must be passed before the statistical measure is evaluated.

i.e.

For a block of coefficients,

If pre-condition is met and statistical measure indicates GT1 group flag likely to be 0.

Code GT1 group flag

Else

GT1 group flag is not coded.

Then, after coding coefficients

If GT1 group flag was not coded [and for implementation 1, if pre-condition had been met]

Update GT1 group flag as if it were coded.

and where the statistical measure indication is given by a threshold of the associated context state.

## Implementation 1 – TU based

In the first case, that being TU-based implementation, the precondition before considering GT1 group coding is a that the size of the TU is at least 4x4 and a test on the (x, y) position of the last significant coefficient.  The x and y values for this coefficient are added together and compared with a threshold.  If the value is greater or equal to the threshold, the statistical measure is considered.  This threshold is half log2 of the maximum of the width and height of the TU.

The group of coefficients considered for the GT1 group flag excludes a small number at the top left of the coefficient block (3 for 4x4 DCT, 6 for larger DCT blocks, 0 for other blocks).  These are those coloured green in the following diagram.

Last (x, y) position indicates if precondition is met.

Last (x, y) position indicates if precondition is met.

Green coefficients are coded using GT1_FLAGs

Green coefficients are coded using GT1_FLAGs

If the GT1 group flag is coded and unset, GT1 flags for blue coefficients do not need to be coded.  If the GT1 Group flag is not coded or set, GT1 flags are used

White positions denote non-significant coefficients.

White positions denote non-significant coefficients.

The following flow chart describes the coding of GT1 group flags for this implementation

## Implementation 2 – Sub-TU based

The second implementation is handled in the same sub-group structure as the significance group flag.

The precondition is dependent on the position of the first significant coefficient in a 4x4 block.  If this indicates at least 8 coefficients could exist to make the new technique worthwhile the precondition is passed.

The flowchart below describes the coding of each coefficient within the sub-TU.  The GT1 group context is updated once all coefficients have been coded if the GT1 group is not coded.

# Results

## TU based Implementation

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.00% | 102% | 101% |
| Class A2 | 0.00% | 0.01% | 0.03% | 102% | 102% |
| Class B | 0.00% | -0.04% | -0.04% | 102% | 101% |
| Class C | 0.00% | -0.01% | 0.02% | 103% | 104% |
| Class E | 0.00% | 0.01% | 0.02% | 101% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Class D | 0.00% | 0.01% | -0.04% | 103% | 101% |
| Class F | -0.01% | -0.01% | 0.07% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.03% | -0.06% | 101% | 101% |
| Class A2 | 0.02% | 0.01% | 0.08% | 101% | 101% |
| Class B | 0.02% | 0.05% | -0.08% | 101% | 101% |
| Class C | -0.01% | -0.13% | -0.13% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.02% | -0.06% | 101% | 101% |
| Class D | 0.01% | -0.04% | 0.02% | 101% | 102% |
| Class F | 0.00% | 0.02% | 0.02% | 102% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.36% | 0.32% | 101% | 101% |
| Class C | 0.05% | 0.04% | 0.02% | 101% | 101% |
| Class E | 0.15% | -0.02% | -0.52% | 101% | 103% |
| Overall | 0.07% | 0.16% | 0.01% | 101% | 102% |
| Class D | 0.06% | -0.08% | 1.12% | 101% | 100% |
| Class F | -0.02% | 0.44% | 0.01% | 101% | 102% |

|  |  |  | AI(LowQP) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.02% | 105% | 104% |
| Class A2 | 0.00% | 0.00% | 0.01% | 102% | 101% |
| Class B | 0.00% | -0.02% | 0.00% | 106% | 105% |
| Class C | 0.00% | 0.01% | 0.00% | 107% | 104% |
| Class E | 0.00% | 0.05% | 0.02% | 105% | 104% |
| Overall | 0.00% | 0.00% | 0.01% | 105% | 104% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | -0.01% | 0.00% | 103% | 102% |
|  |  |  | RA(LowQP) |  |  |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.05% | 101% | 101% |
| Class A2 | -0.09% | -0.02% | -0.04% | 100% | 100% |
| Class B | 0.01% | 0.06% | 0.07% | 102% | 102% |
| Class C | 0.00% | 0.03% | 0.09% | 105% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.03% | 0.05% | 102% | 102% |
| Class D | 0.00% | 0.03% | 0.00% | 104% | 103% |
| Class F | 0.00% | 0.03% | 0.00% | 102% | 100% |
|  |  |  | LDB(LowQP) |  |  |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.06% | 0.07% | 102% | 101% |
| Class C | -0.01% | 0.08% | 0.12% | 103% | 102% |
| Class E | 0.01% | 0.21% | 0.08% | 104% | 103% |
| Overall | 0.00% | 0.10% | 0.09% | 103% | 102% |
| Class D | 0.01% | 0.00% | 0.07% | 103% | 101% |
| Class F | 0.06% | 0.09% | 0.08% | 102% | 100% |

The results show little BD-rate change, but examining the percentage of CABAC coded bins that are GT1 flags shows more significant changes for some sequences.  These lead to the complete elimination of cabac_zero_words at QP=17 (RA) for DaylightRoad2, QP=7 (LDB) for BasketballDrill, and QP=12 (RA) for Cactus.  When figures 4, 5, and 6 are compared with figures 1, 2 and 3, the described TU based implementation appears to reduce GT1 flag usage only where the ratio of GT1 flags that are 1 is low.

Figure 4: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for DaylightRoad2 (Random Access).

Figure 5: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for CatRobot (Random Access).

Figure 6: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for BQTerrace (Random Access).

## Sub-TU Implementation

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.00% | 101% | 100% |
| Class A2 | 0.02% | -0.03% | -0.01% | 101% | 100% |
| Class B | 0.00% | -0.01% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F | 0.00% | 0.00% | -0.01% | 102% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.01% | 100% | 100% |
| Class A2 | 0.02% | -0.03% | 0.01% | 99% | 98% |
| Class B | 0.01% | 0.07% | -0.06% | 100% | 100% |
| Class C | -0.01% | -0.04% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.02% | 100% | 100% |
| Class D | 0.01% | 0.01% | 0.06% | 100% | 100% |
| Class F | 0.00% | -0.02% | -0.01% | 100% | 100% |

Similarly to the TU based implementation, the results show little BD-rate changes, but examining the percentage of CABAC coded bins that are GT1 flags shows more significant changes for some sequences.  This leads to the elimination of cabac_zero_words for DaylightRoad2 at QP=22 (RA) as well as the sequences listed for the TU based implementation.

Figure 7: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for DaylightRoad2 (Random Access).

Figure 8: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for CatRobot (Random Access).

Figure 9: GT1 CABAC flag ratio (i.e 1s vs 0s for that flag) and the percentage of all CABAC coded bins that are GT1 flags, versus QP for BQTerrace (Random Access).

# Conclusion

A new flag GT1_GROUP has been introduced.  This flag signals if one of a group of coefficients has a magnitude greater than one.  A statistical measure has been described which determines if this flag should be used. Use of the flag has been shown to reduce the need for cabac_zero_words in sequences with large numbers of GT1 flags when the value of the flag is predominantly zero.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “JVET common test conditions and software reference configurations”, JVET-M1010, F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring

- “Versatile Video Coding (Draft 5)”, JVET-N1001-v8, B. Bross, J. Chen, and S. Liu