| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0523-v3 |
| --- | --- |

| Title: | CE3-related: simplification of Matrix Intra Prediction (MIP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thierry Dumas, Franck Galpin, Gagan Rath, Fabien Racape | Tel: Email: | +33.2.56.85.62.76 thierry.dumas@InterDigital.com franck.galpin@InterDigital.com gagan.rath@InterDigital.com fabrice.leleannec@InterDigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution reports the result of adding one Matrix-based intra prediction mode per block size, called Simplified Matrix Intra Prediction (SMIP) mode, in addition to the classical intra modes. Compared to the Matrix-based Intra Prediction (MIP) implemented in VTM-5.0, the memory footprint is reduced from

~8 kilobytes to 1 kilobyte. In addition, the decoding process does not require any extra MPM list derivation anymore, nor any mapping table between conventional modes and MIP modes. A loss of 0.14% in AI, and a loss of 0.10% in RA is reported for the luminance, when the proposed mode is not combined with MTS.

# Introduction

In the JVET meeting held in Geneva in March 2019, it was agreed to use Matrix Intra Prediction (MIP) [1] as additional intra prediction tool. In this contribution, we propose to use only 1 mode for each of the following block sizes, instead of the 35 MIP modes in the case of 4x4 blocks to be predicted, 19 MIP modes in the case of 8x4, 4x8, and 8x8 blocks to be predicted, and 11 MIP modes in the other cases.

# Proposed solution

For all block sizes, the MIP modes are replaced by a single mode, called Simplified Matrix Intra Prediction (SMIP) mode.

For predicting the current block from decoded reference samples via the SMIP mode, the averaging of the boundary, the affine transformation, and the linear interpolation are identical to those proposed in [1]. However, the signalization of SMIP does not require any list of MPMs nor mapping between each MIP mode and one of the 67 regular intra prediction modes. The SMIP mode is always mapped to planar. The mapping table footprint of 172 coefficients in VTM-5.0, i.e. 0.17 kilobytes when stored as 8-bit values, is then set to zero.

For signaling SMIP, a flag smipFlag is coded after the first MPM mode, as shown in Table 1.

Table 1: codeword for each MPM and SMIP

| Candidate Index | code |
| --- | --- |
| MPM[0] | 10 |
| SMIP | 111 |
| MPM[1] | 1100 |
| MPM[2] | 11010 |
| MPM[3] | 110110 |
| MPM[4] | 1101110 |
| MPM[5] | 1101111 |

The number of required matrix and bias coefficients is the reduced from 6336 to 800 coefficients, resulting in a memory footprint reduction from 7.92 kilobytes to 1 kilobyte, when coded on 10-bit. The mapping MIP modes to classical ones requires.

Tow tests are reported in section 3:

- Test 1: SMIP as described above,

- Test 2: the possibility to combine SMIP and Multiple Transform Selection (MTS) is disabled.

# Experimental Results

Simulations were performed according to CTC described in [2] for CE3, All Intra (AI) and Random Access (RA) results are reported.

## Test 1

Table 2: AI results of VTM-5.0 with SMIP over VTM-5.0

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.33% | 0.27% | 0.12% | 93% | 99% |
| Class A2 | 0.17% | 0.06% | 0.00% | 96% | 100% |
| Class B | 0.14% | 0.02% | 0.06% | 97% | 100% |
| Class C | 0.13% | 0.12% | 0.18% | 94% | 97% |
| Class E | 0.18% | 0.13% | 0.11% | 97% | 103% |
| Overall | 0.18% | 0.11% | 0.09% | 95% | 100% |
| Class D | 0.20% | 0.21% | -0.07% | 97% | 101% |
| Class F | 0.08% | 0.21% | 0.04% | 103% | 107% |

Table 3: RA results of VTM-5.0 with SMIP over VTM-5.0

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.49% | 0.62% | 106% | 104% |
| Class A2 | 0.08% | 0.07% | 0.02% | 105% | 104% |
| Class B | 0.09% | 0.30% | 0.05% | 102% | 107% |
| Class C | 0.05% | 0.10% | 0.12% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.24% | 0.18% | 103% | 104% |
| Class D | 0.07% | -0.09% | 0.27% | 100% | 100% |
| Class F | 0.01% | -0.02% | 0.19% | 104% | 102% |

## Test 2

Table 4: AI results of VTM-5.0 with the variant of SMIP over VTM-5.0

|  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.31% | 0.24% | 0.19% | 98% | 99% |
| Class A2 | 0.14% | 0.05% | 0.02% | 99% | 99% |
| Class B | 0.11% | 0.06% | 0.14% | 103% | 102% |
| Class C | 0.09% | 0.01% | 0.15% | 99% | 98% |
| Class E | 0.11% | 0.11% | 0.20% | 98% | 99% |
| Overall | 0.14% | 0.09% | 0.14% | 100% | 99% |
| Class D | 0.14% | 0.29% | 0.13% | 102% | 101% |
| Class F | 0.02% | 0.22% | 0.05% | 104% | 107% |

Table 5: RA results of VTM-5.0 with the variant of SMIP over VTM-5.0

|  |  | Random access Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-5.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18% | 0.52% | 0.80% | 103% | 106% |
| Class A2 | 0.08% | -0.08% | 0.17% | 103% | 107% |
| Class B | 0.11% | 0.16% | 0.35% | 100% | 107% |
| Class C | 0.05% | 0.08% | 0.03% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.10% | 0.16% | 0.32% | 101% | 105% |
| Class D | 0.10% | 0.19% | -0.15% | 99% | 100% |
| Class F | 0.01% | -0.11% | 0.15% | 101% | 99% |

# Specification text changes

P56

if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) {

if( cbWidth <= 32 && cbHeight <= 32 )

intra_bdpcm_flag[ x0 ][ y0 ]

if( intra_bdpcm_flag[ x0 ][ y0 ] )

intra_bdpcm_dir_flag[ x0 ][ y0 ]

else {

if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY )

intra_mip_flag[ x0 ][ y0 ]

if( intra_mip_flag[ x0 ][ y0 ] ) {

intra_mip_mpm_flag[ x0 ][ y0 ]

if( intra_mip_mpm_flag[ x0 ][ y0 ] )

intra_mip_mpm_idx[ x0 ][ y0 ]

else

intra_mip_mpm_remainder[ x0 ][ y0 ]

} else {

if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) )

intra_luma_ref_idx[ x0 ][ y0 ]

if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) )

intra_subpartitions_mode_flag[ x0 ][ y0 ]

if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY )

intra_subpartitions_split_flag[ x0 ][ y0 ]

if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 0 )

intra_luma_mpm_flag[ x0 ][ y0 ]

if( intra_luma_mpm_flag[ x0 ][ y0 ] ) {

if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 )

intra_luma_not_planar_flag[ x0 ][ y0 ]

if( sps_mip_enabled_flag && intra_luma_not_planar_flag[ x0 ][ y0 ] ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY )

intra_mip_flag[ x0 ][ y0 ]

if( ! intra_mip_flag[ x0 ][ y0 ] && intra_luma_not_planar_flag[ x0 ][ y0 ])

intra_luma_mpm_idx[ x0 ][ y0 ]

} else

intra_luma_mpm_remainder[ x0 ][ y0 ]

}

}

}

# P116

intra_mip_flag[ x0 ][ y0 ] equal to 1 specifies that the intra prediction type for luma samples is matrix-based intra prediction. intra_mip_flag[ x0 ][ y0 ] equal to 0 specifies that the intra prediction type for luma samples is not matrix-based intra prediction.

When intra_mip_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

The syntax elements intra_mip_mpm_flag[ x0 ][ y0 ], intra_mip_mpm_idx[ x0 ][ y0 ] and intra_mip_mpm_remainder [ x0 ][ y0 ] specify the matrix-based intra prediction mode for luma samples. The array indices x0, y0 specify the location ( x0 , y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture. When intra_mip_mpm_flag[ x0 ][ y0 ] is equal to 1, the matrix-based intra prediction mode is inferred from a neighbouring intra-predicted coding unit according to clause 8.4.2.

When intra_mip_mpm_flag[ x0 ][ y0 ] is not present, it is inferred to be equal to 1.

P134

### Derivation process for MIP mode

Input to this process are:

- a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the matrix-based intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xCb − 1, yCb ) and ( xCb, yCb − 1 ), respectively.

- For X being replaced by either A or B, the variables candMipModeX are derived as follows:

- The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate MIP mode candMipModeX is derived as follows:

- If one or more of the following conditions are true, candMipModeX is set equal to −1.

- The variable availableX is equal to FALSE.

- CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA and ciip_flag[ xNbX ][ yNbX ] is not equal to 1.

- pcm_flag[ xNbX ][ yNbX ] is equal to 1.

- X is equal to B and yCb − 1 is less than ( ( yCb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ).

- Otherwise, the following applies:

- If intra_mip_flag[ xNbX ][ yNbX ] is equal to 1, the following applies:.

- If MipSizeId[ xCb ][ yCb ] is equal to MipSizeId[ xNbX ][ yNbX ], candMipModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- Otherwise, candMipModeX is set equal to −1.

- Otherwise, candMipModeX is derived using IntraPredModeY[ xNbX ][ yNbX ] and MipSizeId[ xCb ][ yCb ] as specified in Table 8-4.

- The candMipModeList[ x ] with x = 0..2 is derived as follows, using mipMpmCand[ sizeId ] as specified in Table 8-2 with sizeId set equal to MipSizeId[ xCb ][ yCb ]:

- If candMipModeA and candMipModeB are both equal to −1, the following applies:

candMipModeList[ 0 ] = mipMpmCand[ sizeId ][ 0 ]	(8-10)

candMipModeList[ 1 ] = mipMpmCand[ sizeId ][ 1 ]	(8-11)

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 2 ]	(8-12)

- Otherwise, the following apllies:

- If candMipModeA is equal to candMipModeB or if either candMipModeA or candMipModeB is equal to −1, the following applies:

candMipModeList[ 0 ] = ( candMipModeA != −1 )  ?  candMipModeA  :  candMipModeB	(8-13)

- If candMipModeList[ 0 ] is equal to mipMpmCand[ sizeId ][ 0 ], the following applies:

candMipModeList[ 1 ] = mipMpmCand[ sizeId ][ 1 ]	(8-14)

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 2 ]	(8-15)

- Otherwise, the following applies:

candMipModeList[ 1 ] = mipMpmCand[ sizeId ][ 0 ]	(8-16)

candMipModeList[ 2 ] = ( candMipModeList[ 0 ] != mipMpmCand[ sizeId ][ 1 ] )  ?  
	mipMpmCand[ sizeId ][ 1 ]  :  mipMpmCand[ sizeId ][ 2 ]	(8-17)

- Otherwise, the following applies:

candMipModeList[ 0 ] = candMipModeA		(8-18)

candMipModeList[ 1 ] = candMipModeB		(8-19)

- If candMipModeA and candMipModeB are both not equal to mipMpmCand[ sizeId ][ 0 ], the following applies:

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 0 ]	(8-20)

- Otherwise, the following applies:

- If candMipModeA and candMipModeB are both not equal to mipMpmCand[ sizeId ][ 1 ], the following applies:

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 1 ]	(8-21)

- Otherwise, the following applies:

candMipModeList[ 2 ] = mipMpmCand[ sizeId ][ 2 ]	(8-22)

- IntraPredModeY[ xCb ][ yCb ] is derived by applying the following procedure:

- If intra_mip_mpm_flag[ xCb ][ yCb ] is equal to 1, IntraPredModeY[ xCb ][ yCb ] is set equal to candMipModeList[ intra_mip_mpm_idx[ xCb ][ yCb ] ].

- Otherwise, IntraPredModeY[ xCb ][ yCb ] is derived by applying the following ordered steps:

- When candMipModeList[ i ] is greater than candMipModeList[ j ] for i = 0..1 and for each i, j = ( i + 1 )..2, both values are swapped as follows:

( candMipModeList[ i ], candMipModeList[ j ] ) = Swap( candMipModeList[ i ], 	candMipModeList[ j ] )	(8-23)

- IntraPredModeY[ xCb ][ yCb ] is derived by the following ordered steps:

  - IntraPredModeY[ xCb ][ yCb ] is set equal to intra_mip_mpm_remainder[ xCb ][ yCb ].

  - For i ranging from 0 to 2, inclusive, when IntraPredModeY[ xCb ][ yCb ] is greater than or equal to candMipModeList[ i ], the value of IntraPredModeY[ xCb ][ yCb ] is incremented by one.

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

Table 8-1 – Specification of mapping between intra prediction and MIP modes

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 17 | 0 | 5 |
| 1 | 17 | 0 | 1 |
| 2, 3 | 17 | 10 | 3 |
| 4, 5 | 9 | 10 | 3 |
| 6,7 | 9 | 10 | 3 |
| 8, 9 | 9 | 10 | 3 |
| 10, 11 | 9 | 10 | 0 |
| 12, 13 | 17 | 4 | 0 |
| 14, 15 | 17 | 6 | 0 |
| 16, 17 | 17 | 7 | 4 |
| 18, 19 | 17 | 7 | 4 |
| 20, 21 | 17 | 7 | 4 |
| 22, 23 | 17 | 5 | 5 |
| 24, 25 | 17 | 5 | 1 |
| 26, 27 | 5 | 0 | 1 |
| 28, 29 | 5 | 0 | 1 |
| 30, 31 | 5 | 3 | 1 |
| 32, 33 | 5 | 3 | 1 |
| 34, 35 | 34 | 12 | 6 |
| 36, 37 | 22 | 12 | 6 |
| 38, 39 | 22 | 12 | 6 |
| 40, 41 | 22 | 12 | 6 |
| 42, 43 | 22 | 14 | 6 |
| 44, 45 | 34 | 14 | 10 |
| 46, 47 | 34 | 14 | 10 |
| 48, 49 | 34 | 16 | 9 |
| 50, 51 | 34 | 16 | 9 |
| 52, 53 | 34 | 16 | 9 |
| 54, 55 | 34 | 15 | 9 |
| 56, 57 | 34 | 13 | 9 |
| 58, 59 | 26 | 1 | 8 |
| 60, 61 | 26 | 1 | 8 |
| 62, 63 | 26 | 1 | 8 |
| 64, 65 | 26 | 1 | 8 |
| 66 | 26 | 1 | 8 |

Table 8-2 – Specification of MIP candidate modes mipMpmCand[ sizeId ][ x ]

| sizeId | candidate mode x |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 17 | 34 | 5 |
| 1 | 0 | 7 | 16 |
| 2 | 1 | 4 | 6 |

P140

The variable IntraPredModeY[ x ][ y ] with  x = xCb..xCb + cbWidth − 1 and y = yCb..yCb + cbHeight − 1 is set to be equal to IntraPredModeY[ xCb ][ yCb ].

Table 8-4 – Specification of mapping between MIP and intra prediction modes

| IntraPredModeY[ xNbX ][ yNbX ] | MipSizeId |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 0 | 0 | 1 |
| 1 | 18 | 1 | 1 |
| 2 | 18 | 0 | 1 |
| 3 | 0 | 1 | 1 |
| 4 | 18 | 0 | 18 |
| 5 | 0 | 22 | 0 |
| 6 | 12 | 18 | 1 |
| 7 | 0 | 18 | 0 |
| 8 | 18 | 1 | 1 |
| 9 | 2 | 0 | 50 |
| 10 | 18 | 1 | 0 |
| 11 | 12 | 0 |  |
| 12 | 18 | 1 |  |
| 13 | 18 | 0 |  |
| 14 | 1 | 44 |  |
| 15 | 18 | 0 |  |
| 16 | 18 | 50 |  |
| 17 | 0 | 1 |  |
| 18 | 0 | 0 |  |
| 19 | 50 |  |  |
| 20 | 0 |  |  |
| 21 | 50 |  |  |
| 22 | 0 |  |  |
| 23 | 56 |  |  |
| 24 | 0 |  |  |
| 25 | 50 |  |  |
| 26 | 66 |  |  |
| 27 | 50 |  |  |
| 28 | 56 |  |  |
| 29 | 50 |  |  |
| 30 | 50 |  |  |
| 31 | 1 |  |  |
| 32 | 50 |  |  |
| 33 | 50 |  |  |
| 34 | 50 |  |  |

# Conclusion

This contribution reports the result of adding one Matrix-based intra prediction mode per block size, instead of the at most 35 modes introduced by MIP. This incurs a reduction of the memory footprint of 7.09 kilobytes for a loss of 0.14% in AI and 0.10% in RA, when the proposed mode is not combined with MTS.

# References

- Jonathan Pfaff, Björn Stallenberger, Michael Schäfer, Philipp Merkle, Philipp Helle, Tobias Hinz, Heiko Schwarz, Detlev Marpe, Thomas Wiegand. JVET-N0217: “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”. Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th meeting, Geneva, CH, 19-27 March 2019.

- JVET-M1010: “JVET common test conditions and software reference configurations”. Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 13th meeting, Marrakech, MA, 9-18 January 2019.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).