| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0527 |
| --- | --- |

| Title: | Non-CE7: Use INTER quantization matrices for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange Ya Chen | Email: | philippe.delagrange@interdigital.com ya.chen@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to use INTER quantization matrices to IBC-coded blocks, instead of INTRA quantization matrices. This is said to align the design with IBC prediction mode being closer to INTER than INTRA.

# Introduction

Quantization matrices were introduced in VVC during the 14th JVET meeting, based on contribution JVET-N0847.

As in HEVC, different quantization matrices are specified for two prediction modes: INTRA and INTER, as shown in table 2. As VVC introduced a new Intra Block Copy prediction mode (IBC), quantization matrices used for this new mode are the INTRA ones.

Table 1: Size identifier (JVET-N0847)

| Size of quantization matrix | sizeId |
| --- | --- |
| 1x1 | 0 |
| 2x2 | 1 |
| 4x4 | 2 |
| 8x8 | 3 |
| 16x16 | 4 |
| 32x32 | 5 |
| 64x64 | 6 |

Table 2: Matrix type identifier (JVET-N0847)

| sizeId | CuPredMode | cIdx (Colour component) | matrixId |
| --- | --- | --- | --- |
| 2, 3, 4, 5, 6 | MODE_INTRA | 0 (Y) | 0 |
| 1, 2, 3, 4, 5 | MODE_INTRA | 1 (Cb) | 1 |
| 1, 2, 3, 4, 5 | MODE_INTRA | 2 (Cr) | 2 |
| 2, 3, 4, 5, 6 | MODE_INTER | 0 (Y) | 3 |
| 1, 2, 3, 4, 5 | MODE_INTER | 1 (Cb) | 4 |
| 1, 2, 3, 4, 5 | MODE_INTER | 2 (Cr) | 5 |
| 2, 3, 4, 5, 6 | MODE_IBC | 0 (Y) | 0 |
| 1, 2, 3, 4, 5 | MODE_IBC | 1 (Cb) | 1 |
| 1, 2, 3, 4, 5 | MODE_IBC | 2 (Cr) | 2 |

JVET-O0223 proposes a new unified scaling matrix identifier, as shown in table 3, but still reuses INTRA matrices for IBC mode.

Table 3: matrixId (JVET-O0223)

| Y | INTRA | 0 |  | 6 |  | 12 |  | 18 |  | 24 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTER | 1 |  | 7 |  | 13 |  | 19 |  | 25 |  |
| Cb | INTRA |  | 2 |  | 8 |  | 14 |  | 20 |  | 26 |
|  | INTER |  | 3 |  | 9 |  | 15 |  | 21 |  | 27 |
| Cr | INTRA |  | 4 |  | 10 |  | 16 |  | 22 |  | 28 |
|  | INTER |  | 5 |  | 11 |  | 17 |  | 23 |  | 29 |
| TU size: luma max(width,height) |  | 64 |  | 32 |  | 16 |  | 8 |  | 4 |  |
| Block size: max(width,height) |  | 64 | 32 |  | 16 |  | 8 |  | 4 |  | 2 |
| Signalled QM size |  | 8x8 + DC |  |  |  |  | 8x8 |  | 4x4 |  | 2x2 |

Since IBC consists in predicting the current block from a portion of the current image with a displacement vector, it could be said to be closer to INTER mode than INTRA mode, which predicts the current block using its borders. The residual can be expected to have statistics closer to an INTER-predicted block than an INTRA-predicted block.

# Proposal description

This contribution proposes to use INTER quantization matrices for IBC blocks.

## Changes to VVC draft 5 (JVET-N0847)

Matrix type identification table is changed as follows:

Table 4: Changes to JVET-N0847
“specification of matrixId according to sizeId, prediction mode and colour component”

| sizeId | CuPredMode | cIdx (Colour component) | matrixId |
| --- | --- | --- | --- |
| 2, 3, 4, 5, 6 | MODE_INTRA | 0 (Y) | 0 |
| 1, 2, 3, 4, 5 | MODE_INTRA | 1 (Cb) | 1 |
| 1, 2, 3, 4, 5 | MODE_INTRA | 2 (Cr) | 2 |
| 2, 3, 4, 5, 6 | MODE_INTER | 0 (Y) | 3 |
| 1, 2, 3, 4, 5 | MODE_INTER | 1 (Cb) | 4 |
| 1, 2, 3, 4, 5 | MODE_INTER | 2 (Cr) | 5 |
| 2, 3, 4, 5, 6 | MODE_IBC | 0 (Y) | 0 3 |
| 1, 2, 3, 4, 5 | MODE_IBC | 1 (Cb) | 1 4 |
| 1, 2, 3, 4, 5 | MODE_IBC | 2 (Cr) | 2 5 |

## Changes to JVET-O0223

The derivation of matrixId in the “derivation process for scaling matrix” is changed as follows:

matrixId = 6 * sizeId + matrixTypeId		(8-1)
with subWidth = (cIdx > 0) ? SubWidthC : 1,
subHeight = (cIdx > 0) ? SubHeightC : 1,
sizeId = 6 – max( log2( blkWidth * subWidth ), log2( blkHeight * subHeight ) ), and
matrixTypeId = ( 2 * cIdx + ( predMode == MODE_INTER ? 1 : 0 ) )
matrixTypeId = ( 2 * cIdx + ( predMode == MODE_INTRA ? 0 : 1 ) )

# References

[JVET-N1001]	B.Bross, J.Chen, S.Liu, “Versatile Video Coding (Draft 5)”, JVET-N1001, Geneva, CH, March 2019

[JVET-N0847]	O.Chubach, T.Toma, S.C.Lim, et al., “CE7-related: Support of quantization matrices for VVC”, JVET-N0847, Geneva, CH, March 2019

[JVET_O0223]	P. de Lagrange, F. Le Léannec, E. François, K. Naser, “Non-CE7: Quantization matrices with single identifier and prediction from larger ones”, JVET-NO0223, Gothenburg, SE, July 2019

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).