| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0539-v2 |
| --- | --- |

| Title: | CE6-related: Unified LFNST using block size independent kernel |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The Low Frequency Non-Separable Transform (LFNST) in VVC Draft v5 applies two different sizes of non-separable secondary transform kernel, the one with size 16x16 is applied for blocks with either width or height smaller than 8, the other one with size 16x48 is applied for blocks with both width or height greater than or equal to 8. To unify the LFNST design, in this contribution, it is proposed to use only the 16x48 kernels as in VTM-5.0 for LFNST. The reported benefits include: 1) the memory for storing the 16x16 LFNST kernels, i.e., 2KB memory, can be saved, 2) the calculation of LFNST is simplified because the switching of transform sizes is not needed, and 3) both VTM-5.0 and Spec text can be simplified. Experimental results reportedly show that, compared to VTM-5.0, the proposed method gives 0.05%, 0.01% luma BD-rate changes for All Intra (AI) and Random Access (RA) configurations, respectively, with 3% overall encoding time saving for AI. Compared to CE6-2.1a, the proposed method gives 0.00%, -0.01% luma BD-rate changes for AI and RA configurations, respectively.

# Introduction

In VVC Draft v5, two different kernel sizes are applied in LFNST, their applicable block sizes and memory cost is described in below Table 1.

Table 1: LFNST kernels applied in VTM-5.0.

| LFNST kernel size | Applicable block sizes | Memory cost |
| --- | --- | --- |
| 16×16 | 4×N and N×4, N>=4 | 2 KB (16×16×2×4) |
| 16×48 | 8×N and N×8, N>=8 | 6 KB (16×48×2×4) |

The 16×16 LFNST kernels cost 2KB memory and 3 pages of Spec text.

Two different shapes of the LFNST are applied. For 16x16 LFNST kernels which include 16 bases, each basis can be represented by one 4x4 matrix. For 16x48 LFNST kernels which include 16 bases, each basis can be represented by one L shape region, as shown in Figure 1.

Figure 1: Transform basis shape of LFNST with 16x16 kernels (left) and 16x48 kernels (right).

Due to the application of different LFNST kernel sizes for different block sizes, many LFNST processing and intermediate variables are block size dependent, as described in both VTM-5.0 and VVC Draft v5 (highlighted below).

8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-953)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-954)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-955)

nLfnstSize = 1  <<  log2LfnstSize		(8-956)

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	(8-957)

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	(8-958)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(8-959)

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- For xSbIdx = 0..numLfnstX − 1 and ySbIdx = 0..numLfnstY − 1, the following applies:

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = ( xSbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 0 ]	(8-960)

yC = ( ySbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 1 ]	(8-961)

u[ x ] = d[ xC ][ yC ] 			(8-962)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(8-963)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(8-964)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

8.7.4.2	Low frequency non-separable transformation process

Inputs to this process are:

- a variable nonZeroSize specifying the transform input length,

- a variable nTrS specifying the transform output length,

- a list of of scaled non-zero transform coefficients x[ j ] with j = 0..nonZeroSize − 1,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTrS − 1.

The transformation matrix derivation process as specified in clause 8.7.4.3 is invoked with the transform output length nTrS, the intra prediction mode for LFNST set selection  predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnstIdx as inputs, and the (nTrS)x(nonZeroSize) LFNST matrix lowFreqTransMatrix as output.

The list of transformed samples y[ i ] with i = 0..nTrS − 1 is derived as follows:

y[i] = Clip3( CoeffMin, CoeffMax, ( ( j=0nonZeroSize-1lowFreqTransMatrix[i][j]*x[j] ) + 64 ) >> 7 )	(8-970)

# Proposal

To unify the LFNST calculation and reduce the memory cost, it is proposed to use only 16×48 kernels for LFNST. For small block sizes 4×N and N×4, they are considered as part of 8×N and N×8 blocks, and only the overlapping part with 16x48 basis are used.

For a forward LFNST, when a 16×48 kernel is applied for a 4×N or N×4 block, only the overlapping part with the basis are used for secondary transform. Examples of applying 16×48 LFNST kernel for 4×4, 8×4, 4×8 and 16×4, as shown in shaded area in Figure 2.

Figure 2: Applying 16×48 LFNST kernel for 4x4, 8x4, 4x8 and 4x16 (from left to right).

For an inverse LFNST, the input are 8 or 16 coefficients, and the output is 48 samples which form the L shape as described in Figure 1. However, only the samples inside the block size region need to be calculated and the remaining samples are not calculated, e.g, for a 4x4 TU, given the 8 coefficients and 16x48 kernel, only the top-left 4x4 is calculated as the output, other coefficients outside 4x4 are not calculated.

To keep the worst-case multiplication per coefficient unchanged, for 8x4 and 4x8 blocks, only 8 coefficients are calculated, which is aligned with LFNST applied on 4x4 and 8x8 TUs in VTM-5.0.

# Experimental results

The following table summarizes the simulations for reporting the coding performance of proposed method. Two test conditions are applied, one is Common Test Condition (CTC) and the other one is CTC with Low QP configuration, as required in CE6.

Table 2: Simulations for the proposed method.

| Test # | Anchor | Test | Test condition |
| --- | --- | --- | --- |
| Test 1 | VTM-5.0 | VTM-5.0 + Proposed method | CTC |
| Test 2 | VTM-5.0 | VTM-5.0 + Proposed method | Low QP |
| Test 3 | CE6-2.1a | CE6-2.1a + Proposed method | CTC |
| Test 4 | CE6-2.1a | CE6-2.1a + Proposed method | Low QP |
| Test 3 | CE6-2.1b | CE6-2.1b + Proposed method | CTC |
| Test 4 | CE6-2.1b | CE6-2.1b + Proposed method | Low QP |

Detailed results are reported in below tables.

Table 3: Simulation results of Test 1 (Anchor VTM-5.0, CTC).

|  |  | All Intra Main 10 |  |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | -0.02% |  | 0.14% | 0.02% | 98% | 101% | -0.01% |  | 0.03% | -0.05% | 99% | 100% |
| Class A2 | -0.01% |  | 0.16% | 0.15% | 97% | 100% | 0.00% |  | 0.15% | 0.05% | 99% | 100% |
| Class B | 0.06% |  | 0.01% | 0.03% | 97% | 99% | 0.01% |  | 0.08% | -0.01% | 99% | 98% |
| Class C | 0.08% |  | 0.07% | 0.12% | 96% | 95% | 0.01% |  | 0.16% | 0.01% | 100% | 99% |
| Class E | 0.14% |  | -0.14% | -0.13% | 98% | 96% |  |  |  |  |  |  |
| Overall | 0.05% |  | 0.04% | 0.04% | 97% | 98% | 0.01% |  | 0.11% | 0.00% | 99% | 99% |
| Class D | 0.12% |  | 0.35% | 0.24% | 97% | 101% | 0.07% |  | 0.36% | 0.13% | 100% | 102% |
| Class F | 0.01% |  | 0.16% | 0.27% | 98% | 97% | 0.00% |  | 0.28% | 0.10% | 100% | 101% |

Table 4: Simulation results of Test 2 (Anchor VTM-5.0, LowQP).

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.08% | 0.06% | 93% | 101% |  |  |  |  |  |
| Class A2 | 0.09% | 0.08% | 0.10% | 93% | 101% |  |  |  |  |  |
| Class B | 0.09% | 0.20% | 0.21% | 92% | 99% | 0.03% | 0.00% | 0.02% | 97% | 102% |
| Class C | 0.16% | 0.18% | 0.12% | 91% | 100% | 0.05% | 0.00% | 0.03% | 98% | 99% |
| Class E | 0.19% | 0.39% | 0.29% | 93% | 98% |  |  |  |  |  |
| Overall | 0.12% | 0.19% | 0.16% | 92% | 100% |  |  |  |  |  |
| Class D | 0.16% | 0.20% | 0.12% | 91% | 101% | 0.05% | 0.07% | 0.08% | 99% | 99% |
| Class F | 0.15% | 0.16% | 0.11% | 95% | 101% | 0.11% | 0.10% | 0.03% | 99% | 100% |

Table 5: Simulation results of Test 3 (Anchor CE6-2.1a, CTC).

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.09% | -0.08% | 100% | 101% | -0.01% | -0.12% | -0.03% | 101% | 100% |
| Class A2 | -0.02% | 0.16% | 0.09% | 100% | 102% | -0.03% | 0.14% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.08% | 0.03% | 99% | 100% | 0.01% | 0.01% | 0.03% | 100% | 101% |
| Class C | 0.01% | 0.14% | 0.17% | 100% | 99% | -0.03% | 0.07% | 0.05% | 100% | 98% |
| Class E | 0.06% | 0.05% | -0.03% | 100% | 100% |  |  |  |  |  |
| Overall | 0.00% | 0.10% | 0.04% | 100% | 100% | -0.01% | 0.03% | 0.02% | 100% | 100% |
| Class D | 0.04% | 0.19% | 0.36% | 99% | 97% | 0.03% | 0.12% | 0.11% | 100% | 100% |
| Class F | 0.01% | -0.01% | 0.21% | 99% | 97% | -0.07% | 0.10% | 0.07% | 100% | 100% |

Table 6: Simulation results of Test 4 (Anchor CE6-2.1a, Low QP).

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | 0.01% | 98% | 101% | 0.00% | 0.02% | -0.01% | 99% | 100% |
| Class A2 | 0.01% | 0.00% | 0.01% | 98% | 100% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.02% | 0.01% | 0.06% | 97% | 99% | 0.01% | -0.01% | 0.03% | 99% | 99% |
| Class C | 0.01% | 0.03% | -0.01% | 97% | 97% | 0.00% | -0.01% | -0.06% | 99% | 100% |
| Class E | 0.01% | 0.21% | 0.23% | 98% | 97% |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.06% | 98% | 98% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.03% | 0.06% | 0.04% | 97% | 97% | 0.00% | 0.01% | 0.03% | 99% | 96% |
| Class F | 0.06% | 0.08% | 0.06% | 99% | 99% | 0.06% | 0.04% | 0.15% | 99% | 99% |

Table 7: Simulation results of Test 5 (Anchor CE6-2.1b, CTC).

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | 0.12% | 99% | 98% | 0.02% | 0.20% | 0.06% | 100% | 100% |
| Class A2 | -0.01% | 0.19% | 0.12% | 99% | 98% | -0.02% | -0.09% | 0.04% | 100% | 100% |
| Class B | 0.00% | 0.16% | 0.12% | 100% | 95% | 0.00% | -0.06% | -0.02% | 99% | 97% |
| Class C | 0.01% | 0.05% | 0.12% | 101% | 97% | 0.04% | 0.05% | 0.17% | 99% | 95% |
| Class E | 0.01% | 0.27% | 0.19% | 100% | 100% |  |  |  |  |  |
| Overall | 0.00% | 0.13% | 0.13% | 100% | 97% | 0.01% | 0.02% | 0.06% | 100% | 98% |
| Class D | 0.05% | 0.10% | 0.06% | 100% | 99% | 0.03% | -0.04% | 0.08% | 100% | 99% |
| Class F | -0.01% | 0.08% | 0.26% | 99% | 98% | -0.04% | -0.05% | -0.09% | 99% | 98% |

Table 8: Simulation results of Test 6 (Anchor CE6-2.1b, Low QP).

|  | All Intra Main 10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.02% | 99% | 101% | 0.00% | 0.00% | -0.03% | 100% | 99% |
| Class A2 | 0.01% | 0.00% | 0.00% | 99% | 102% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.02% | 0.02% | 0.00% | 98% | 98% | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class C | 0.01% | 0.01% | 0.02% | 98% | 97% | -0.01% | 0.02% | 0.00% | 98% | 99% |
| Class E | 0.00% | 0.18% | 0.16% | 99% | 98% |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.03% | 98% | 99% | 0.00% | 0.01% | 0.00% | 99% | 99% |
| Class D | 0.02% | 0.03% | 0.05% | 98% | 95% | 0.01% | 0.01% | -0.06% | 99% | 96% |
| Class F | 0.01% | 0.01% | 0.06% | 99% | 99% | -0.01% | -0.01% | -0.06% | 98% | 98% |

The LFNST kernels applied in VTM-5.0 and proposed methods are compared in Table 9. As reported, the proposed method uses only 16x48 kernels for all applicable block sizes of LFNST in VTM-5.0, and the memory cost for storing LFNST kernels is reduced from 8KB to 6KB, i.e., 25% reduction.

Table 9: Comparison on the LFNST kernels applied in VTM-5.0 and proposed method.

|  | LFNST kernel size | Applicable block sizes | Memory cost |
| --- | --- | --- | --- |
| VTM-5.0 | 16×16 | 4×N and N×4, N>=4 | 2 KB (16×16×2×4) |
|  | 16×48 | 8×N and N×8, N>=8 | 6 KB (16×48×2×4) |
| Proposed | 16x48 | M×N, M>=4 && N>=4 | 6 KB (16×48×2×4) |

# Spec text changes

The following proposed text modifications are based on JVET-N1001-v8. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

7.3.7.11	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| …… |  |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) { |  |  |
| numSigCoeff++ |  |  |
| if( ( n >= 8 && i = = 0 && ( log2TbWidth =< = 23 ¦ ¦ log2TbWidth = = 3 ) && ( log2TbWidth = = log2TbHeight ) ) ¦ ¦ ( ( i = = 1 ¦ ¦ i = = 2 ) && log2TbWidth >= 3 && log2TbHeight >= 3 ) ) |  |  |
| numZeroOutSigCoeff++ |  |  |
| } |  |  |
| …… |  |
| } |  |  |

8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, numLfnstX, numLfnstY, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(8-953)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(8-954)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(8-955)

nLfnstSize = 1  <<  log2LfnstSize		(8-956)

numLfnstX = ( nTbW > 8  &&  nTbH  = = 4 )  ?  2  :  1	(8-957)

numLfnstY = ( nTbW = = 4  &&  nTbH  > 8 )  ?  2  :  1	(8-958)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW =< = 8  &&  nTbH  =< = 8 ) )  ?  8  :  16	(8-959)

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- For xSbIdx = 0..numLfnstX − 1 and ySbIdx = 0..numLfnstY − 1, the following applies:

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = ( xSbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 0 ]	(8-960)

yC = ( ySbIdx << log2LfnstSize ) + DiagScanOrder[ log2LfnstSize ][ log2LfnstSize ][ x ][ 1 ]	(8-961)

u[ x ] = d[ xC ][ yC ] 			(8-962)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] with x = 0..min( nTbW, nLfnstSize ) − 1, y = 0.. min( nTbH, nLfnstSize ) − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, or equal to INTRA_LT_CCLM, INTRA_T_CCLM, or INTRA_L_CCLM, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(8-963)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ ( xSbIdx << log2LfnstSize ) + x ][ ( ySbIdx << log2LfnstSize ) + y ] = 
	  ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(8-964)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

The variable implicitMtsEnabled is derived as follows:

……

8.7.4.2	Low frequency non-separable transformation process

Inputs to this process are:

- a variable nonZeroSize specifying the transform input length,

- a variable nTrS specifying the transform output length,

- a list of of scaled non-zero transform coefficients x[ j ] with j = 0..nonZeroSize − 1,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTrS − 1.

The transformation matrix derivation process as specified in clause 8.7.4.3 is invoked with the transform output length nTrS, the intra prediction mode for LFNST set selection  predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnstIdx as inputs, and the (nTrS)x(nonZeroSize) LFNST matrix lowFreqTransMatrix as output.

The list of transformed samples y[ i ] with i = 0..nTrS − 1 is derived as follows:

y[i] = Clip3( CoeffMin, CoeffMax, ( ( j=0nonZeroSize-1lowFreqTransMatrix[i][j]*x[j] ) + 64 ) >> 7 )	(8-970)

8.7.4.3	Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

The variable lfnstTrSetIdx is specified in Table 8-18 – Specification of lfnstTrSetIdx depending on predModeIntra.

Table 8-18 – Specification of lfnstTrSetIdx

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 12 | 1 |
| 13 <= predModeIntra <= 23 | 2 |
| 24 <= predModeIntra <= 44 | 3 |
| 45 <= predModeIntra <= 55 | 2 |
| 56 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

The transformation matrix lowFreqTransMatrix is derived based on nTrS, lfnstTrSetIdx, and lfnstIdx as follows:

- If nTrS is equal to 16, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  108  -44  -15    1  -44   19    7   -1  -11    6    2   -1    0   -1   -1    0  }

{  -40  -97   56   12  -11   29  -12   -3   18   18  -15   -3   -1   -3    2    1  }

{   25  -31   -1    7  100  -16  -29    1  -54   21   14   -4   -7    2    4    0  }

{  -32  -39  -92   51   -6  -16   36   -8    3   22   18  -15    4    1   -5    2  }

{    8   -9   33   -8  -16 -102   36   23   -4   38  -27   -5    5   16   -8   -6  }

{  -25    5   16   -3  -38   14   11   -3  -97    7   26    1   55  -10  -19    3  }

{    8    9   16    1   37   36   94  -38   -7    3  -47   11   -6  -13  -17   10  }

{    2   34   -5    1   -7   24  -25   -3    8   99  -28  -29    6  -43   21   11  }

{  -16  -27  -39 -109    6   10   16   24    3   19   10   24   -4   -7   -2   -3  }

{   -9  -10  -34    4   -9   -5  -29    5  -33  -26  -96   33   14    4   39  -14  }

{  -13    1    4   -9  -30  -17   -3  -64  -35   11   17   19  -86    6   36   14  }

{    8   -7   -5  -15    7  -30  -28  -87   31    4    4   33   61   -5  -17   22  }

{   -2   13   -6   -4   -2   28  -13  -14   -3   37  -15   -3   -2  107  -36  -24  }

{    4    9   11   31    4    9   16   19   12   33   32   94   12    0   34  -45  }

{    2   -2    8  -16    8    5   28  -17    6   -7   18  -45   40   36   97   -8  }

{    0   -2    0  -10   -1   -7   -3  -35   -1   -7   -2  -32   -6  -33  -16 -112  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  119  -30  -22   -3  -23   -2    3    2  -16    3    6    0   -3    2    1    0  }

{  -27 -101   31   17  -47    2   22    3   19   30   -7   -9    5    3   -5   -1  }

{    0   58   22  -15 -102    2   38    2   10  -13   -5    4   14   -1   -9    0  }

{   23    4   66  -11   22   89   -2  -26   13   -8  -38   -1   -9  -20   -2    8  }

{  -19   -5  -89    2  -26   76  -11  -17   20   13   18   -4    1  -15    3    5  }

{  -10   -1   -1    6   23   25   87   -7  -74    4   39   -5    0   -1  -20   -1  }

{  -17  -28   12   -8  -32   14  -53   -6  -68  -67   17   29    2    6   25    4  }

{    1  -24  -23    1   17   -7   52    9   50  -92  -15   27  -15  -10   -6    3  }

{   -6  -17   -2 -111    7  -17    8  -42    9   18   16   25   -4    2   -1   11  }

{    9    5   35    0    6   21   -9   34   44   -3  102   11   -7   13   11  -20  }

{    4   -5   -5  -10   15   19   -2    6    6  -12  -13    6   95   69  -29  -24  }

{   -6   -4   -9  -39    1   22    0  102  -19   19  -32   30  -16  -14   -8  -23  }

{    4   -4    7    8    4  -13  -18    5    0    0   21   22   58  -88  -54   28  }

{   -4   -7    0  -24   -7    0  -25    3   -3  -30    8  -76  -34    4  -80  -26  }

{    0    6    0   30   -6    1  -13  -23    1   20   -2   80  -44   37  -68    1  }

{    0    0   -1    5   -1   -7    1  -34   -2    3   -6   19    5  -38   11 -115  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -111   39    4    3   44   11  -12   -1    7  -16   -5    2    3   -1    4    2  }

{  -47  -27   15   -1  -92   43   20   -2   20   39  -16   -5   10   -5  -13    2  }

{  -35  -23    4    4  -17  -72   32    6  -59   18   50   -6    0   40    0  -13  }

{   13   93  -27   -4  -48   13  -34    4  -52   11    1   10    3   16   -3    1  }

{  -11  -27    1    2  -47   -4  -36   10   -2  -85   14   29  -20   -2   57    4  }

{    0  -35   32   -2   26   60   -3  -17  -82    1  -30    0  -37   21    3   12  }

{  -17  -46  -92   14    7  -10  -39   29  -17   27  -28   17    1  -15  -13   17  }

{    4  -10  -23    4   16   58  -17   26   30   21   67    2  -13   59   13  -40  }

{    5  -20   32   -5    8   -3  -46   -7   -4    2  -15   24  100   44    0    5  }

{   -4   -1   38  -18   -7  -42  -63   -6   33   34  -23   15  -65   33  -20    2  }

{   -2  -10   35  -19    5    8  -44   14  -25   25   58   17    7  -84  -16  -18  }

{    5   13   18   34   11   -4   18   18    5   58   -3   42   -2  -10   85   38  }

{   -5   -7  -34  -83    2   -1   -4  -73    4   20   15  -12    4   -3   44   12  }

{    0    4   -2  -60    5    9   42   34    5  -14    9   80   -5   13  -38   37  }

{   -1    2    7  -57    3   -7    9   68   -9    6  -49  -20    6   -4   36  -64  }

{   -1    0  -12   23    1   -4   17  -53   -3    4  -21   72   -4   -8   -3  -83  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{   88  -55    6   -3  -66   27    9   -2   11   11  -13    1   -2   -7    1    2  }

{  -58  -20   27   -2  -27   75  -29    0   47  -42  -11   11   -9   -3   19   -4  }

{  -51   23  -22    5  -63    3   37   -5    1   64  -35   -4   29  -31  -11   13  }

{  -27  -76   49   -2   40   14    9  -17  -56   36  -25    6   14    3   -6    8  }

{   19   -4  -36   22   52    7   36  -23   28  -17  -64   15   -5  -44   48    9  }

{   29   50   13  -10    1   34  -59    1  -51    4  -16   30   52  -33   24   -5  }

{  -12  -21  -74   43  -13   39   18   -5  -58  -35   27   -5   19   26    6   -5  }

{   19   38  -10   -5   28   66    0   -5   -4   19  -30  -26  -40   28  -60   37  }

{   -6   27   18   -5  -37  -18   12  -25  -44  -10  -38   37  -66   45   40   -7  }

{  -13  -28  -45  -39    0   -5  -39   69  -23   16  -12  -18  -50  -31   24   13  }

{   -1    8   24  -51  -15   -9   44   10  -28  -70  -12  -39   24  -18   -4   51  }

{   -8  -22  -17   33  -18  -45  -57  -27    0  -31  -30   29   -2  -13  -53   49  }

{    1   12   32   51   -8    8   -2  -31  -22    4   46  -39  -49  -67   14   17  }

{    4    5   24   60   -5  -14  -23   38    9    8  -34  -59   24   47   42   28  }

{   -1   -5  -20  -34    4    4  -15  -46   18   31   42   10   10   27   49   78  }

{   -3   -7  -22  -34   -5  -11  -36  -69   -1   -3  -25  -73    5    4    4  -49  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -112   47   -2    2  -34   13    2    0   15   -7    1    0    8   -3   -1    0  }

{   29   -7    1   -1 -108   40    2    0  -45   13    4   -1    8   -5    1    0  }

{  -36  -87   69  -10  -17  -33   26   -2    7   14  -11    2    6    8   -7    0  }

{   28   -5    2   -2  -29   13   -2    0  103  -36   -4    1   48  -16   -4    1  }

{  -12  -24   15   -3   26   80  -61    9   15   54  -36    2    0   -4    6   -2  }

{   18   53   69  -74   14   24   28  -30   -6   -7  -11   12   -5   -7   -6    8  }

{    5   -1    2    0  -26    6    0    1   45   -9   -1    0 -113   28    8   -1  }

{  -13  -32   18   -2   15   34  -27    7  -25  -80   47   -1  -16  -50   28    2  }

{   -4  -13  -10   19   18   46   60  -48   16   33   60  -48    1    0    5   -2  }

{   15   33   63   89    8   15   25   40   -4   -8  -15   -8   -2   -6   -9   -7  }

{   -8  -24  -27   15   12   41   26  -29  -17  -50  -39   27    0   35  -67   26  }

{   -2   -6  -24   13   -1   -8   37  -22    3   18  -51   22  -23  -95   17   17  }

{   -3   -7  -16  -21   10   24   46   75    8   20   38   72    1    2    1    7  }

{    2    6   10   -3   -5  -16  -31   12    7   24   41  -16  -16  -41  -89   49  }

{    4    8   21   40   -4  -11  -28  -57    5   14   31   70    7   18   32   52  }

{    0    1    4   11   -2   -4  -13  -34    3    7   20   47   -6  -19  -42 -101  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{  -99   39   -1    2   65  -20   -5    0  -15   -2    5   -1    0    3   -1    0  }

{   58   42  -33    3   33  -63   23   -1  -55   32    3   -5   21   -2   -8    3  }

{  -15   71  -44    5  -58  -29   25    3   62   -7   -4   -4  -19    4    0    1  }

{   46    5    4   -6   71  -12  -15    5   52  -38   13   -2  -63   23    3   -3  }

{  -14  -54  -29   29   25   -9   61  -29   27   44  -48    5  -27  -21   12    7  }

{   -3    3   69  -42  -11  -50  -26   26   24   63  -19   -5  -18  -22   12    0  }

{   17   16   -2    1   38   18  -12    0   62    1  -14    5   89  -42    8   -2  }

{   15   54   -8    6    6   60  -26   -8  -30   17  -38   22  -43  -45   42   -7  }

{   -6  -17  -55  -28    9   30   -8   58    4   34   41  -52  -16  -36  -20   16  }

{   -2   -1   -9  -79    7   11   48   44  -13  -34  -55    6   12   23   20  -11  }

{    7   29   14   -6   12   53   10  -11   14   59  -15   -3    5   71  -54   13  }

{   -5  -24  -53   15   -3  -15  -61   26    6   30  -16   23   13   56   44  -35  }

{    4    8   21   52   -1   -1   -5   29   -7  -17  -44  -84    8   20   31   39  }

{   -2  -11  -25   -4   -4  -21  -53    2   -5  -26  -64   19   -8  -19  -73   39  }

{   -3   -5  -23  -57   -2   -4  -24  -75    1    3    9  -25    6   15   41   61  }

{    1    1    7   18    1    2   16   47    2    5   24   67    3    9   25   88  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 1, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -114   37    3    2  -22  -23   14    0   21  -17   -5    2    5    2   -4   -1  }

{  -19  -41   19   -2   85  -60  -11    7   17   31  -34    2  -11   19    2   -8  }

{   36  -25   18   -2  -42  -53   35    5   46  -60  -25   19    8   21  -33   -1  }

{  -27  -80   44   -3  -58    1  -29   19  -41   18  -12   -7   12  -17    7   -6  }

{  -11  -21   37  -10   44   -4   47  -12  -37  -41   58   18   10  -46  -16   31  }

{   15   47   10   -6  -16  -44   42   10  -80   25  -40   21  -23   -2    3  -14  }

{   13   25   79  -39  -13   10   31   -4   49   45   12   -8    3   -1   43    7  }

{   16   11  -26   13  -13  -74  -20   -1    5   -6   29  -47   26  -49   54    2  }

{   -8  -34  -26    7  -26  -19   29  -37    1   22   46   -9  -81   37   14   20  }

{   -6  -30  -42  -12   -3    5   57  -52   -2   37  -12    6   74   10    6  -15  }

{    5    9   -6   42  -15  -18   -9   26   15   58   14   43   23  -10  -37   75  }

{   -5  -23  -23   36    3   22   36   40   27   -4  -16   56  -25  -46   56  -24  }

{    1    3   23   73    8    5   34   46  -12    2   35  -38   26   52    2  -31  }

{   -3   -2  -21  -52    1  -10  -17   44  -19  -20   30   45   27   61   49   21  }

{   -2   -7  -33  -56   -4   -6   21   63   15   31   32  -22  -10  -26  -52  -38  }

{   -5  -12  -18  -12    8   22   38   36   -5  -15  -51  -63   -5    0   15   73  }

},

- Otherwise, if nTrS is equal to 16, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 2, the following applies:

lowFreqTransMatrix[ m ][ n ] =

{

{ -102   22    7    2   66  -25   -6   -1  -15   14    1   -1    2   -2    1    0  }

{   12   93  -27   -6  -27  -64   36    6   13    5  -23    0   -2    6    5   -3  }

{  -59  -24   17    1  -62   -2   -3    2   83  -12  -17   -2  -24   14    7   -2  }

{  -33   23  -36   11  -21   50   35  -16  -23  -78   16   19   22   15  -30   -5  }

{    0  -38  -81   30   27    5   51  -32   24   36  -16   12  -24   -8    9    1  }

{   28   38    8   -9   62   32  -13    2   51  -32   15    5  -66   28    0   -1  }

{   11  -35   21  -17   30  -18   31   18  -11  -36  -80   12   16   49   13  -32  }

{  -13   23   22  -36  -12   64   39   25  -19   23  -36    9  -30  -58   33   -7  }

{   -9  -20  -55  -83    3   -2    1   62    8    2   27  -28    7   15  -11    5  }

{   -6   24  -38   23   -8   40  -49    0   -7    9  -25  -44   23   39   70   -3  }

{   12   17   17    0   32   27   21    2   67   11   -6  -10   89  -22  -12   16  }

{    2   -9    8   45    7   -8   27   35   -9  -31  -17  -87  -23  -22  -19   44  }

{   -1   -9   28  -24   -1  -10   49  -30   -8   -7   40    1    4   33   65   67  }

{    5  -12  -24  -17   13  -34  -32  -16   14  -67   -7    9    7  -74   49    1  }

{    2   -6   11   45    3  -10   33   55    8   -5   59    4    7   -4   44  -66  }

{   -1    1  -14   36   -1    2  -20   69    0    0  -15   72    3    4    5   65  }

},

- Otherwise, iIf nTrS is equal to 48, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 1 the following applies:

……

- Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 0, and lfnstIdx is equal to 2 the following applies:

……

- Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 1 the following applies:

……

- Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 1, and lfnstIdx is equal to 2 the following applies:

……

- Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 1 the following applies:

……

- Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 2, and lfnstIdx is equal to 2 the following applies:

……

- Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 1 the following applies:

……

- Otherwise, if nTrS is equal to 48, lfnstTrSetIdx is equal to 3, and lfnstIdx is equal to 2 the following applies:

……

# Conclusion

This contribution proposed a unified LFNST which applies only one kernel size. Compared to VTM-5.0, the proposed method achieved 2KB memory reduction (25% of LFNST table size) with 3% overall encoder run-time saving for All Intra and 0.05% and 0.01% luma BD-rate for All Intra and Random Access configurations, respectively. In addition, due to the block-size independent LFNST kernel selection, both the VTM-5.0 SW and Spec text (more than 3 pages reduction) can be simplified.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).