| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0548 |
| --- | --- |

| Title: | Combined bilateral/SAO loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström Per Wennersten Jack Enhorn Du Liu Kenneth Andersson Rickard Sjöberg | Email: | {jacob.strom, per.wennersten, jack.enhorn, du.liu, kenneth.r.andersson, rickard.sjoberg}@ ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes a combination of the bilateral filter from JVET-N0493 with the sample adaptive offset (SAO) loop filter. It is reported that both SAO and the bilateral filter will operate on the same input, i.e., the output from the deblocking filter. Their outputs are then summed together and clipped. It is claimed that the same per-pixel complexity is reached as for JVET-N0493 with a total of 18 additions, 8 shifts and 8 checks are used per pixel, of which five of the additions are reported to be rounding operations. BD rate figures for luma are reported to be -0.41% (AI),  -0.49% (RA) and -0.32% (LD) over VTM-5.0. Decoder run times are reported to be 104% (AI), 102% (RA) and 104% (LD). Encoder run times are reported to be 106% (AI) and 102% (RA).

# Introduction

At the meeting in Geneva, a loop-filter based bilateral filter was proposed in contribution JVET-N0493. At the presentation, one of the major criticisms from the floor was the fact that it constituted an extra loop filter stage. Another concern was that the group was not certain that the bilateral filter would have a gain if deblocking RDO was turned on in the anchor: The meeting notes state

“Further results employing this filter + deblocking with joint RDO opt. against a non-CTC anchor enabling the RDO-optimized deblocking would be welcome, to understand if such an additional processing element in the loop would have benefit by itself.”

Thirdly, it was discussed what happens if the encoder panics, and has no time to run the bilateral filter on all the remaining output. Turning the filter off at a lower level than the entire slice was not possible for JVET-N0493, and the meeting notes state that

“It was also suggested by other experts to think about enabling/disabling at low level.”

In this proposal we are addressing these three issues.

## Combination with SAO

The current version of VVC already has three loop filters; deblocking, SAO and ALF. One major concern when it comes to many loop filter stages as expressed by Broadcom is that they can have to wait for each other: Assume a situation where SAO happens before ALF. An efficient decoder may start ALF filtering as soon as a sufficient number of samples have been output by SAO. However, ALF may be very quick on some parts of the image and slow on other, while the opposite may be true for SAO. When ALF is quick it may catch up with SAO and will have to wait until SAO has produced sufficient data for it to continue. Then when ALF can start again it may hit a slow patch of image data and have trouble finishing in time. Introducing an extra loop filter stage is thus undesirable.

In order not to increase the number of loop-filter stages, this contribution proposes the use of a combined bilateral/SAO loop-filter stage.

Figure 1: The bilateral filter and the SAO filters access the same 3×3 pixels

As can be seen in Figure 1, the bilateral filter (BIF) accesses the same 3×3 pixels as the SAO filter. Hence it is not necessary to increase any line buffer sizes when combining BIF with SAO.

Figure 2: Left: structure of JVET-N0493. Right: structure of current proposal. Note that BIF and SAO operate on the same input data and can therefore be executed in parallel if desired.

Figure 2 shows the difference in structure between JVET-N0493 and the current proposal. Previously a sample I(x,y) first needed to be filtered with the bilateral filter to generate BIF(I(x,y)) and only after that was it possible to start SAO filtering, SAO(BIF(I(x,y)). In contrast, in the current proposal, both BIF and SAO act on the input sample I(x,y) to create delta values

BIF = BIF(I(x,y))										Eq. 1

SAO = SAO(I(x,y))										Eq. 2

and the final pixel is constructed as

output = clip( I(x,y) + BIF + SAO).								Eq. 3

This means that BIF and SAO can be filtered completely independently from each other, and the worst-case scenario where one filter has to wait for the other is no longer happening. It is also possible to create a hardware decoder that performs BIF and SAO in synchrony for every sample, which avoids the waiting scenario altogether.

Since BIF actually works by calculating an offset to the current pixel, it is actually less expensive to calculate the difference BIF = BIF(I(x,y))-I(x,y) compared to calculating BIF(I(x,y)). Also, in the current proposal, one less clip-operation is needed per sample compared to JVET-N0493.

If BIF is turned off, SAO operates as in the CTC.

### Test 1: BIF+SAO results

The results for the BIF+SAO filter shown below against the CTC anchor (which includes SAO). There are two columns for the encoder run times. An older, slower version and a newer faster one, which is still running for some classes. When Qualcomm crosschecked they noticed that the class F results were not run with the RDPCM flag turned on, so these are being corrected.

All intra

|  |  | Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | EncT fast | DecT |
| Class A1 | -0.30% |  | 0.30% | 0.22% | 118% | 107% | 107% |
| Class A2 | -0.39% |  | 0.08% | 0.11% | 114% | 106% | 102% |
| Class B | -0.35% |  | 0.23% | 0.27% | 115% | 107% | 103% |
| Class C | -0.55% |  | 0.11% | 0.20% | 112% | 106% | 104% |
| Class E | -0.46% |  | 0.18% | 0.21% | 114% | 104% | 106% |
| Overall | -0.41% |  | 0.18% | 0.21% | 115% | 106% | 104% |
| Class D | -0.49% |  | 0.30% | -0.02% | 109% | 105% | 102% |
| Class F | -0.61% |  | 0.14% | -0.03% |  | 105% | 103% |

Random access

|  |  | Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | EncT fast | DecT |
| Class A1 | -0.37% |  | -0.14% | 0.05% | 109% | 103% | 103% |
| Class A2 | -0.49% |  | -0.13% | 0.01% | 108% | 103% | 102% |
| Class B | -0.48% |  | 0.08% | 0.01% | 107% | 102% | 103% |
| Class C | -0.61% |  | 0.04% | 0.01% | 108% | 101% | 101% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.49% |  | -0.02% | 0.02% | 108% | 102% | 102% |
| Class D | -0.54% |  | 0.21% | 0.27% | 105% | 101% | 100% |
| Class F | -0.65% |  | 0.00% | 0.18% |  | 97% | 104% |

Low delay

|  |  | Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | EncT fast | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.25% |  | 0.93% | 0.76% | 110% |  | 103% |
| Class C | -0.52% |  | 0.82% | 0.59% | 111% | 104% | 104% |
| Class E | -0.16% |  | 0.44% | 1.02% | 108% | 101% | 105% |
| Overall | -0.32% |  | 0.77% | 0.77% | 110% |  | 104% |
| Class D | -0.71% |  | 0.82% | 1.30% | 110% | 104% | 99% |
| Class F |  |  |  |  |  |  |  |

## Test 2: Testing with deblocking RDO enabled

At the Geneva meeting, it was argued that the JVET-N0493 proposal actually only did deblocking RDO. Therefore, it was argued that it would be interesting to compare it in a test where deblocking RDO is turned on for both the test and the anchor. If JVET-N0493 only did deblocking RDO, the result would be no gain over an anchor with deblocking RDO turned on. As can be seen in the results, this is not the case, instead the bilateral filter just loses about 0.1% from turning on the deblocking RDO.

### Results test 2 with deblocking RDO enabled in test and anchor

The results when deblocking RDO is turned on in the test and anchor is shown below. The encoder run times are measured on the slower version of the code. Newer encoder run time measurements are pending. When Qualcomm crosschecked they noticed that the class F results were not run with the RDPCM flag turned on, so these are being corrected.

All intra

|  | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | 0.21% | 0.22% | 111% | 105% |
| Class A2 | -0.29% | 0.15% | 0.09% | 109% | 107% |
| Class B | -0.20% | 0.22% | 0.23% | 111% | 102% |
| Class C | -0.44% | 0.25% | 0.03% | 119% | 104% |
| Class E | -0.32% | 0.14% | 0.24% | 108% | 105% |
| Overall | -0.28% | 0.20% | 0.16% | 112% | 104% |
| Class D | -0.42% | 0.17% | 0.11% | 112% | 100% |
| Class F |  |  |  |  |  |

Random access

|  | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | 0.10% | 0.08% | 104% | 104% |
| Class A2 | -0.39% | -0.02% | -0.13% | 104% | 100% |
| Class B | -0.36% | 0.05% | 0.06% | 107% | 100% |
| Class C | -0.53% | 0.02% | -0.26% | 106% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | 0.04% | -0.06% | 105% | 101% |
| Class D | -0.56% | 0.16% | 0.13% | 106% | 99% |
| Class F |  |  |  |  |  |

Low delay

|  | Over VTM-5.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.12% | 0.93% | 1.46% | 107% | 100% |
| Class C | -0.45% | 1.00% | 0.71% | 106% | 100% |
| Class E | 0.03% | 0.90% | 0.02% | 106% | 103% |
| Overall | -0.19% | 0.95% | 0.85% | 106% | 101% |
| Class D | -0.71% | 0.59% | 0.58% | 104% | 98% |
| Class F |  |  |  |  |  |

## Test 3: Testing with CTU-off flag for panic mode

Another problem brought up during discussions of the bilateral filter had to do with a so-called “panic mode”. The background is that a real-time encoder, such as one used in TV broadcast equipment, the encoder might encounter a very difficult frame that takes longer than normal to encode. In such a situation, the encoder might realize after having coded 75% of the image that it has very little time to finish the remaining 25%. It may then go into a “panic mode” where it just selects the fastest possible options with little regard to bit rate and quality. However, in such a situation it is a problem to just have an off-flag per slice for the bilateral filter, the encoder wants to turn off the filter as soon as possible. For this reason, we have included an on/off flag per CTU for the filter. Since this is only supposed to be used for panic mode, the initial CABAC context is set to a low probability of it being off. Hence the effects on the BD rate figures is very small, between 0.00% (AI), 0.01% (RA) and 0.03% (LD) as is seen in the results below.

### Results for test 3; BIF+SAO with BIF on/off flag per CTU

The results for the BIF+SAO combined filter with a CTU on/off flag for the BIF part are shown below against the CTC anchor (which includes SAO). The encoder run times are measured on the slower version of the code. When Qualcomm crosschecked they noticed that the class F results were not run with the RDPCM flag turned on, so these are being corrected.

All intra

|  |  | Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | EncT fast | DecT |
| Class A1 | -0.30% |  | 0.31% | 0.23% | 118% | 111% | 103% |
| Class A2 | -0.39% |  | 0.08% | 0.11% | 115% | 110% | 106% |
| Class B | -0.35% |  | 0.23% | 0.27% | 114% | 110% | 104% |
| Class C | -0.55% |  | 0.11% | 0.21% | 116% | 106% | 105% |
| Class E | -0.45% |  | 0.18% | 0.22% | 118% | 106% | 105% |
| Overall | -0.41% |  | 0.18% | 0.21% | 116% | 109% | 104% |
| Class D | -0.48% |  | 0.30% | -0.02% | 115% | 113% | 105% |
| Class F | -0.61% |  | 0.15% | -0.02% | 106% | 106% | 105% |

Random Access

|  |  | Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | EncT fast | DecT |
| Class A1 | -0.35% |  | -0.13% | 0.06% | 108% | 107% | 102% |
| Class A2 | -0.48% |  | -0.12% | 0.02% | 107% | 106% | 100% |
| Class B | -0.47% |  | 0.09% | 0.02% | 108% | 105% | 102% |
| Class C | -0.60% |  | 0.05% | 0.02% | 107% | 103% | 101% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.48% |  | -0.01% | 0.03% | 108% | 105% | 102% |
| Class D | -0.52% |  | 0.23% | 0.29% | 105% | 101% | 96% |
| Class F | -0.62% |  | 0.03% | 0.20% | 102% | 102% | 103% |

Low delay:

|  |  | Over VTM-5.0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y |  | U | V | EncT | EncT fast | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.24% |  | 0.94% | 0.77% | 111% |  | 102% |
| Class C | -0.51% |  | 0.83% | 0.61% | 111% | 109% | 102% |
| Class E | -0.07% |  | 0.52% | 1.10% | 109% | 106% | 103% |
| Overall | -0.29% |  | 0.80% | 0.80% | 110% |  | 103% |
| Class D | -0.69% |  | 0.84% | 1.32% | 113% | 109% | 100% |
| Class F |  |  |  |  |  |  |  |

# Proposal

Given that the bilateral filter gives a substantial gain of about -0.5%, we propose that it is included into the current draft of the VVC standard.

# Thanks

Many thanks to Qualcomm and HHI for their comprehensive efforts in crosschecking.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).