| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O0551 |
| --- | --- |

| Title: | AHG13: Tool-off tests on OpenImage database images. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jacob Ström Davood Saffar Per Wennersten Rickard Sjöberg | Email: | {jacob.strom, davood.saffar, per.wennersten, rickard.sjoberg}@ ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution tests the VVC-5.0 all-intra configuration on non-CTC data. The test set used contains 235 images from the OpenImage database. For each tool, the tool is turned off and then compared against the anchor. The results are compared against the CTC on/off tools tests carried out in AHG13.

# Introduction

The current way to evaluate the all intra use case is with the CTC sequences. To save computation resources, these are temporally subsampled by a factor of eight. Hence a typical 400-frame sequence generates about 50 different images. However, it should be noted that these 50 images are often still very similar to each other. This can be seen in Table 1, where we have shown the first and last image of every sequence in the CTC test set for AI. In half of the cases (CampfireParty, Catrobot, DaylightRoad, ParkRunning, Cactus, BasketballDrill, FourPeople, Johnny, KristenAndSara) there is very little difference. Thus, although the CTC test set includes approximately 18*50=900 different images, we are not getting as much information out of this as if all the 900 images were different. In this contribution we therefore want to run VVC on a test set that is more diverse compared with the CTC test set.

| first frame last frame | first frame last frame |
| --- | --- |
| CampfireParty | Catrobot |
| FoodMarket | DaylightRoad |
| Tango | ParkRunning |
| MarketPlace | RitualDance |
| Cactus | BasketballDrive |
| BQTerrace | BasketballDrill |
| BQMall | PartyScene |
| RaceHorses832x432 | FourPeople |
| Johnny | KristenAndSara |

Table 1. The first and last frame in each sequence used to compute the average AI gain for the CTC.

# Methodology

## Test set

We have been using the OpenImage database [1] which is used in the machine learning community. The github page [3] provides a large comma separated value file (.csv file) where every row contains the URL of an image that can be downloaded from the internet. We use version 1 of the csv file [2].

Given that the training part of the data set contains around 9 million images, it is possible to reject most images and still get a large data set. The images are in JPEG format, and some of them contain compression artifacts. To minimize the risk of including these, we have rejected all images with a JPEG quality setting under 99. We have also rejected images with a quality setting of 100 (which is the maximum) since a lot of those have been manually recompressed: People tend to use the highest quality setting when saving, and hence images of quality setting 100 sometimes anyway contain block compression artifacts.

We have also rejected all single-component images (JPEG-Y) and 4-component images (CMYK).

To facilitate compression, we have opted to use the same resolution, 1920×1080, for all images in our new test set, which we call the OpenImage test set. Therefore, we have also rejected all images with a pixel count smaller than 1920*1080, as well as images with a pixel count larger than 3640*2160. We have then cropped out the middle 1920×1080 pixels (after transposing if necessary). The images are then converted to YCbCr 4:2:0 and saved as a yuv-file. The contribution contains a python3 script that recreates the test set.

## Characteristics of this test set

The OpenImage test set has a number of characteristics that are not ideal for testing video. First, the data has previously been compressed with JPEG, whereas normally uncompressed data is used for testing. However, in the real world, codecs are often used on mezzanine data that has been compressed lightly, so to have a complementary test set that is compressed lightly is not unrealistic. Second, it consists of images and not of still frames from video. Hence it should contain less images with motion blur, which occur in video sequences, and more images with a bokeh effect (where the background is blurry for artistic effect). Also, no attempt has been made to manually scrub the data from images that contain screen-content or other types of content we would not like to be included. This is not ideal. However, given the diversity of the test set, it can still be useful as a complement to the CTC, especially when testing the robustness of the different tools.

## Tool-off test luma

In order to save computation time, we have partitioned the test set into 14 roughly similar parts of about 300 images each. The CTC anchor has been run on all 14 parts, but the tool-off test has only been run on the first part of 235 images. Even though that is considerably less than the number of images in the CTC-AI test (which is roughly 900), we believe that it can still give interesting results given the fact that each image in our test set is so different from every other.

Each image is compressed as its own 1-image long video sequence. For each one of these 235 “sequences”, a BD-rate figure is calculated using the macro-codes in the CTC excel-sheet. These 235 BD-rate numbers are then (arithmetically) averaged to provide a final average BD-rate value. A positive such value hence indicates that the compression efficiency on average is suffering when removing the tool. A negative value means that the compression efficiency improves when turning off the tool.

| Test | Description | Acronym | BDR-Y 235 images | AHG13 BDR-Y |
| --- | --- | --- | --- | --- |
| 1 | Chroma separate tree | CST | 0.61% | 0.41% |
| 2 | Dependent quantization/sign data hiding | DQ | 2.14% | 1.90% |
| 3 | Cross-component linear model | CCLM | 1.70% | 1.85% |
| 4 | Multiple transform set | MTS | 0.99% | 0.84% |
| 5 | Adaptive loop filter | ALF | 2.19% | 2.42% |
| 6 | Multi-line reference prediction | MLRP | 0.18% | 0.36% |
| 7 | Intra sub-partitioning | ISP | 0.24% | 0.31% |
| 8 | Matrix based intra prediction | MIP | 0.41% | 0.36% |
| 9 | Low frequency non-separable transform | LFNST | 0.88% | 1.26% |
| 10 | Sample adaptive offset | SAO | 0.41% | - |
| 11 | Reshaper | ILRS | -0.22% | 0.81% |

Table 2. Luma tool-off-tests. VTM-5.0 is executed with each tool turned off, and the result is compared against the CTC-anchor. A positive number indicates that the tool improves compression efficiency.

Table 2 shows the result of the tool-off tests for luma. The figures in bold face show the luma BD-rate number averaged over all 235 test images in the first part of the OpenImage test set. As a reference, the results from the tool-off tests run on the CTC [4] are included in the right-most column. For row 2, dependent quantization has been turned off and sign data hiding has simultaneously been turned on. To avoid enc/dec mismatches, we had to apply a fix from ticket #293. It was applied directly to VTM-5.0, and a git diff between VTM-5.0 and the software for the DQ test is included in the contribution.

The biggest surprise in this table is perhaps test 11, where the reshaper gets a negative average number. To investigate whether there was something special about the 235 images used in the test for Table 2, this tool was further tested on more data, as is shown in Table 3.

| Test part | Number of images in test set part | BDR-Y change when IRLS is turned off |
| --- | --- | --- |
| 0 | 235 | -0.22% |
| 1 | 242 | -0.24% |
| 2 | 218 | -0.15% |
| 3 | 271 | -0.19% |
| 4 | 243 | -0.22% |
| summary: | 1209 (sum) | -0.21% (average) |

Table 3. IRLS tool tested with more parts of the test set.

Averaging over more data improves the figure somewhat, from -0.22% to -0.21%. When looking at the figures for the individual images however, it becomes clear that some images gain hugely in BD-rate when turning off the reshaper – up to 23%. To avoid the effect of these outliers we have also calculated a number where all BD-rate numbers below -2% or above +2% are removed. The average then becomes -0.06%. The tool-off test on the CTC test set gave +0.81%.

## Tool-off tests chroma

We have also compiled the results for the chroma components, shown in Table 4. Even though we have excluded images consisting of only one luma channel from the test set, there are still some test images where the chroma channels are set to exactly zero. These images have been removed before calculating the figures in Table 4.

| Test | Description | Acronym | BDR-U no BW | BDR-V no BW | AHG13 BDR-U | AHG13 BDR-V |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Chroma separate tree | CST | 18.10% | 18.63% | 12.73% | 13.50% |
| 2 | Dependent quantization | DQ | 0.64% | 0.38% | -0.94% | -0.95% |
| 3 | Cross-component linear model | CCLM | 25.91% | 23.27% | 17.20% | 17.54% |
| 4 | Multiple transform set | MTS | 0.55% | 0.40% | 0.37% | 0.42% |
| 5 | Adaptive loop filter | ALF | 3.55% | 4.09% | 2.73% | 3.06% |
| 6 | Multi-line reference prediction | MLRP | 0.11% | -0.08% | 0.16% | 0.18% |
| 7 | Intra sub-partitioning | ISP | 0.21% | 0.02% | 0.25% | 0.25% |
| 8 | Matrix based intra prediction | MIP | 0.22% | 0.08% | 0.10% | 0.14% |
| 9 | Low frequency non-separable transform | LFNST | 0.22% | -0.03% | 0.39% | 0.55% |
| 10 | Sample adaptive offset | SAO | 0.18% | 0.24% | - | - |
| 11 | Reshaper | ILRS | -0.58% | -0.65% | -1.20% | -0.98% |

Table 4. Chroma tool-off-tests. A positive number indicates that the tool improves compression efficiency.

The biggest discrepancies between the numbers in our test and the test in AHG13 is that the results for CST and CCLM are considerably better in the OpenImage test set. As an example, CST almost doubles its performance. This seems to be due to the fact that, even though we have excluded images that have a chroma channel that is exactly zero, there are still a few images which are almost monochrome, and these are disproportionally helped by CST and CCLM.

# Conclusion

We have presented a tool-off test on a data set which we argue has a lot more variability than the CTC test set. It should therefore provide a good stress test for the all intra performance of VVC.

Given the strange performance of the ILRS tool we propose that this is excluded from the all intra CTC for SDR until a more robust version of the encoder software has been created.

# References

[1] Krasin I., Duerig T., Alldrin N., Veit A., Abu-El-Haija S., Belongie S., Cai D., Feng Z., Ferrari V., Gomes V., Gupta A., Narayanan D., Sun C., Chechik G, Murphy K. OpenImages: A public dataset for large-scale multi-label and multi-class image classification, 2016. Available from https://github.com/openimages

[2] CSV file https://storage.googleapis.com/openimages/2016_08/images_2016_08_v5.tar.gz

[3] OpenImage v1 Github page: https://github.com/openimages/dataset/blob/master/READMEV1.md

[4] AhG 13 Tool reporting https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-5.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).